RELEASE NOTES FOR

CISCO UNIFIED CME TELEPHONE SERVICE PROVIDER (TSP)

Version 2.2.0.5

Cisco System, Inc

1.  OVERVIEW

The Cisco CME TSP Version 2.2.0.5 provides updates to the current release CME TSP 2.1 in the following areas:

- The CME TSP support SCCP v9 protocol messages. This is in addition to the existing support of the SCCP V5 protocol.

- Support for CME 7.0 line types – including Octal Lines

- Updated CME TSP Wizard with changes for the Call Appearance line configuration

The Cisco CME TSP includes several modules including the ccCMETSP.tsp file. This is the CME TSP executable. It also includes the ccSkinny.dll that provides the protocol handling for the SCCP interface to the CME. And also a configuration Wizard to setup the CME TSP. Changes to these components are described below.

2. CHANGES MADE TO SKINNY Call Controll  library

The latest UCC Skinny Library is included with the 2.2.0.5 TSP build. The changes in the library include:

- Improvements in the SCCP register/unregister handling

- Support for Skinny v9 messages

- Support for concurrent v9 and v5 message types

- Detection and handling of method for providing transferred call digit buffer for the originating call leg or new call leg

- Detection of line type – single, dual or octal with CME 7.0

- Making RTP media streaming thread safe

- Bug fixes include:

- Setting and clearing call forward

- Problem with Group pickup

3. CHANGES MADE TO THE CME TSP CODE

Changes to the CME TSP include:

- Support for Cisco CME 7.0

- Support for Octal lines

- Extension of device specific messages to support monitor, watch and intercom

- Changes to allow digits not to be sent when monitor, watch or intercom lines are selected.

- Support for auto-line feature

- Changes in the call handling for remote multi-line for shared calls

- Set and clear call forward

- Support  for Microsoft Vista 32-bit

- Bug fixes include:

- Fixed crashing with skinny v9 library

- Fix for CSCsw79354: 3 second pause in ephone Skinny negotiation causes TAPI to get stuck

4. CHANGES MADE TO CME TSP WIZARD

Changes to the Wizard include:

- Support for Single, Dual and Octal line types

- Support for monitor, watch and intercom specifications on the lines

- Support for auto-line specification for the ephone

- Use of the standard ccTrog logging library

- Changes in the Call Appearance window in the GUI.

- Display of registration error messages ( now providing specific error information). This information is only provided for CME 7.0

## 5. List CISCO’ CRs corrected

Resolved

| Bug ID | Headline | Submitter | Submitted |
| --- | --- | --- | --- |
| CSCsw79354 | 3 second pause in ephone Skinny negotiation causes TAPI to get stuck |  |  |

# 6. OPEN CAVEATS.

# 3. TEST PLAN FOR EACH CHANGE.

CME TSP Test Plan is available upon request.

# 4. USER DOCUMENTATION IMPACTS WITH NEW VERSION.

- No Change.

# 5. MIGRATION/BACKOUT PROCEDURES

# 6. IDENTIFY & INCLUDE COMPATIBLE VERSIONS OF SOFTWARE, FIRMWARE, HARDWARE AND DOCUMENTATION.

# 7. IDENTIFY KNOWN PRODUCT DEFICIENCIES (OPERATING ISSUES).

See open CRs.

# 7. RECOMMENDATIONS FOR EQUIPMENT TO BE USED WITH THE CPE TO OPTIMIZE PERFORMANCE.