| Product name | Confidentiality level |
| --- | --- |
| E3372h-153 | CONFIDENTIAL |
| Product version | Total 8 pages |
| V13.0 |  |

HUAWEI E3372h TCPU-22.321.01.00.00

Release Notes V13.0

| Prepared by | E3372h Team | Date | 2016/10/31 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2014-9-30 | 1.0 | FW 22.180.03.00.00 | First version | Tanzhengbin |
| 2014-10-11 | 2.0 | FW 22.180.05.00.00 | Second version | Tanzhengbin |
| 2014-11-11 | 3.0 | FW 22.180.09.00.00 | Third version | Tanzhengbin |
| 2014-12-18 | 4.0 | FW 22.200.01.00.00 | Fourth version | Tanzhengbin |
| 2014-12-28 | 5.0 | FW 22.200.03.00.00 | Fifth version | Tanzhengbin |
| 2015-1-22 | 6.0 | FW 22.200.05.00.00 | Sixth version | Tanzhengbin |
| 2015-4-8 | 7.0 | FW 22.200.07.00.00 | Seventh version | Huwuming |
| 2015-4-18 | 8.0 | FW 22.200.09.00.00 | Eighth version | Huwuming |
| 2015-6-19 | 9.0 | FW 22.200.13.00.00 | nineth version | Liuhuan |
| 2015-8-29 | 10.0 | FW 22.200.15.00.00 | MR version | Liuhuan |
| 2015-11-24 | 11.0 | FW 22.315.01.00.00 | MR version | Liuhuan |
| 2016-4-13 | 12.0 | FW 22.317.01.00.00 | MR version | Liuhuan |
| 2016-10-31 | 13.0 | FW 22.321.01.00.00 | MR version | Liuhuan |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	4

2.3	Improvements in the Previous Version	5

2.4	Known Limitations and Issues	5

3	Firmware	5

3.1	Version Description	5

3.2	Firmware Specifications	6

3.3	Improvement in the Previous Version	6

3.4	Known Limitations and Issues	7

4	WebUI/HiLink	7

4.1	Version Description	7

4.2	WebUI/HiLink Specifications	7

4.3	Improvement in the Previous Version	7

4.4	Known Limitations and Issues	8

5	Software Vulnerabilities Fixes	8

6	Accessory Product from other Vendor	8

6.1	Known Limitations and Issues	9

7	Others	9

8	Reference	9

HUAWEI E3372h TCPU-V200R002B321D01SP00C00 Release Notes V13.0

| - Abbreviations | Description |
| --- | --- |

# Main Features

- The E3372h supports the following standards:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- microSD Card Slot (Up to 32G)

- Data and SMS Service

- Plug and play

- Standard USB interface

- CSFB

# Hardware

## Version Description

| Hardware Version: | CL2E3372HM Ver.A |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, |

## Hardware Specifications

| Item | Specifications |
| --- | --- |
| Hardware Version | - CL2E3372HM |
| Technical standard | - LTE 3GPP R9 - HSPA+/UMTS: 3GPP R99/R5/R6/R7/R8 - GSM/GPRS/EDGE: 3GPP R99 |
| External interfaces | - USB: Type A with standard USB 2.0 High speed interface |
|  | LED: indicating the status of the Data Card |
|  | SD card: standard TF card interface |
|  | SIM/USIM card: standard 6-pin SIM card interface |
|  | RF interface: external RF interface |
| Maximum power consumption |  3.5 W |
| Power supply | 5V |
| Dimensions (D × W × H) | - About 88mm(D) × 28mm(W) × 11.5mm (H) |
| Weight |  25 g |
| Temperature | - Operating: –10℃ to +40℃ - Storage: –20℃ to +70℃ |
| Humidity | 5% to 95% |
| Base Information | - Plug and play (PnP) |
|  | - Standard USB 2.0 High Speed interface, auto installation, convenient for use |
| Note: 3GPP = The 3rd Generation Partnership Project TS = Technical Specification LED = Light-Emitting Diode SIM = Subscriber Identity Module USIM = UMTS Subscriber Identity Module |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL2E3372HM Ver.A |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 22.321.01.00.00 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 22.321.01.00.00 |
| Previous Firmware Version |  | 22.317.01.00.00 |
| 1 | DTS2016060102084 | 【E3372h】单板解pin后驻留3G和衰减LTE信号到3G，单板死机 |
| 2 | DTS2016041903166 | 【安全】谷歌4#补丁发布，需要产品分析涉及项并合入。 |
| 3 | DTS2016082305806 | 【V711共性】【mac10.12】安装服务后，在mac上一键升级，工具提示查找端口失败（错误码10），单板变成光盘模式。 |
| 4 | DTS2015111305462 | 【E3372h-153TCPU-V200R002B200D13SP00C00 2015-11-09】恢复出厂设置后，由于恢复出厂nv列表限制，导致定制需求不能实现。 |
| 5 | DTS2016061503963 | 【新需求】【V711共性问题】eUAP在线升级无需登录功能上库 |
| 6 | DTS2016070508092 | 【V711共性问题】【在线升级】【E5573s-156】单板当前为B317D01版本通过一键升级回退到B200的版本，恢复出厂设置后进行在线升级升级到最新的B317D01版本发现在线升级进度条只能到99%，实际在线升级已经成功（但在服务器短不会上报升级成功状态） |
| 7 | DTS2016070702467 | 【新需求】【V711共性】流量统计方案变更，需按照最新的方案实现 |
| 8 | DTS2016070806609 | 【V711共性问题】【euap主线LINUX-V100R007B100D17SP00C03】在线升级重定向不生效 |
| 9 | DTS2016071406444 | 【V711共性问题】【安全】：谷歌2016#7补丁请具体分析涉及项并合入。 |
| 10 | DTS2016071904855 | 【V711共性问题】【自动升级】CPE自动升级下载版本完成在写flash过程中手动下电，单板死机 |
| 11 | DTS2016080201813 | 【新需求】【V711共性问题】eUAP平台支持产品强制升级功能（合入此需求还需要合入两个一般问题DTS2016081203068、DTS2016081010521） |
| 12 | DTS2016081608801 | 【V711共性问题】【开源声明信息不完整_共性问题】开源声明中只有单板的，没有WEBUI的开源声明信息，需要添加WEBUI开源信息。需要重新整理产品的OPEN_SOURCE_NOTICE |
| 13 | DTS2016101004762 | 【V711共性问题】【安全】谷歌10#补丁发布，需要产品分析涉及项并合入。 SLA时间2016.11.5 |
| 14 | DTS2016082305806 | 【V711共性】【mac10.12】安装服务后，在mac上一键升级，工具提示查找端口失败（错误码10），单板变成光盘模式。 |
| 15 | DTS2016102807729 | 【V711共性问题】【安全升级】【必现】非一次性写入的组件A在一键升级时如果异常掉电，单板进升级模式，此时，在升级模式下升级单独的WEBUI版本，发现之前已经写入的组件A的部分数据并未先做处理。 |
| 16 | DTS2016102705280 | 【V711共性问题】【安全_开源声明】【必现】：开源组件netfilter中缺少权利人Harald Welte的描述；组件busybox中缺少权利人Patrick McHardy的描述； |
| 17 | DTS2016103100244 | 【安全漏洞_必现】linux内核Dirty COW(脏牛漏洞)排查需合入，CVE编号CVE-2016-5195 。 |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |
| 2 |  |  |
| 3 |  |  |

# WebUI/HiLink

## Version Description

| WebUI/HiLink Version: | 17.100.14.01.03 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| WebUI Version |  | 17.100.15.02.03 |
| Previous WebUI Version |  | 17.100.14.01.03 |
| 1 | New Features |  |
| 2 |  |  |
| 3 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features |  |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| Linux kernel | Linux kernel 3.14.1 | CVE-2014-2851 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | DTS2015021307041 |
| Linux Kernel | Linux kernel 3.7.10 | CVE-2013-1763 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | DTS2015021307041 |
| Linux kernel | Linux kernel 3.15.6 | CVE-2014-4943 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | DTS2015021307041 |
| Ipv4 | NA | ANDROID-20770158 | 【Android补丁】ANDROID-20770158： ipv4: Missing sk_nulls_node_init() in ping_unhash(). | 在ping_unhash()中缺少sk_nulls_node_init（）。 |
| linux kenerl | Linux kernel 3.4.5 | CVE-2015-1805 | 【安全问题】linux kenerl漏洞 CVE-2015-1805 需要合入 | DTS2016031808763 |
| linux kenerl | Linux kernel 3.4.5 | CVE-2016-5195 | 【安全漏洞_必现】linux内核Dirty COW(脏牛漏洞)排查需合入，CVE编号CVE-2016-5195 。 | DTS2016103100244 |

# Accessory Product from other Vendor

Version Description

| Accessory Product Version: |  |
| --- | --- |

## Known Limitations and Issues

# Others

# Reference