| Product name | Confidentiality level |
| --- | --- |
| E5372 | CONFIDENTIAL |
| Product version | Total 8 pages |
| V2.4 |  |

HUAWEI E5372 Release Notes

V2.4

| Prepared by | Liujuan | Date | 2014/12/10 |
| --- | --- | --- | --- |
| Reviewed by | Zhang Xiaofeng | Date | 2014/12/10 |
| Approved by |  | Date |  |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2013-03-24 | 1.0 | FW 21.200.03.00.00 WebUI 13.100.03.01.03 | The 1st Version for TR4 | Li Yanxin |
| 2013-05-03 | 1.1 | FW 21.221.05.00.00 WebUI 13.100.05.00.03 | The 2nd Version for TR4 | Li Yanxin |
| 2013-06-03 | 1.2 | FW 21.221.15.00.00 WebUI 13.100.06.00.03 | The 3rdVersion for TR5/6 | Li Yanxin |
| 2013-06-18 | 1.3 | FW 21.221.19.00.00 WebUI 13.100.06.00.03 | The 4thVersion for TR5/6 | Li Yanxin |
| 2013-8-20 | 1.4 | FW21.221.23.00.00 WebUI 13.100.07.00.03 | The MR Version | Fanrui |
| 2013-9-23 | 1.5 | FW21.236.03.00.00 WebUI 13.100.08.00.03 | The MR Version | Fanrui |
| 2013-12-5 | 1.6 | FW21.270.07.00.00 WebUI 13.100.08.00.03 | 1 The MR 3rd Version 2 Add support E5372s-620 | Fanrui |
| 2014-1-2 | 1.7 | FW21.270.09.00.00 WebUI 13.100.10.00.03 | 1 Add support E5372Ts-601 | Liujuan |
| 2014/3/11 | 1.8 | FW21.270.11.00.00 WebUI 15.100.10.00.03 | 1 Add support E5372Ts-620 2Add support E5372Ts-22 | Liujuan |
| 2014/3/25 | 1.9 | FW21.270.13.00.00 WebUI 15.100.10.00.03 | 1.The 2nd Version for E5372s-620 TR6 2.Add support E5372Ts-32 | Liujuan |
| 2014/6/23 | 2.0 | FW21.290.15.00.00 WebUI 17.100.02.00.03 | 1 The MR 4rd Version | Liujuan |
| 2014/9/4 | 2.1 | FW21.290.17.00.00 WebUI 17.100.05.00.03 | 1 The MR 5rd Version | Liujuan |
| 2014/12/10 | 2.2 | FW21.290.19.00.00 WebUI 17.100.08.00.03 | The MR 6rd Version | Liujuan |
| 2015/2/6 | 2.3 | FW21.290.21.00.00 WebUI 17.100.08.00.03 | The MR 7th Version | zhouchangfu |
| 2015/4/20 | 2.4 | FW21.290.23.00.00 WebUI 17.100.09.03.03 | The MR 8th Version | ZhouChangfu |

Table of Contents

1	Summary	4

2	Main Features	4

3	Hardware	4

3.1	Version Description	4

3.1.1	Hardware Version	4

3.1.2	Platform & Chipset	5

3.2	Hardware Specifications	5

3.3	Improvements in the Previous Version	6

3.4	Known Limitations and Issues	7

4	Firmware	7

4.1	Version Description	7

4.2	Firmware Specifications	7

4.3	Improvement in the Previous Version	7

4.4	Known Limitations and Issues	7

5	Web UI	8

5.1	Version Description	8

5.2	Web UI Specifications	8

5.3	Improvement in the Previous Version	8

5.4	Known Limitations and Issues	8

HUAWEI E5372 Release Notes V2.4

| - Abbreviations | Description |
| --- | --- |

# Summary

This release notes is applicable to E5372s-32, E5372s-601, E5372s-22 ,E5372s-620, E5372Ts-601, E5372Ts-22, E5372Ts-620, E5372Ts-32..

# Main Features

- The E5372 supports the following standards:

- LTE data service up to 150 Mbit/s(cat 4)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- microSD Card Slot (Up to 32G)

- Data and SMS Service

- WEB UI, Auto connect

- Plug and play

- Standard USB interface

- Support Windows XP/Windows 7/Windows 8/MAC

- Auto PIN

- Support charging to other device

- Support menu operation on LCD

- Support WiFi 2.4G/5G

# Hardware

## Version Description

### Hardware Version

| Index | Product | Hardware Version |
| --- | --- | --- |
| 1 | E5372s-32 | CL1E5372SM |
| 2 | E5372s-601 | CL1E5372SM02 |
| 3 | E5372s-22 | CL1E5372SM03 |
| 4 | E5372s-620 | CL1E5372SM06 |
| 5 | E5372Ts-601 | CL1E5372TSM02 |
| 6 | E5372Ts-22 | CL1E5372TSM03 |
| 7 | E5372Ts-620 | CL1E5372TSM04 |
| 8 | E5372Ts-32 | CL1E5372TSM |

### Platform & Chipset

Balong Hi6920 V7R1, Broadcomm 43241

## Hardware Specifications

| Item | Specifications |  |  |  |
| --- | --- | --- | --- | --- |
| Technical Standard | 3GPP |  |  | R99/R5/R6/R7/R8/R9 |
|  | IEEE |  |  | 802.11b/g/n |
| Operating Frequency | E5372s-32 | LTE |  | B7(2600),B1(2100),B3(1800), B8(900), B20(800) MHz |
|  |  | UMTS |  | B1(2100), B2(1900), B8(900), B5(850) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
|  | E5372s-601 | LTE |  | B3(1800), B7(2600),B40 MHz |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
|  | E5372s-22 | LTE |  | B3(1800),B7(2600),B20(800),B38 MHz |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
|  | E5372s-620 | LTE |  | TDD 2300MHz |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  | E5372Ts-601 | LTE |  | B3(1800), B7(2600),B40 MHz |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
|  | E5372Ts-22 | LTE |  | B3、B7、B20、B38、B41 |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
|  | E5372Ts-620 | LTE |  | TDD 2300MHz |
|  |  | UMTS |  | B1(2100), B8(900) MHz |
|  | E5372Ts-32 | LTE |  | FDD B7(2600), B1(2100), B3(1800), B8(900) |
|  |  | UMTS |  | B1(2100), B2(1900), B8(900), B5(850) MHz |
|  |  | GSM |  | 1900,1800,900,850 MHz |
| Maximum Transmitter Power | LTE |  |  | +23dBm (Class 3) |
|  | UMTS |  |  | +24dBm (Class 3) |
|  | GSM |  |  | GSM/GPRS 850/900MHz: +32dBm (Class 4) GSM/GPRS 1800/1900MHz: +29dBm (Class 1) EDGE 850/900MHz: +26dBm (Class E2) EDGE 1800/1900MHz: +25dBm (Class E2) |
| Maximum Power Consumption | WiFi: 100mW |  |  |  |
| Memory | 256M NAND Flash, 128M DDR SDRAM |  |  |  |
| WLAN Rate | 802.11b: 11Mbit/s, 5.5Mbit/s, 2Mbit/s, 1Mbit/s 802.11g: 54Mbit/s, 48Mbit/s, 36Mbit/s, 24Mbit/s, 18Mbit/s, 12Mbit/s, 9Mbit/s, 6Mbit/s 802.11n: MCS0-MCS7(WiFi 1x1), MSC0-MCS15(WiFi 2x2) |  |  |  |
| External Interfaces | - USB: Micro USB |  |  |  |
|  | LED: 1.45 inch TFT LCD |  |  |  |
|  | SIM/USIM card: 6pin, 1.8/3V |  |  |  |
|  | SD card: up to 32GB |  |  |  |
| Display | - TFT, 1.45inch |  |  |  |
| Keys | 1 Power, 1 Menu, 1 Reset |  |  |  |
| Antenna | Internal |  |  |  |
| Static Receiver Sensitivity | Compliant with 3GPP TS 36.101(R8) for LTE, TS 25.101(R7) for UMTS and TS 05.05(R99) for GSM. |  |  |  |
| Battery | E5372s-32/22/601/620 |  | 1780mAh |  |
|  | E5372Ts-601/-620/-22 |  | 3560mAh |  |
| Dimensions (D × W × H) | E5372s-32/22/601/620 |  | 99.0mm×62.2mm×14.4mm |  |
|  | E5372Ts-601/-620/-22 |  | 99*62*20.5mm |  |
| Weight | <150g(include Battery) |  |  |  |
| Ambient Temperature | 0℃～+35℃ |  |  |  |
| Humidity | 5%-95% |  |  |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | E5372s-32: CL1E5372SM E5372s-601: CL1E5372SM02 E5372s-22: CL1E5372SM03 E5372s-620: CL1E5372SM06 E5372Ts-601:CL1E5372TSM02 E5372Ts-22:CL1E5372TSM03 E5372Ts-620:CL1E5372TSM04 E5372Ts-32:CL1E5372TSM |
| Previous Hardware Version |  | E5372s-32: CL1E5372SM E5372s-601: CL1E5372SM02 E5372s-22: CL1E5372SM03 |
| 1 |  | NA |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.290.23.00.00 |
| --- | --- |
| Baseline information | Hi6920 V7R1 C35B290 |

## Firmware Specifications

| Item | Specifications |
| --- | --- |
| 1 | NA |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.290.23.00.00 |
| Previous Firmware Version |  | 21.290.21.00.00 |
| 1 |  | Safe issue fixed |
| 2 |  |  |
| 3 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 |  | NA |

# Web UI

## Version Description

| Web UI Version: | 17.100.09.03.03 |
| --- | --- |

## Web UI Specifications

| Item | Specifications |
| --- | --- |
| 1 | NA |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| WebUI Version |  | 17.100.09.03.03 |
| Previous WebUI Version |  | 17.100.08.00.03 |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | NA | NA |