| Product name | Confidentiality level |
| --- | --- |
| E5577Bs-932 | CONFIDENTIAL |
| Product version | Total 6 pages |
| V1.0 |  |

HUAWEI E5577Bs-932TCPU-V200R001B318D03SP00C1133 Release Notes V1.0

| Prepared by | E5577s Team | Date | 2016/6/23 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2016/06/23 | 1.0 | FW 21.318.03.00.1133 | The 1en Version | E5577s Team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	4

2.3	Improvements in the Previous Version	5

2.4	Known Limitations and Issues	6

3	Firmware	6

3.1	Version Description	6

3.2	Firmware Specifications	6

3.3	Improvement in the Previous Version	6

3.4	Known Limitations and Issues	7

4	Software Vulnerabilities Fixes	7

5	Accessory Product from other Vendor	7

5.1	Known Limitations and Issues	7

6	Others	7

7	Reference	7

HUAWEI E5577Bs-932TCPU-V200R001B318D03SP00C1133 Release Notes V1.0

| - Abbreviations | Description |
| --- | --- |

# Main Features

- The E5577Bs-932 supports the following features:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- Data and SMS Service

- WEB UI, Auto connect

- Plug and play

- Standard USB2.0

- Support WiFi 2.4GHz/5GHz

- Support LCD

# Hardware

## Version Description

| Hardware Version: | CL1E5577SM03 Ver.A |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, Broadcomm 43241 |

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical Standard | 3GPP | R99/R5/R6/R7/R8/R9 |
|  | IEEE | 802.11a/b/g/n |
| Operating Frequency | LTE | TDD B38,B40,B41;FDD B1, B3, B8 |
|  | UMTS | B1, B8 |
|  | GSM | B2,B3,B5,B8 |
| Maximum Transmitter Power | LTE | +23dBm (Class 3) |
|  | UMTS | +24dBm (Class 3) |
|  | GSM | GSM/GPRS 850M/900MHz： +33dBm (Power Class 4) GSM/GPRS 1800MHz/1900MHz： +30dBm (Power Class 1) EDGE 850M/900MHz： +27dBm (Power Class E2) EDGE 1800MHz/1900MHz： +26dBm (Power Class E2) |
| Maximum Power Consumption | 3.5W |  |
| Memory | 128M NAND Flash, 128M DDR |  |
| WLAN Rate | 802.11a:54Mbit/s,48Mbit/s,36Mbit/s,24Mbit/s,18Mbit/s,12Mbit/s,9Mbit/s,6Mbit/s 802.11b: 11Mbit/s, 5.5Mbit/s, 2Mbit/s, 1Mbit/s 802.11g: 54Mbit/s, 48Mbit/s, 36Mbit/s, 24Mbit/s, 18Mbit/s, 12Mbit/s, 9Mbit/s, 6Mbit/s 802.11n: MCS0-MCS7(WiFi 1x1), MSC0-MCS15(WiFi 2x2) |  |
| External Interfaces | - USB: Standard USB2.0 |  |
|  | SD CARD |  |
|  | LCD |  |
|  | SIM/USIM card: 6pin, 1.8/3V |  |
| Display | - LCD |  |
| Keys | 1 Power, 1 Reset,1 Menu |  |
| Antenna | Internal |  |
| Static Receiver Sensitivity | Compliant with 3GPP TS 36.101(R9) for LTE, TS 25.101(R8) for UMTS. |  |
| Battery | 3000mAh |  |
| Dimensions (D × W × H) | 96.8mm×58.0mm×17.3mm |  |
| Weight | <110g(include Battery) |  |
| Ambient Temperature | 0-35°C |  |
| Humidity | 5%-95% |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL1E5577SM Ver.A |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.318.03.00.1133 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.318.03.00.1133 |
| Previous Firmware Version |  | 21.318.03.00.00 |
| 1 | Band customize | When use 42003,42075 sim card, it cannot be register on FDD1800 |
| 2 | Network Type | When insert 42001 sim card, the Network type option should add 4G TDD2300，Network type option should be Auto, 4G only, 3G only, 2G only, 4G TDD2300，set Auto as default When select 4G TDD2300 in last time,and then insert nor-42001 simcard,network need change to Auto mode. other status as default. |
| 3 | LTE ability | Modify LTE ability from R9 to R8 |
| 4 | Operator name and Roaming white list | 1.when simcard's PLMN is 42005 and SPN is Virgin Mobile,when register to 42001's network,Operator name will display SPN name(Virgin Mobile),and it can not dispaly Roaming. 2.when simcard's PLMN is 42005 and SPN is FRiENDi mobile,when register to 42001's network,Operator name will display SPN name(FRiENDi mobile),and it can not dispaly Roaming. 3.when simcard's PLMN is 42005 and SPN is null or other string,when register to 42001's network,Operator name will display 42005,and it can not dispaly Roaming. |
| 5 | Profile Name disaply | When simcard is 42005, Profile Name display as simcard's SPN.(Virgin Mobile or FRiENDi mobile) PS:if simcard's SPN is not Virgin Mobile or FRiENDi mobile(simcard 42005),and it can not adapter by SPN，pls use Dynamic APN to dail up and Profile Name dispaly AutoAPN. |
| 6 | PDP Type(IPv4 or IPv4v6) | Support IPv4 and IPv4V6,and IPv4 by default. |
| 7 | Search network sequence | Search network sequence: TDD>FDD |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | DTS2016062706041 | 【新需求】eUAP在线升级无需登录功能上库 |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |

# Accessory Product from other Vendor

Version Description

| Accessory Product Version: |  |
| --- | --- |

## Known Limitations and Issues

# Others

# Reference