| Product name | Confidentiality level |
| --- | --- |
| E5577s-932 | CONFIDENTIAL |
| Product version | Total 6 pages |
| V10.0 |  |

HUAWEI E5577s-932TCPU-V200R001B328D03SP00C00 Release Notes V10.0

| Prepared by | E5577s Team | Date | 2017/9/6 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2014/11/26 | 1.0 | FW 21.180.03.00.00 | The 1st Version | Fangxiaolong/wx218104 |
| 2014/12/30 | 2.0 | FW 21.200.01.00.00 | The 2nd Version | Fangxiaolong/wx218104 |
| 2015/01/26 | 3.0 | FW 21.200.03.00.00 | The 3rd Version | Fangxiaolong/wx218104 |
| 2015/02/16 | 4.0 | FW 21.200.05.00.00 | The 4ur Version | Fangxiaolong/wx218104 |
| 2015/03/12 | 5.0 | FW 21.200.09.00.00 | The 5ve Version | Fangxiaolong/wx218104 |
| 2015/03/20 | 6.0 | FW 21.200.09.00.00 | The 6ix Version | Fangxiaolong/wx218104 |
| 2016/05/30 | 7.0 | FW 21.318.01.00.00 WEBUI_17.100.14.01.03 | The 7en Version | Liuhuan 00219313 |
| 2016/06/22 | 8.0 | FW 21.318.03.00.00 WEBUI_17.100.14.01.03 | The 8th Version | Liuhuan 00219313 |
| 2017/09/6 | 9.0 | FW 21.328.01.00.00 WEBUI_17.100.19.01.03 | The 9th Version | Liuhuan 00219313 |
| 2017/10/20 | 10.0 | FW 21.328.03.00.00 WEBUI_17.100.19.02.03 | The 10th Version | Liuhuan 00219313 |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	4

2.3	Improvements in the Previous Version	5

2.4	Known Limitations and Issues	6

3	Firmware	6

3.1	Version Description	6

3.2	Firmware Specifications	6

3.3	Improvement in the Previous Version	6

3.4	Known Limitations and Issues	6

4	WebUI/HiLink	6

4.1	Version Description	6

4.2	WebUI/HiLink Specifications	7

4.3	Improvement in the Previous Version	7

4.4	Known Limitations and Issues	7

5	Accessory Product from other Vendor	7

5.1	Version Description	16

5.2	Known Limitations and Issues	17

6	Others	17

7	Reference	17

HUAWEI E5577s-932TCPU-V200R001B328D03SP00C00 Release Notes V10.0

| - Abbreviations | Description |
| --- | --- |

# Main Features

- The E5577s-932 supports the following features:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- Data and SMS Service

- WEB UI, Auto connect

- Plug and play

- Standard USB2.0

- Support WiFi 2.4GHz/5GHz

- Support LCD

# Hardware

## Version Description

| Hardware Version: | CL1E5577SM Ver.A |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, Broadcomm 43241 |

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical Standard | 3GPP | R99/R5/R6/R7/R8/R9 |
|  | IEEE | 802.11a/b/g/n |
| Operating Frequency | LTE | TDD B38,B40,B41;FDD B1, B3 |
|  | UMTS | B1, B8 |
|  | GSM | B2,B3,B5,B8 |
| Maximum Transmitter Power | LTE | +23dBm (Class 3) |
|  | UMTS | +24dBm (Class 3) |
|  | GSM | GSM/GPRS 850M/900MHz： +33dBm (Power Class 4) GSM/GPRS 1800MHz/1900MHz： +30dBm (Power Class 1) EDGE 850M/900MHz： +27dBm (Power Class E2) EDGE 1800MHz/1900MHz： +26dBm (Power Class E2) |
| Maximum Power Consumption | 3.5W |  |
| Memory | 128M NAND Flash, 128M DDR |  |
| WLAN Rate | 802.11a:54Mbit/s,48Mbit/s,36Mbit/s,24Mbit/s,18Mbit/s,12Mbit/s,9Mbit/s,6Mbit/s 802.11b: 11Mbit/s, 5.5Mbit/s, 2Mbit/s, 1Mbit/s 802.11g: 54Mbit/s, 48Mbit/s, 36Mbit/s, 24Mbit/s, 18Mbit/s, 12Mbit/s, 9Mbit/s, 6Mbit/s 802.11n: MCS0-MCS7(WiFi 1x1), MSC0-MCS15(WiFi 2x2) |  |
| External Interfaces | - USB: Standard USB2.0 |  |
|  | SD CARD |  |
|  | LCD |  |
|  | SIM/USIM card: 6pin, 1.8/3V |  |
| Display | - LCD |  |
| Keys | 1 Power, 1 Reset,1 Menu |  |
| Antenna | Internal |  |
| Static Receiver Sensitivity | Compliant with 3GPP TS 36.101(R9) for LTE, TS 25.101(R8) for UMTS. |  |
| Battery | 3000mAh |  |
| Dimensions (D × W × H) | 96.8mm×58.0mm×17.3mm |  |
| Weight | <110g(include Battery) |  |
| Ambient Temperature | 0-35°C |  |
| Humidity | 5%-95% |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL1E5577SM Ver.A |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.328.03.00.00 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.328.03.00.00 |
| Previous Firmware Version |  | 21.328.01.00.00 |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |
| 2 |  |  |
| 3 |  |  |

# WebUI/HiLink

## Version Description

| WebUI/HiLink Version: | 17.100.19.02.03 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| WebUI Version |  | 17.100.19.02.03 |
| Previous WebUI Version |  | 17.100.19.01.03 |
| 1 | New Features |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features |  |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

[Software Vulnerabilities include Android Vulnerability, Third-party software Vulnerability, and Huawei Vulnerability]

[Android Vulnerability is from Google, which reported publicly.]

[Third-party software is a type of computer software that is sold together with or provided for free in Huawei products or solutions with the ownership of intellectual property rights (IPR) held by the original contributors. Third-party software can be but is not limited to: Purchased software, Software that is built in or attached to purchased hardware, Software in products of the original equipment manufacturer (OEM) or original design manufacturer (ODM), Software that is developed with technical contribution from partners (ownership of IPR all or partially held by the partners), Software that is legally obtained free of charge.

The data of third-party software vulnerabilities fixes can be exported from PDM.

If the table is excessively long, you can divide it into multiple ones by product version, or deliver it in an excel file with patch release notes and provide reference information in this section.]

[Huawei Vulnerability is Huawei own software’ Vulnerability, which found by outside]

Vulnerabilities information is available through CVE IDs in NVD (National Vulnerability Database) website: http://web.nvd.nist.gov/view/vuln/search

| Software/Module name | Version | CVE ID | Vulnerability Description | Impact Description |
| --- | --- | --- | --- | --- |
| linux_kernel | 3.4.5 | CVE-2016-5195 | Race condition in mm/gup.c in the Linux kernel 2.x through 4.x before 4.8.3 allows local users to gain privileges by leveraging incorrect handling of a copy-on-write (COW) feature to write to a read-only memory mapping, as exploited in the wild in October 2016, aka "Dirty COW." | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=19be0eaffa3ac7d8eb6784ad9bdbc7d67ed8e619 |
| Android | 4.4.4, 5.0.2, 5.1.1 | CVE-2016-6700 | The entry->uncompressedSize and entry->data values could be modified to a position beyond the bounds of the buffer, which could lead to the possibility of code execution. The fix is designed to add bounds checks and reject zip file entries that have no filenames. | https://source.android.com/security/bulletin/2016-11-01.html |
| linux_kernel | 3.10, 3.18 | CVE-2016-6828 | When the tcp_sendmsg function allocates a fresh and empty skb, it puts it at the tail of the write queue. On a failure condition, a dangling pointer is left leading to a potential use-after-free vulnerability. The fix is designed to set the highest_sack variable to null to prevent the potential use-after-free vulnerability. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=bb1fceca22492109be12640d49f5ea5a544c6bb4 |
| linux_kernel | 3.10, 3.18 | CVE-2016-7910 | There is a failure condition that can lead to a potential use-after-free vulnerability. The fix is designed to set the private pointer to NULL to prevent the potential use-after-free vulnerabilit | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=77da160530dd1dc94f6ae15a981f24e5f0021e84 |
| linux_kernel | 3.10, 3.18 | CVE-2016-7911 | There is a race condition accessing task->io_context that can potentially lead to a use-after-free vulnerability. The fix is designed to add locking to prevent the potential use-after-free vuln | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=8ba8682107ee2ca3347354e018865d8e1967c5f4 |
| linux_kernel | 3.10, 3.18 | CVE-2015-8964 | The line discipline drivers may mistakenly misuse ldisc-related fields when initializing leading to a potential information disclosure. The fix is designed to initialize relevant tty fields before instancing the new line discipline to prevent the potential information disclosure. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=dd42bf1197144ede075a9d4793123f7689e164bc |
| linux_kernel | 3.18 | CVE-2016-6753 | The format specifier %p can leak kernel addresses while not valuing the kptr_restrict system settings. The fix is designed to use %pK instead of %p, which also evaluates whether kptr_restrict is set. | https://source.android.com/security/bulletin/2016-11-01.html |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2014-9529 | There is a race condition in the key_gc_unused_keys function in security/keys/gc.c file, in the Linux kernel through 3.18.2, that allows local users to execute code in the kernel or possibly have unspecified other impact via keyctl commands that trigger access to a key structure member during garbage collection of a key. The fix is designed to put the key->user after the destroy callback is called to prevent the race condition. Link to publicly available patch: Upstream kernel fix | http://git.kernel.org/?p=linux/kernel/git/torvalds/linux-2.6.git;a=commit;h=a3a8784454692dd72e5d5d34dcdab17b4420e74c |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2016-4470 | The key_reject_and_link function in the security/keys/key.c file, in the Linux kernel through 4.6.3, contains an error in which a key-lookup could fail and in an attempt to cache, the failed lookup may attempt to free memory that could still be in use, leading to a potential use-after-free vulnerability. The fix is designed to add an additional check condition to ensure the data structure is initialized and prevents the potential use-after-free vulnerability. Link to publicly available patch: Upstream kernel fix | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=38327424b40bcebe2de92d07312c89360ac9229a |
| linux_kernel | 3.4 | CVE-2015-2922 | The ndisc_router_discovery function in net/ipv6/ndisc.c in the Neighbor Discovery (ND) protocol implementation in the IPv6 stack in the Linux kernel before 3.19.6 allows remote attackers to reconfigure a hop-limit setting via a small hop_limit value in a Router Advertisement (RA) message. | https://git.kernel.org/pub/scm/linux/kernel/git/torvalds/linux.git/commit/?id=38327424b40bcebe2de92d07312c89360ac9229a |
| linux_kernel | 3.4 | CVE-2014-8173 | The pmd_none_or_trans_huge_or_clear_bad function in include/asm-generic/pgtable.h in the Linux kernel before 3.13 on NUMA systems does not properly determine whether a Page Middle Directory (PMD) entry is a transparent huge-table entry, which allows local users to cause a denial of service (NULL pointer dereference and system crash) or possibly have unspecified other impact via a crafted MADV_WILLNEED madvise system call that leverages the absence of a page-table lock. | https://github.com/torvalds/linux/commit/ee53664bda169f519ce3c6a22d378f0b946c8178 |
| Android | 4.4.4, 5.0.2, 5.1.1 | CVE-2016-2842 | The internal ¦fmtstr¦ function used in processing a "%s" format string in the BIO_*printf functions could overflow while calculating the length of a string and cause an out of bounds read when printing very long strings. Additionally, the internal ¦doapr_outch¦ function can attempt to write to an out of bounds memory location (at an offset from the NULL pointer) in the event of a memory allocation failure. The fix is designed to address these memory issues. | https://git.openssl.org/?p=openssl.git;a=commit;h=578b956fe741bf8e84055547b1e83c28dd902c73 |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2015-2686 | The net/socket.c file, in the Linux kernel before 3.19.3, does not validate certain range data for sendto and recvfrom system calls. This could allow local users to gain privileges by leveraging a subsystem that uses the copy_from_iter function in the iov_iter interface, as demonstrated by the Bluetooth subsystem. The fix is to designed to add additional bounds checks for validation. Link to publicly available patch: Upstream kernel fix | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=4de930efc23b92ddf88ce91c405ee645fe6e27ea |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2016-3841 | There is a race condition in the kernel networking component that could lead to a use-after-free vulnerability. The fix is to designed to add read-copy-update synchronization protection to prevent the race condition. Link to publicly available patch: Upstream kernel fix | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=45f6fad84cc305103b28d73482b344d7f5b76f39 |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2016-4482 | The proc_connectinfo function in the drivers/usb/core/devio.c file, in the Linux kernel through 4.6, does not initialize a certain data structure, which could allow local users to obtain sensitive information from kernel stack memory via a crafted USBDEVFS_CONNECTINFO IOCTL call. The fix is designed to zero out the usbdevfs_connectinfo struct before using it so no sensitive information can be returned. Link to publicly available patch: Upstream kernel fix | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=681fef8380eb818c0b845fca5d2ab1dcbab114ee |
| linux_kernel | 3.4, 3.10, 3.18 | CVE-2016-4578 | sound/core/timer.c in the Linux kernel through 4.6 does not initialize certain r1 data structures, which allows local users to obtain sensitive information from kernel stack memory via crafted use of the ALSA timer interface, related to the (1) snd_timer_user_ccallback and (2) snd_timer_user_tinterrupt functions. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=9a47e9cff994f37f7f0dbd9ae23740d0f64f9fe6 |
| linux_kernel | 3.4, 3.10, 3.18 | ## CVE-2016-4569 | The snd_timer_user_params function in sound/core/timer.c in the Linux kernel through 4.6 does not initialize a certain data structure, which allows local users to obtain sensitive information from kernel stack memory via crafted use of the ALSA timer interface. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=cec8f96e49d9be372fdb0c3836dcf31ec71e457e |
| OpenSSL | 1.0.1o and 1.0.2 | ## CVE-2016-2108 | The ASN.1 implementation in OpenSSL before 1.0.1o and 1.0.2 before 1.0.2c allows remote attackers to execute arbitrary code or cause a denial of service (buffer underflow and memory corruption) via an ANY field in crafted serialized data, aka the "negative zero" issue. | https://git.openssl.org/?p=openssl.git;a=commit;h=3661bb4e7934668bd99ca777ea8b30eedfafa871 |
| linux_kernel | 3.4.5 | CVE-2016-2493 | The Broadcom Wi-Fi driver in Android before 2016-06-01 on Nexus 5, Nexus 6, Nexus 6P, Nexus 7 (2013), Nexus Player, and Pixel C devices allows attackers to gain privileges via a crafted application, aka internal bug 26571522. | Google 2016 5# https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-2493 |
| linux_kernel | All Linux version | CVE-2016-0774 | The (1) pipe_read and (2) pipe_write implementations in fs/pipe.c in a certain Linux kernel backport in the linux package before 3.2.73-2+deb7u3 on Debian wheezy and the kernel package before 3.10.0-229.26.2 on Red Hat Enterprise Linux (RHEL) 7.1 do not properly consider the side effects of failed __copy_to_user_inatomic and __copy_from_user_inatomic calls, which allows local users to cause a denial of service (system crash) or possibly gain privileges via a crafted application, aka an "I/O vector array overrun." NOTE: this vulnerability exists because of an incorrect fix for CVE-2015-1805. | Google 2016 4# https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-0774 |
| linux_kernel | All Linux version | CVE-2016-2438 | DO NOT USE THIS CANDIDATE NUMBER. ConsultIDs: CVE-2016-2547, CVE-2016-2548. Reason: This candidate is a duplicate of CVE-2016-2547 and CVE-2016-2548. Notes: All CVE users should reference CVE-2016-2547 and/or CVE-2016-2548 instead of this candidate. All references and descriptions in this candidate have been removed to prevent accidental usage. | Google 2016 4# https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-2438 |
| Android | 4.4.4, 5.0.2, 5.1.1, 6.0, 6.0.1 | CVE-2016-2447 | DO NOT USE THIS CANDIDATE NUMBER. ConsultIDs: CVE-2016-4477. Reason: This candidate is a reservation duplicate of CVE-2016-4477. Notes: All CVE users should reference CVE-2016-4477 instead of this candidate. All references and descriptions in this candidate have been removed to prevent accidental usage. | Google 2016 4# http://source.android.com/security/bulletin/2016-05-01.html |
| linux_kernel | before 3.16 | CVE-2015-1805 | The (1) pipe_read and (2) pipe_write implementations in fs/pipe.c in the Linux kernel before 3.16 do not properly consider the side effects of failed __copy_to_user_inatomic and __copy_from_user_inatomic calls, which allows local users to cause a denial of service (system crash) or possibly gain privileges via a crafted application, aka an "I/O vector array overrun." | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=f0d1bec9d58d4c038d0ac958c9af82be6eb18045 |
| linux_kernel | Linux kernel through 4.9.9 | CVE-2017-5970 | Technical details: The ipv4_pktinfo_prepare function in net/ipv4/ip_sockglue.c in the Linux kernel through 4.9.9 allows attackers to cause a denial of service (system crash) via (1) an application that makes crafted system calls or possibly (2) IPv4 traffic with invalid IP options. This is due to dropping dst when bad IP options were present which could lead to a NULL pointer dereference. Fix details: The fix is designed to only drop the dst packet if it's safe to do so. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=34b2cef20f19c87999fff3da4071e66937db9644 |
| linux_kernel | 3.4.5 | CVE-2017-0710 | Technical details: A process with CAP_SYS_RESOURCE bypasses the permission check allowing arbitrary ptrace access. Fix details: The fix replaced CAP_SYS_RESOURCE with CAP_SYS_PTRACE for processes needing ptrace capability, and removed the CAP_SYS_RESOURCE bypass. |  |
| linux_kernel | 3.4.5 | CVE-2016-9555 | The sctp_sf_ootb function in net/sctp/sm_statefuns.c in the Linux kernel before 4.8.8 lacks chunk-length checking for the first chunk, which allows remote attackers to cause a denial of service (out-of-bounds slab access) or possibly have unspecified other impact via crafted SCTP data. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=bf911e985d6bbaa328c20c3e05f4eb03de11fdd6 |
| linux_kernel | 3.4.5 | CVE-2017-9074 | The IPv6 fragmentation implementation in the Linux kernel through 4.11.1 does not consider that the nexthdr field may be associated with an invalid option, which allows local users to cause a denial of service (out-of-bounds read and BUG) or possibly have unspecified other impact via crafted socket and send system calls. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=2423496af35d94a87156b063ea5cedffc10a70a1 |
| linux_kernel | 3.4.5 | CVE-2017-7487 | The ipxitf_ioctl function in net/ipx/af_ipx.c in the Linux kernel through 4.11.1 mishandles reference counts, which allows local users to cause a denial of service (use-after-free) or possibly have unspecified other impact via a failed SIOCGIFADDR ioctl call for an IPX interface. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=ee0d8d8482345ff97a75a7d747efc309f13b0d80 |
| linux_kernel | 3.4.5 | CVE-2017-9242 | The __ip6_append_data function in net/ipv6/ip6_output.c in the Linux kernel through 4.11.3 is too late in checking whether an overwrite of an skb data structure may occur, which allows local users to cause a denial of service (system crash) via crafted system calls. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=232cd35d0804cc241eb887bb8d4d9b3b9881c64a |
| linux_kernel | 3.4.5 | CVE-2017-8890 | The inet_csk_clone_lock function in net/ipv4/inet_connection_sock.c in the Linux kernel through 4.10.15 allows attackers to cause a denial of service (double free) or possibly have unspecified other impact by leveraging use of the accept system call. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=657831ffc38e30092a2d5f03d385d710eb88b09a |
| linux_kernel | 3.4.5 | CVE-2017-9075 | The sctp_v6_create_accept_sk function in net/sctp/ipv6.c in the Linux kernel through 4.11.1 mishandles inheritance, which allows local users to cause a denial of service or possibly have unspecified other impact via crafted system calls, a related issue to CVE-2017-8890. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=fdcee2cbb8438702ea1b328fb6e0ac5e9a40c7f8 |
| linux_kernel | 3.4.5 | CVE-2017-9076 | The dccp_v6_request_recv_sock function in net/dccp/ipv6.c in the Linux kernel through 4.11.1 mishandles inheritance, which allows local users to cause a denial of service or possibly have unspecified other impact via crafted system calls, a related issue to CVE-2017-8890. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=83eaddab4378db256d00d295bda6ca997cd13a52 |
| linux_kernel | 3.4.5 | CVE-2017-9077 | The tcp_v6_syn_recv_sock function in net/ipv6/tcp_ipv6.c in the Linux kernel through 4.11.1 mishandles inheritance, which allows local users to cause a denial of service or possibly have unspecified other impact via crafted system calls, a related issue to CVE-2017-8890. | https://git.kernel.org/pub/scm/linux/kernel/git/torvalds/linux.git/commit/?id=83eaddab4378db256d00d295bda6ca997cd13a52 |
| linux_kernel | 3.4.5 | CVE-2016-4913 | The get_rock_ridge_filename function in fs/isofs/rock.c in the Linux kernel before 4.5.5 mishandles NM (aka alternate name) entries containing \0 characters, which allows local users to obtain sensitive information from kernel memory or possibly have unspecified other impact via a crafted isofs filesystem. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=99d825822eade8d827a1817357cbf3f889a552d6 |
| linux_kernel | 3.4.5 | CVE-2017-7472 | The KEYS subsystem in the Linux kernel before 4.10.13 allows local users to cause a denial of service (memory consumption) via a series of KEY_REQKEY_DEFL_THREAD_KEYRING keyctl_set_reqkey_keyring calls. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=c9f838d104fed6f2f61d68164712e3204bf5271b |
| linux_kernel | 3.4.5 | CVE-2015-8966 | arch/arm/kernel/sys_oabi-compat.c in the Linux kernel before 4.4 allows local users to gain privileges via a crafted (1) F_OFD_GETLK, (2) F_OFD_SETLK, or (3) F_OFD_SETLKW command in an fcntl64 system call. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=76cc404bfdc0d419c720de4daaf2584542734f42 |
| linux_kernel | 3.4.5 | CVE-2016-7117 | Use-after-free vulnerability in the __sys_recvmmsg function in net/socket.c in the Linux kernel before 4.5.2 allows remote attackers to execute arbitrary code via vectors involving a recvmmsg system call that is mishandled during error processing. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=34b88a68f26a75e4fded796f1a49c40f82234b7d |
|  |  | CVE-2017-0427 | If a thread?€?s logd.auditd is scheduled while the group_leader of the thread?€?s group is killed, that leader may be freed before the logging is completed. This may lead to use-after-free and memory corruption in the kernel, and possible code execution. | Google 2017 11# https://source.android.com/security/bulletin/2017-02-01.html |

# Accessory Product from other Vendor

Version Description

| Accessory Product Version: |  |
| --- | --- |

## Known Limitations and Issues

# Others

# Reference