| Product name | Confidentiality level |
| --- | --- |
| E5770s-320 | CONFIDENTIAL |
| Product version | Total 9 pages |
| V1.0 |  |

HUAWEI E5770s-320TCPU-V200R001B318D01SP00C00

Release Notes V1.0

| Prepared by | E5770s-320 Team | Date | 2016/05/11 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2016/05/11 | 1.0 | FW 21.318.01.00.00 | The first Version | E5770s-320 Team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	5

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	7

2.4	Known Limitations and Issues	7

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	7

3.4	Known Limitations and Issues	8

4	WebUI/HiLink	8

4.1	Version Description	8

4.2	WebUI/HiLink Specifications	8

4.3	Improvement in the Previous Version	8

4.4	Known Limitations and Issues	8

5	Software Vulnerabilities Fixes	9

6 	Accessory Product from other Vendor	9

6.1	Version Description	9

6.2	Known Limitations and Issues	9

7	  Others	9

8	Reference	9

HUAWEI E5770s-320TCPU-V200R001B318D01SP00C00 Release Notes V1.0

| - Abbreviations | Description |
| --- | --- |

# Main Features

- The E5770s-320 supports the following features:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s (Configuration 2)

- LTE TDD (UL) data service of up to 20 Mbit/s (Configuration 1)

- DC-HSPA+ (DL) data service of up to 43.2 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- GPRS data service of up to 85.6 kbit/s

- Ethernet data service of up to100 Mbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on UMTS and GSM

- Built-in LTE/UMTS/GSM and WLAN high gain antenna

- Micro Secure Digital Memory (microSD) Card

- SIM lock

- Wi-Fi and WPS

- LTE/3G/Wi-Fi auto offload

- Supply power to devices (working as a power bank)

- Support for HUAWEI Mobile WiFi App

- Press and Play

- IPv6/IPv4 dual stack (optional)

- Display current SSID and Wi-Fi key on the screen

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- Ethernet port

- PPPoE dial up

- Standard Micro USB interface

- USB interface

- OLED-LCD screen

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Version Description

| Hardware Version: | CL2E5770SM01 Ver.A |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, RTL8192 |

## Hardware Specifications

## 2.2.1 Hardware

2.2 lists the hardware specifications.

- Hardware specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical standard | - WAN: LTE/DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |
|  | - WLAN: IEEE 802.11b/g/n |  |
| Operating frequency | LTE： - FDD B1(2100 MHz)/B3(1800 MHz)/B5(850 MHz)/B7(2600 MHz)/B8(900 MHz)/B20(800 MHz) - TDD B38(2600 MHz) |  |
|  | DC-HSPA+/HSPA+/HSPA/UMTS：B1(2100 MHz)/B2(1900 MHz)/B5(850 MHz)/B8(900 MHz) |  |
|  | EDGE/GPRS/GSM: 1900/1800/900/850 MHz |  |
|  | WLAN: 2.4 GHz |  |
| Internal memory | 128 MB NAND flash and 128 MB DDR SDRAM |  |
| Maximum transmitter power | LTE: Conform to Power Class 3 Definition |  |
|  | UMTS: Conform to Power Class 3 Definition |  |
|  | WLAN | 802.11b: 15 dBm |
|  |  | 802.11g: 12 dBm |
|  |  | 802.11n: 11 dBm |
| Receiver sensitivity | LTE: Confirm to 3GPP Requirements |  |
|  | UMTS: Confirm to 3GPP Requirements |  |
|  | WLAN | 802.11b: -76 dBm@11 Mbit/s |
|  |  | 802.11g: -65 dBm@54 Mbit/s |
|  |  | 802.11n: -64 dBm@65 Mbit/s |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |
|  | 802.11g: Up to 54 Mbit/s |  |
|  | 802.11n | HT20: Support MCS0–MCS7; Up to 72.2 Mbit/s. Support MCS8–MCS15; Up to 144.4 Mbit/s. HT40: Support MCS0–MCS7; Up to 150 Mbit/s. Support MCS8–MCS15; Up to 300 Mbit/s. |
| Maximum power consumption | 3.5 W |  |
| Charger | - AC: 100–240 V (Input) |  |
|  | - DC: 5 V, 2 A (Output) |  |
| Supplying Power to Devices | - rated output: 5 V, 2 A |  |
| Battery | - Type: Li (rechargeable and irremovable) |  |
|  | - Capacity: 3.7 V, 5200 mAh |  |
|  | - Maximum working time: 20 hours (depending on the network) |  |
|  | - Maximum standby time: 500 hours (depending on the network) |  |
| External interfaces | Micro USB interface |  |
|  | USB interface (supplying power to other devices) |  |
|  | Standard microSD card interface |  |
|  | Standard 6-pin SIM card interface |  |
|  | Ethernet port: RJ45 |  |
| Screen | OLED |  |
| key-press | Power switch, WPS switch, Reset switch |  |
| Antenna | - Built-in LTE/UMTS/GSM main antenna |  |
|  | - Built-in LTE/UMTS diversity antenna |  |
|  | - Built-in WLAN antenna |  |
| Dimensions (W × D × H) | 106.0 mm × 68.4 mm × 22.5 mm |  |
| Weight | about 200 g (including the battery) |  |
| Temperature | - Operating: 0°C to +35°C |  |
|  | - Storage: -20°C to +60°C |  |
| Humidity | 5% to 95% (non-condensing) |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL2E5770SM01 Ver.A |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.318.01.00.00 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.318.01.00.00 |
| Previous Firmware Version |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |
| 2 |  |  |
| 3 |  |  |

# WebUI/HiLink

## Version Description

| WebUI/HiLink Version: | 17.100.14.01.00 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |
| NA |  |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| WebUI Version |  | 17.100.14.01.00 |
| Previous WebUI Version |  | N/A |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | NA |  |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| NA |  |  |  |  |

# Accessory Product from other Vendor

## Version Description

| Accessory Product Version: |  |
| --- | --- |

## Known Limitations and Issues

# Others

# Reference