| Product name | Confidentiality level |
| --- | --- |
| E5770s | CONFIDENTIAL |
| Product version | Total 8 pages |
| V1.0 |  |

E5770s Firmware Release Notes

V1.0

| Prepared by | liuli | Date | 2014-4-7 |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |
| Approved by |  | Date |  |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/Stick Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2015-4-7 | 1.0 | FW 21.200.15.00.00 | First version | liuli |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Hardware Specifications	5

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	7

3.4	Known Limitations and Issues	12

E5770s Firmware Release Notes V1.0

# Main Features

- The E5770s supports the following standards:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s (Configuration 2)

- LTE TDD (UL) data service of up to 20 Mbit/s (Configuration 1)

- DC-HSPA+ (DL) data service of up to 43.2 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- GPRS data service of up to 85.6 kbit/s

- Ethernet data service of up to100 Mbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on UMTS and GSM

- Built-in LTE/UMTS/GSM and WLAN high gain antenna

- Micro Secure Digital Memory (microSD) Card

- SIM lock

- Wi-Fi and WPS

- LTE/3G/Wi-Fi auto offload

- Supply power to devices (working as a power bank)

- Support for HUAWEI Mobile WiFi App

- Press and Play

- IPv6/IPv4 dual stack (optional)

- Display current SSID and Wi-Fi key on the screen

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- Ethernet port

- PPPoE dial up

- Standard Micro USB interface

- USB interface

- OLED-LCD screen

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical standard | - WAN: LTE/DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |
|  | - WLAN: IEEE 802.11b/g/n |  |
| Operating frequency | LTE: - FDD B1(2100 MHz)/B3(1800 MHz)/B7(2600 MHz) - TDD B38(2600 MHz)/B40(2400 MHz)/B41(2600 MHz) |  |
|  | DC-HSPA+/HSPA+/HSPA/UMTS: B1(2100 MHz)/B8(900 MHz) |  |
|  | EDGE/GPRS/GSM: B2(1900MHz)/B3(1800MHz)/B8(900MHz)/B5(850MHz) |  |
|  | WLAN: 2.4 GHz |  |
| Internal memory | 128 MB NAND flash and 128 MB DDR SDRAM |  |
| Maximum transmitter power | LTE: Conform to Power Class 3 Definition |  |
|  | UMTS: Conform to Power Class 3 Definition |  |
|  | WLAN | 802.11b: 12 dBm |
|  |  | 802.11g: 10 dBm |
|  |  | 802.11n: 9 dBm |
| Receiver sensitivity | LTE: Confirm to 3GPP Requirements |  |
|  | UMTS: Confirm to 3GPP Requirements |  |
|  | WLAN | 802.11b: -76 dBm@11 Mbit/s |
|  |  | 802.11g: -65 dBm@54 Mbit/s |
|  |  | 802.11n: -64 dBm@65 Mbit/s |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |
|  | 802.11g: Up to 54 Mbit/s |  |
|  | 802.11n | HT20: Support MCS0–MCS7; Up to 72.2 Mbit/s. Support MCS8–MCS15; Up to 144.4 Mbit/s. HT40: Support MCS0–MCS7; Up to 150 Mbit/s. Support MCS8–MCS15; Up to 300 Mbit/s. |
| Maximum power consumption | 4 W |  |
| Power supply | - AC: 100–240 V |  |
|  | - DC: 5 V, 2 A |  |
| Battery | - Type: Li (rechargeable and irremovable) |  |
|  | - Capacity: 3.7 V, 5200 mAh |  |
|  | - Maximum working time: 20 hours (depending on the network) |  |
|  | - Maximum standby time: 500 hours (depending on the network) |  |
| External interfaces | Micro USB interface |  |
|  | USB interface (supplying power to other devices) |  |
|  | Standard microSD card interface |  |
|  | Standard 6-pin SIM card interface |  |
|  | Ethernet port: RJ45 |  |
| Screen | OLED-LCD |  |
| key-press | Power switch, WPS switch, Reset switch |  |
| Antenna | - Built-in LTE/UMTS/GSM main antenna |  |
|  | - Built-in LTE/UMTS diversity antenna |  |
|  | - Built-in WLAN antenna |  |
| Dimensions (W × D × H) | 106.0 mm × 68.4 mm × 22.5 mm |  |
| Weight | about 200 g (including the battery) |  |
| Temperature | - Operating: 0°C to +35°C |  |
|  | - Storage: -20°C to +60°C |  |
| Humidity | 5% to 95% (non-condensing) |  |

# Firmware

## Version Description

| Firmware Version: | 21.200.15.00.00 |
| --- | --- |
| Baseline information | Balong V7R11 C30B200 |

## Firmware Specifications

| Firmware |  |  |
| --- | --- | --- |
| Item | Specifications |  |
| Version | 21.200.15.00.00 |  |
| Information | Platform | Balong V7R11 |
|  | Baseline | C30B200 |
|  | Feature | LTE Supported |
| Data service | GSM CS domain | - Uplink: 9.6/14.4 kbit/s - Downlink: 9.6/14.4 kbit/s |
|  | GPRS | - Uplink: 85.6 kbit/s - Downlink: 85.6 kbit/s |
|  | EDGE | - Uplink: 236.8 kbit/s - Downlink: 236.8 kbit/s |
|  | UMTS PS domain | - Uplink: 384 kbit/s - Downlink: 384 kbit/s |
|  | HSDPA | - 14.4Mbit/s |
|  | HSUPA | - 5.76 Mbit/s |
|  | HSPA+ | - 21.6Mbit/s |
|  | DC HSPA+ | - Uplink:5.76 Mbit/s - Downlink:43.2Mbit/s |
|  | LTE FDD | - Uplink: 50Mbit/s - Downlink:150Mbit/s |
| SMS | - SMS over SGs |  |
| Operating System | - Windows XP/Windows Vista/WIN 7/MAC/WIN8 |  |

## Improvement in the Previous Version

## Known Limitations and Issues