| Product name | Confidentiality level |
| --- | --- |
| E5770s | CONFIDENTIAL |
| Product version | Total 8 pages |
| V1.0 |  |

E5770s Firmware Release Notes

V1.0

| Prepared by | liuli | Date | 2014-4-7 |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |
| Approved by |  | Date |  |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/Stick Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2015-4-7 | 1.0 | FW 21.200.15.00.00 | First version | liuli |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Hardware Specifications	5

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	7

3.4	Known Limitations and Issues	12

E5770s Firmware Release Notes V1.0

# Main Features

- The E5770s supports the following standards:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s (Configuration 2)

- LTE TDD (UL) data service of up to 20 Mbit/s (Configuration 1)

- DC-HSPA+ (DL) data service of up to 43.2 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- GPRS data service of up to 85.6 kbit/s

- Ethernet data service of up to100 Mbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on UMTS and GSM

- Built-in LTE/UMTS/GSM and WLAN high gain antenna

- Micro Secure Digital Memory (microSD) Card

- SIM lock

- Wi-Fi and WPS

- LTE/3G/Wi-Fi auto offload

- Supply power to devices (working as a power bank)

- Support for HUAWEI Mobile WiFi App

- Press and Play

- IPv6/IPv4 dual stack (optional)

- Display current SSID and Wi-Fi key on the screen

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- Ethernet port

- PPPoE dial up

- Standard Micro USB interface

- USB interface

- OLED-LCD screen

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical standard | - WAN: LTE/DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |
|  | - WLAN: IEEE 802.11b/g/n |  |
| Operating frequency | LTE: - FDD B1(2100 MHz)/B3(1800 MHz)/B7(2600 MHz) - TDD B38(2600 MHz)/B40(2400 MHz)/B41(2600 MHz) |  |
|  | DC-HSPA+/HSPA+/HSPA/UMTS: B1(2100 MHz)/B8(900 MHz) |  |
|  | EDGE/GPRS/GSM: B2(1900MHz)/B3(1800MHz)/B8(900MHz)/B5(850MHz) |  |
|  | WLAN: 2.4 GHz |  |
| Internal memory | 128 MB NAND flash and 128 MB DDR SDRAM |  |
| Maximum transmitter power | LTE: Conform to Power Class 3 Definition |  |
|  | UMTS: Conform to Power Class 3 Definition |  |
|  | WLAN | 802.11b: 12 dBm |
|  |  | 802.11g: 10 dBm |
|  |  | 802.11n: 9 dBm |
| Receiver sensitivity | LTE: Confirm to 3GPP Requirements |  |
|  | UMTS: Confirm to 3GPP Requirements |  |
|  | WLAN | 802.11b: -76 dBm@11 Mbit/s |
|  |  | 802.11g: -65 dBm@54 Mbit/s |
|  |  | 802.11n: -64 dBm@65 Mbit/s |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |
|  | 802.11g: Up to 54 Mbit/s |  |
|  | 802.11n | HT20: Support MCS0–MCS7; Up to 72.2 Mbit/s. Support MCS8–MCS15; Up to 144.4 Mbit/s. HT40: Support MCS0–MCS7; Up to 150 Mbit/s. Support MCS8–MCS15; Up to 300 Mbit/s. |
| Maximum power consumption | 4 W |  |
| Power supply | - AC: 100–240 V |  |
|  | - DC: 5 V, 2 A |  |
| Battery | - Type: Li (rechargeable and irremovable) |  |
|  | - Capacity: 3.7 V, 5200 mAh |  |
|  | - Maximum working time: 20 hours (depending on the network) |  |
|  | - Maximum standby time: 500 hours (depending on the network) |  |
| External interfaces | Micro USB interface |  |
|  | USB interface (supplying power to other devices) |  |
|  | Standard microSD card interface |  |
|  | Standard 6-pin SIM card interface |  |
|  | Ethernet port: RJ45 |  |
| Screen | OLED-LCD |  |
| key-press | Power switch, WPS switch, Reset switch |  |
| Antenna | - Built-in LTE/UMTS/GSM main antenna |  |
|  | - Built-in LTE/UMTS diversity antenna |  |
|  | - Built-in WLAN antenna |  |
| Dimensions (W × D × H) | 106.0 mm × 68.4 mm × 22.5 mm |  |
| Weight | about 200 g (including the battery) |  |
| Temperature | - Operating: 0°C to +35°C |  |
|  | - Storage: -20°C to +60°C |  |
| Humidity | 5% to 95% (non-condensing) |  |

# Firmware

## Version Description

| Firmware Version: | 21.200.15.00.00 |
| --- | --- |
| Baseline information | Balong V7R11 C30B200 |

## Firmware Specifications

| Firmware |  |  |
| --- | --- | --- |
| Item | Specifications |  |
| Version | 21.200.15.00.00 |  |
| Information | Platform | Balong V7R11 |
|  | Baseline | C30B200 |
|  | Feature | LTE Supported |
| Data service | GSM CS domain | - Uplink: 9.6/14.4 kbit/s - Downlink: 9.6/14.4 kbit/s |
|  | GPRS | - Uplink: 85.6 kbit/s - Downlink: 85.6 kbit/s |
|  | EDGE | - Uplink: 236.8 kbit/s - Downlink: 236.8 kbit/s |
|  | UMTS PS domain | - Uplink: 384 kbit/s - Downlink: 384 kbit/s |
|  | HSDPA | - 14.4Mbit/s |
|  | HSUPA | - 5.76 Mbit/s |
|  | HSPA+ | - 21.6Mbit/s |
|  | DC HSPA+ | - Uplink:5.76 Mbit/s - Downlink:43.2Mbit/s |
|  | LTE FDD | - Uplink: 50Mbit/s - Downlink:150Mbit/s |
| SMS | - SMS over SGs |  |
| Operating System | - Windows XP/Windows Vista/WIN 7/MAC/WIN8 |  |

## Improvement in the Previous Version

| 问题单号 | 简要描述 |
| --- | --- |
| DTS2015041409374 | 【E5770s-923】【对外充电】电池关机对外充电至低电情况下显示low battery（反复提示）再接外充情况下，插入usb线对内充电，会引起高概率自动开机 |
| DTS2015041405718 | 【E5770s-923】【LCD】单板开机拨号成功后，LCD屏概率性出现连接时间显示。 |
| DTS2015041101170 | 和入42国语言相关提示语言资源 |
| DTS2015041008639 | 【E5770s-923】【WEBUI】升级产品WEBUI版本到主线最新版本WEBUI-V100R007B100D11SP00C03(17.100.11.00.03) ----- 当前E5770s-923的WEBUI版本为：WEBUI-V100R007B100D11SP00C03(17.100.09.01.00) |
| DTS2015040808780 | 【E5770s-923】【LCD】关机自闭环形成后开机，单板外屏不显示USB ERROR |
| DTS2015040808696 | 【E5770s-923】【LCD】开机单板形成自闭环后拔出USB线此时单板仍显示“USB ERROR” |
| DTS2015040808647 | 【E5770s-923】【USB HOST】反复上传文件后USB HOST功能异常，新建文件夹、删除操作均提示失败，继续上传文件时提示“存储设备不存在”，但此时可以正常下载U盘或移动硬盘内的文件 |
| DTS2015040307997 | 【E5770s-923】【webui】当流量达到限值时，webui上无提示用户流量超限断网的提示语。 |
| DTS2015040300972 | 【安全特性】【CTF产品】CTF场景下，当二层fdb表创建后，usb到wifi的ARP欺骗报文不经过协议栈，导致通过ARP欺骗嗅探可成功嗅探整个内网的密码 |
| DTS2015040200829 | 【E5770s-320】关机时同时内外充拔掉内充后外充仍限流1A没有恢复2A输出 |
| DTS2015033000908 | 【E5770s-923】【对外充电】单板低电关机对外充电，LCD屏幕上华为图标和低电提示切换闪烁。 |
| DTS2015032608232 | 【E5770s-923】【usb host】SD卡和硬盘（或U盘）均在位，移除USB线cradle接入进行多次文件上传，出现UI卡死，刷新后WEB UI打不开但可以ping通网关 |
| DTS2015032607057 | 【E5770s-923】【usb host】单板仅SD卡在位，多次拔插SD卡刷新页面查看，极低概率出现SD卡未加载上 |
| DTS2015022707080 | 【E5770s-923】【存储管理-help】最新方案10%为低点量，usb host和硬盘不能共享，但help文档和用户手册中还没有更新 |
| DTS2015012602965 | 【Hilink】使用Hilink APP配套喵王二代测试SD卡功能，在插入SD卡或硬盘的情况下，点击SD卡管理，需要等待几秒才能进入SD卡页面，并且经常提示获取数据失败 |
| DTS2015040801758 | 【E5770s-923】【USB HOST】开关机情况下插入U盘会持续耗电120mA，移动硬盘耗电290mA，最终会导致过放。 |
| DTS2015031807121 | 【E5770s-923】【wifi】三星D710手机背靠单板放置，手机上WIFI信号间歇性显示的RSSI值到-255。 |
| DTS2015031806580 | 【E5770s-923】【对外充电】单板开机状态下，给手机对外充电，概率性出现单板自动关机 |
| DTS2015031609056 | 【E5770s-923】【对外充电】E5电量为12%，开机状态，连接USB与低电关机的手机sc-cl00，手机会反复重启 |
| DTS2015030403540 | 【E5770s-923】【用户体验】：当单板关机电量≤10%可以对外充电，但开机后（电量≤10%）却无法对外充电，这会给用户带来困扰 |
| DTS2015012304694 | 【E5770s-923】【充电】使用充电器给单板充电同时连接充电外设进行同时对内对外充电，保持12小时左右后查看单板电量充满，但充电外设电量为65%左右，并且此时停止对外充电。拔插充电外设后可以恢复 |
| DTS2015021306010 | 【E5770s-923】【电源管理】HiLink上显示的喵王工作时间不准确，95%电量，Hilink电池续航时间为5小时16分，而实际单板宣称的工作时间为20小时，喵王已提供相关接口，待HiLink更新版本后进行验证 |
| DTS2015042400097 | 单板关机对外充电，电量为40%左右时无法开机 |

## Known Limitations and Issues

|  | 问题单号 | 简要描述 | 等级 |
| --- | --- | --- | --- |
| 1 | DTS2015030307895 | E5770s-923】【库仑计】：单板接入充电器电量充至100%时，重启开机电量会减少 | Suggestion |
| 2 | DTS2015022606987 | 【E5770s-923】【特性模块】流量显示体验性不好：当用户流量超了导致停机后就不能看流量，用户想看超多少都没办法，建议用户停机时“仅不显示上下箭下载符号，流量还是要显示的，另外其他人有的需要流量自动清0有的不需要，建议在流量前面加个日期区间或者在配置界面设置清0模式” | Suggestion |
| 3 | DTS2015021306010 | 【E5770s-923】【电源管理】HiLink上显示的喵王工作时间不准确，95%电量，Hilink电池续航时间为5小时16分，而实际单板宣称的工作时间为20小时，喵王已提供相关接口，待HiLink更新版本后进行验证 | Suggestion |
| 4 | DTS2015012304345 | 【E5770s-923】【对外充电】开机状态下对外充电，单板的LED屏幕上没有任何关于当前处于对外充电状态的提示图标。体验不好。 | Suggestion |
| 5 | DTS2015011502488 | 【E5770s-923】【LCD】单板静置放置保证单板已经休眠,点击power使LCD点亮,LCD先显示UINCOM--->正在推出休眠的提示语---》再出现拨号动画--在显示UINCOM | Suggestion |
| 6 | DTS2015010903297 | 【E5770S-923】【显示屏】当服务器上有新版本时，单板上提示更新的提示界面不够直白、美观。 | Suggestion |
| 7 | DTS2015010801720 | 【E5770s-923】【21.200.05.01.00】【WIFI】在屏蔽环境下，5770-923抗干扰能力差于5573-320,5770-923平均值为43.83Mbps，5573-320为71.48Mbps | Suggestion |
| 8 | DTS2014122304335 | 【E5770S-923】【21.200.01.00.00】【路由网关】在USB连接下，单板以太网拨号成功后，ping包大小为60000以上，无法ping通 | Suggestion |
| 9 | DTS2014121907658 | 【E5770s-923 -PC兼容性测试--F(03)】单板连接PC,安装后台后，USB口遍历过程中，webui页面自动弹出，页面内容为空，可以正常ping通网络，见截图 | Minor |
| 10 | DTS2014121608245 | 【E5770s-923】【LCD】单板长按(单板会关机)Power和短按(单板进入二级菜单)Power未作出区分，不管是长按还是短按外屏界面都会跳转到二级菜单。 | Minor |