# File System Check Utility for 07.36.xx Firmware Running on DS4000 and DS5000 Storage Systems

# Utility Overview

The File System Check Utility (i.e. DbFix) can be found on the Storage Manager’s Enterprise Management Window that runs as a result of starting this utility.

NOTE: 	You should run the utility from the same system you use for storage management but the utility can be run from a different system if your storage management isn’t on a Windows system.  In either case, the system must be on the same Ethernet network as the controllers because network communication between the utility and the controllers is essential.

Once run, the utility enters a discovery phase and determines which of the controllers found is running a supported firmware version. Only systems running firmware version 07.15.10.01, 07.36.xx.xx, 07.50.xx.xx or 07.60.xx.xx will be recognized. This tool is only required to be run against a subsystem with firmware 07.36.08.00 or 07.36.12.00 or a subsystem on a newer version which was upgraded from either 07.36.08.00 or 07.36.12.00.

For assistance with the results and next steps, contact IBM Support.

# Key Restrictions/Considerations

The following restrictions and considerations apply to the utility program. Please read this entire section before running the utility.

- The utility issues commands via an included FTP server.  If an FTP server is already running on the system used to run the utility, it must be stopped prior to running the file system check.  Use the “net stop service” Windows system command or use the Administrator >> Services Utility from the control panel to stop the other FTP server.

- The utility may be installed on the same system that the Storage Manager software is running on or it may be installed on an entirely different system.

- The system the utility is installed on must be running Windows.

- This tool is designed to run when low I/O is present from the application and hosts.  For very large systems (200+ drives), this tool can take approximately 20 min to run on a storage system when no I/O is present.  Therefore, please note that it might take some time to run the test and that by doing this test during lower I/O periods will improve response time and reduce any possible performance impacts in running the tool.

- As with other system tools, no configuration changes should be made to the system while running the tool to prevent inaccurate results.   If a change is made inadvertently or a component fails, the tool should be run again to validate outcomes after the system is once again stable and optimal.  Any results obtained during such an event should be ignored.

- The system that the utility is installed on and the controllers to be checked must be on the same network.  This utility uses network connections to communicate with the controllers.  If there is a firewall between the utility system and the controllers it must allow both telnet and FTP traffic to pass.  If this is not possible then run the utility on a computer inside the firewall.

- This utility requires network access to the controllers in order to load and run the file system check.  In order to check a system that is managed in-band, you will need to set up a local network connecting the controllers. Then install the utility on a separate computer, such as a laptop that is on the network, and run the standalone version of the utility.

- Telnet sessions are susceptible to interruption by circumstances such as a controller reboot.  If this happens, please allow several minutes for the operation of the file system check task on the controller to finish before attempting to run it again.  Only one file system check task be may run at a time.

NOTE: 	After using this tool, you may uninstall it by simply stopping the FTP service and deleting the folder into which you extracted the files.  Instructions on how to stop the FTP service are included under the Operation section, below.

# Installation

Install the File System Check Utility by following this procedure.

- Copy the zipped file called DbFix_v25.zip to a temporary directory on the system attached to the storage system.

- Right click the zipped file and choose Extract All to unzip the file.

- Extract all files to the temporary directory.
The extraction process creates the following file structure: C:\temp\dbfix.  In the folder dbfix you will also find two folders: ftpserver and jre.

# Operation

ATTENTION: 	Before beginning this procedure, please use the Storage Manager to save all support data for any affected storage systems.

Use these instructions to run the utility.

- From the dbfix folder run the file called startftpserver.bat.  A command line window will open and a text message in the window will state “FtpServer started”.  Leave this window open.  It may be minimized if you wish but must remain running.  When you are finished close this window to stop the ftp server.

- There are two batch files available to check the stable storage file system on each controller:

  - If you have installed this utility on the same system as the Storage Manager, run SMdbCk.bat.

  - If you are running the utility on a system without a Storage Manager, run SMdbCkStandalone.bat.

- The File System Check Utility interface presents a new menu item named System Check.  Select the menu and choose System Check from the menu items.
A separate dialog box will display all the systems that are running supported firmware.

NOTE:	There are two modes of operation.  You may check all controllers by using the Check All button or you may select individual controllers by selecting them and then clicking Check Selected.

- Click Check All or Check Selected, depending on which operation method you are using.
A working window is displayed while the file system checks are running.  Allow the operation to run to completion without interruption.  The time it takes to check each storage system is dependent on the number of drives in the system.  The more drives you have the longer it takes.  For example, a DS4800 storage system with the maximum number of drives installed could take several minutes to complete.

- Consult the Troubleshooting section (below) if any messages are displayed on the screen while the utility checks the system.

Contact your Customer and Technical Support Representative that provided this utility to you for assistance with the results.

NOTE:	If the utility indicates that there are no errors and either of your controllers reboot before you perform a controller firmware upgrade you must run this utility again.

Delete any of the following files from any Storage Manager systems so they are not inadvertently installed during future upgrades:

- FW_DS4200_07360800.dlp

- FW_DS4200_07361200.dlp

- FW_DS4700_07360800.dlp

- FW_DS4700_07361200.dlp

- FW_DS4800_07360800.dlp

- FW_DS4800_07361200.dlp

- FW_DS5100_07360800.dlp

- FW_DS5100_07361200.dlp

- FW_DS5300_07360800.dlp

- FW_DS5300_07361200.dlp

# Troubleshooting

The following messages might be displayed while the utility checks the storage system.  If any message is displayed on your system, consult the following table for an explanation of the message.

| Message | Explanation |
| --- | --- |
| Check Needed | This system matches the model and firmware version of a possibly affected system. The check has not been executed on this storage system yet. |
| Check Passed, clear configuration not required | The test passed and no failures were detected. |
| Check Failed, further action is required, check logs | A failure was detected by the utility. |
| Unable to establish telnet connection to controller | The GUI was unable to telnet to the controller. |
| Invalid telnet username/password | The known username and password combinations were tried without success. The shell login and password has been changed from the values set at the factory. Call IBM support for further assistance. |
| Telnet parsing timed out | The client waited for 30 seconds without getting an expected result. |
| Failed to connect to FTP server | The controller returned that it was unable to connect to the FTP server running on the host or user machine. |
| Files missing from FTP server | The storage system was able to connect to the FTP server but the expected files were not found. |
| Was attempting to send shell username | The telnet program was attempting to send the shell username but encountered an error. The connection probably failed or timed out. |
| Was setting up FTP share | This error is sent if any of a series of commands that set up the net host on the controller encounter an error. If this message is seen, the controller probably sent shell output out of order and confused the client, or the connection timed out. |
| Was attempting to connect to FTP server | An “ls” command was issued to the controller which caused the controller to log into the FTP server and issue an “ls” on the starting directory of the FTP. If this message is seen, the controller probably timed out while trying to connect to the FTP server. |

| Message | Explanation |
| --- | --- |
| Unable to load test code from the FTP server | An error occurred while issuing the “ld” command to load the controller firmware check tool. This could be a connection issue to the FTP server. |
| Unable to start check function | An error occurred while attempting to start a background check task on the controller. |
| Was checking if the module was already loaded | Some problem or unknown case occurred while checking if the fileSysCheck module was already loaded on the controller. |
| Another system check is currently in progress | The tool was attempting to run the fileSysCheck on an array that is showing a check is currently in process. You need to wait for the other check to complete before starting another. |
| Unable to get the status of system check | The client was in the process of repeatedly asking the controller the status of the system check but encountered an unexpected error. |
| Mismatch exists between drives in the database | Each drive in the system contains a copy of the file system. During one phase of the test a drive by drive comparison is made. If a mismatch is indicated please contact IBM support for further assistance. |
| Could not unload check module, it must be manually unloaded | The tool determined the fileSysCheck code is still present on the controller but was unable to unload it. Issue the following commands from the controller shell. moduleShow unld "filename.vxe" ;filename is gotten from the moduleShow output |
| Successfully disabled cache on controller A. | This will tend to only show as a STATUS string in the log file. It means that the SSTOR cache was successfully disabled. |
| Successfully disabled cache on controller B. | (same as above) |
| Cache is already disabled for this controller. | This will tend to only show in the STATUS string in the log file. The GUI telnetted into the same controller for a second time; one that already had its cache disabled. This happens if the EMW has multiple paths to the same controller. |
| Was attempting to determine controller letter. | The telnet code was trying to determine if it was logged into controller A or B, but was unable to determine which it was logged into. |
| Unable to determine the controller letter. | (same as above, different step in the process) |

| Message | Explanation |
| --- | --- |
| The disable cache command failed. | This means that either the firmware .vxe file did not load proper or the loaded .vxe did not have the cache disable command or we got an unexpected result back from the command. |
| Failed to disable cache on controller A. | This will mean that cache was disabled on Ctrl B but not A. Something went wrong while trying to disable the SSTOR cache on Ctrl A. Check the log for more details. |
| Failed to disable cache on controller B. | (same as above) |
| Failed to disable cache on both controllers. | Check the log for more details. This will mean that neither controller could have its cache disabled, and the reasoning for it was different between the 2 controllers (possibly). |
| Check status unknown | An unknown and unexpected error occurred. Please contact IBM support for further assistance. |

# Example Log Files

The following log files are provided for informational purposes only. They are included here only as examples and references.

- The following information is saved in a log file and is simply a summary of what the utility did.  This file contains a date as well as “check_status” in the name of the file.

2009-01-09_13-19-01-225 - IFW4-3994-65 (172.22.242.65/172.22.242.66) - Check Passed, clear configuration not required.

- The following information is saved in a second log file and contains the full log details if they are needed for support purposes.  The name of this file contains a date and machine name.

=====================================================

Attempting cache disable on: 172.22.242.45

=====================================================

VxWorks login: Password:

-> fcAll

fcAll (Tick 0000074952) ==> 04/03/09-21:59:25 (GMT)

7091-B          Our  Num  ::...Exchange Counts...:: Num  ..Link Up..

Chip  LinkStat  Port Port ::                     :: Link  Bad  Bad

ID  Logi ::Open    Total  Errors:: Down  Char Frame

0-Dst Up-Loop      2    2 ::  0       3405      0::    2     0     0

1-Dst Up-Loop      2   31 ::  3     575349      9::    2     0     0

2-Dst Up-Loop      2    2 ::  0       3338      0::    2     0     0

3-Dst Up-Loop      2    2 ::  0       3327      0::    3     0     0

4-Dst Up-Loop      2    2 ::  0       3326      0::    4     0     0

5-Dst Up-Loop      2    2 ::  0       3368      1::    4     0     0

6-Dst Up-Loop      2   31 ::  4     573603     10::    5     0     0

7-Dst Up-Loop      2    2 ::  0       3337      0::    4     0     0

8-Src Down---      0    0 ::  0          0      0::    1     0     0

9-Src Down---      0    0 ::  0          0      0::    1     0     0

10-Src Down---      0    0 ::  0          0      0::    1     0     0

11-Src Down---      0    0 ::  0          0      0::    1     0     0

12-Src Up-Fab   20200    2 ::  7     502936    466::    1     0     0

13-Src Down---      0    0 ::  0          0      0::    1     0     0

14-Src Down---      0    0 ::  0          0      0::    1     0     0

15-Src Up-Fab   20400    1 ::  0         14      0::    1     0     0

value = 2 = 0x2

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = -1 = 0xffffffff

-> unld "fileSysCheck_7091_12.vxe"

value = -1 = 0xffffffff

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> hostAdd("aaromnei","172.22.33.189")

value = 0 = 0x0

-> netDevCreate("aaromnei:","aaromnei",1)

value = 0 = 0x0

-> iam("arraycheck","arraycheck")

value = 0 = 0x0

-> cd("aaromnei:/")

value = 0 = 0x0

-> ls

fileSysCheck_399x_07_15_10_01.vxe

fileSysCheck_399x_07_36_07_00.vxe

fileSysCheck_399x_07_36_08_00.vxe

fileSysCheck_399x_07_36_12_00.vxe

fileSysCheck_399x_07_36_14_00.vxe

fileSysCheck_399x_07_36_14_01.vxe

fileSysCheck_6091_07_15_10_01.vxe

fileSysCheck_6091_07_36_07_00.vxe

fileSysCheck_6091_07_36_08_00.vxe

fileSysCheck_6091_07_36_12_00.vxe

fileSysCheck_6091_07_36_13_30.vxe

fileSysCheck_6091_07_36_14_00.vxe

fileSysCheck_6091_07_36_14_01.vxe

fileSysCheck_6091_07_36_14_30.vxe

fileSysCheck_7091_07_36_07_00.vxe

fileSysCheck_7091_07_36_08_00.vxe

fileSysCheck_7091_07_36_12_00.vxe

fileSysCheck_7091_07_36_13_30.vxe

fileSysCheck_7091_07_36_14_00.vxe

fileSysCheck_7091_07_36_14_01.vxe

fileSysCheck_7091_07_36_14_30.vxe

value = 0 = 0x0

-> ld<fileSysCheck_7091_07_36_12_00.vxe

value = 661689796 = 0x277095c4

-> dbmDisableDatabaseCache

04/03/09-21:59:29 (GMT) (tShellRem1): NOTE:  sstor::Cache - allocating zero sized cache

SSTOR cache was successfully resized to 0 blocks

value = 0 = 0x0

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = 0 = 0x0

->

=====================================================

Successfully disabled cache on controller B.

Status: Successfully disabled cache on controller B.

=====================================================

=====================================================

Attempting cache disable on: 172.22.242.44

=====================================================

VxWorks login: Password:

-> fcAll

fcAll (Tick 0000075834) ==> 04/03/09-21:59:37 (GMT)

7091-A          Our  Num  ::...Exchange Counts...:: Num  ..Link Up..

Chip  LinkStat  Port Port ::                     :: Link  Bad  Bad

ID  Logi ::Open    Total  Errors:: Down  Char Frame

0-Dst Up-Loop      1    2 ::  0       3415      0::    4     0     0

1-Dst Up-Loop      1   31 :: 32     434705     12::    5     0     0

2-Dst Up-Loop      1    2 ::  0       3349      0::    4     0     0

3-Dst Up-Loop      1    2 ::  0       3337      0::    5     0     0

4-Dst Up-Loop      1    2 ::  0       3336      0::    2     0     0

5-Dst Up-Loop      1    2 ::  0       3373      1::    2     0     0

6-Dst Up-Loop      1   31 :: 38     648604      8::    3     0     0

7-Dst Up-Loop      1    2 ::  0       3348      0::    2     0     0

8-Src Down---      0    0 ::  0          0      0::    1     0     0

9-Src Down---      0    0 ::  0          0      0::    1     0     0

10-Src Down---      0    0 ::  0          0      0::    1     0     0

11-Src Down---      0    0 ::  0          0      0::    1     0     0

12-Src Up-Fab   20300    2 ::  2     553689    448::    1     0     0

13-Src Down---      0    0 ::  0          0      0::    1     0     0

14-Src Down---      0    0 ::  0          0      0::    1     0     0

15-Src Up-Fab   20500    1 ::  0         14      0::    1     0     0

value = 2 = 0x2

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = -1 = 0xffffffff

-> unld "fileSysCheck_7091_12.vxe"

value = -1 = 0xffffffff

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> hostAdd("aammpbht","172.22.33.189")

value = 0 = 0x0

-> netDevCreate("aammpbht:","aammpbht",1)

value = 0 = 0x0

-> iam("arraycheck","arraycheck")

value = 0 = 0x0

-> cd("aammpbht:/")

value = 0 = 0x0

-> ls

fileSysCheck_399x_07_15_10_01.vxe

fileSysCheck_399x_07_36_07_00.vxe

fileSysCheck_399x_07_36_08_00.vxe

fileSysCheck_399x_07_36_12_00.vxe

fileSysCheck_399x_07_36_14_00.vxe

fileSysCheck_399x_07_36_14_01.vxe

fileSysCheck_6091_07_15_10_01.vxe

fileSysCheck_6091_07_36_07_00.vxe

fileSysCheck_6091_07_36_08_00.vxe

fileSysCheck_6091_07_36_12_00.vxe

fileSysCheck_6091_07_36_13_30.vxe

fileSysCheck_6091_07_36_14_00.vxe

fileSysCheck_6091_07_36_14_01.vxe

fileSysCheck_6091_07_36_14_30.vxe

fileSysCheck_7091_07_36_07_00.vxe

fileSysCheck_7091_07_36_08_00.vxe

fileSysCheck_7091_07_36_12_00.vxe

fileSysCheck_7091_07_36_13_30.vxe

fileSysCheck_7091_07_36_14_00.vxe

fileSysCheck_7091_07_36_14_01.vxe

fileSysCheck_7091_07_36_14_30.vxe

value = 0 = 0x0

-> ld<fileSysCheck_7091_07_36_12_00.vxe

value = 19614812 = 0x12b4c5c

-> dbmDisableDatabaseCache

SSTOR cache was successfully resized to 0 blocks

04/03/09-21:59:41 (GMT) (tShellRem1): NOTE:  sstor::Cache - allocating zero sized cache

value = 0 = 0x0

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = 0 = 0x0

->

=====================================================

Successfully disabled cache on controller A.

Status: Successfully disabled cache on controller A.

=====================================================

=====================================================

Attempting to run System Check.

=====================================================

VxWorks login: Password:

-> moduleShow

MODULE NAME     MODULE ID  GROUP #    TEXT START DATA START  BSS START

--------------- ---------- ---------- ---------- ---------- ----------

RAID   0x7e166c          1  0x4076ee0  0x570d200  0x5765ca0

value = 0 = 0x0

-> moduleList

==============================================

Title:     Disk Array Controller

Copyright 2007-2008 LSI Logic Corporation, All Rights Reserved.

Name:      RC

Version:   07.36.12.00

Date:      12/18/2008

Time:      13:39:50 CST

Models:    7091

Manager:   devmgr.v1036api01.Manager

==============================================

value = 0 = 0x0

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = -1 = 0xffffffff

-> unld "fileSysCheck_7091_12.vxe"

value = -1 = 0xffffffff

-> getFileSysCheckResult

C interp: unknown symbol name 'getFileSysCheckResult'.

-> hostAdd("aapwdumw","172.22.33.189")

value = 0 = 0x0

-> netDevCreate("aapwdumw:","aapwdumw",1)

value = 0 = 0x0

-> iam("arraycheck","arraycheck")

value = 0 = 0x0

-> cd("aapwdumw:/")

value = 0 = 0x0

-> ls

fileSysCheck_399x_07_15_10_01.vxe

fileSysCheck_399x_07_36_07_00.vxe

fileSysCheck_399x_07_36_08_00.vxe

fileSysCheck_399x_07_36_12_00.vxe

fileSysCheck_399x_07_36_14_00.vxe

fileSysCheck_399x_07_36_14_01.vxe

fileSysCheck_6091_07_15_10_01.vxe

fileSysCheck_6091_07_36_07_00.vxe

fileSysCheck_6091_07_36_08_00.vxe

fileSysCheck_6091_07_36_12_00.vxe

fileSysCheck_6091_07_36_13_30.vxe

fileSysCheck_6091_07_36_14_00.vxe

fileSysCheck_6091_07_36_14_01.vxe

fileSysCheck_6091_07_36_14_30.vxe

fileSysCheck_7091_07_36_07_00.vxe

fileSysCheck_7091_07_36_08_00.vxe

fileSysCheck_7091_07_36_12_00.vxe

fileSysCheck_7091_07_36_13_30.vxe

fileSysCheck_7091_07_36_14_00.vxe

fileSysCheck_7091_07_36_14_01.vxe

fileSysCheck_7091_07_36_14_30.vxe

value = 0 = 0x0

-> ld<fileSysCheck_7091_07_36_12_00.vxe

value = 233361100 = 0xde8cecc

-> isp fileSysCheck

value = 662014788 = 0x27758b44

-> fileSysCheck started in task 661963696

Starting Database File System Check...Version: 1.5

Filesystem Structure Test

Passed (1 seconds)

Meta Data Test

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->     Passed (36 seconds)

Read Test

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

-> getFileSysCheckResult

FSC_IN_PROGRESS

value = 2 = 0x2

->

->     Passed (135 seconds)

File System Check Done:    Passed (172 seconds)

getFileSysCheckResult

FSC_PASSED

value = 0 = 0x0

-> unld "fileSysCheck_7091_07_36_12_00.vxe"

value = 0 = 0x0

->

=====================================================

Log Date: 2009-04-03 17:02:48

Final Status: Check Passed, clear configuration not required.

Controller IP: 172.22.242.45