ARIASOHO V3.9

FEATURE DESCRIPTION

(Based on ipLDK V3.9Da)

2010-Oct-25

Revision History

| DATE | BY | BEFORE | AFTER |
| --- | --- | --- | --- |
| 25 October 2010 | LDK | Initial release | A.0 |

Contents

1. DND Operation of Pre-selected MSG Station	5

2. DID/DISA Call Routing of DND Station	6

3. Enhanced CCR Operation	7

3.1 CCR Announcement Replay	7

3.2 CCR Reroute Destination	7

3.3 CCR Destination – Each station voice mail box.	7

4. Ring Mode Choice – Lunch mode	9

5. Group Name Service	11

6. Secretary controls Exec’s DND	12

7. Hunt Group Mail Box	13

8. Hunt Group Queued Tone	16

9. Flashing DSS of Call Forward	17

10. DND for Ringing Call	18

11. Enhanced Operations of Idle Line Selection Feature in Hot/Warm Line	19

12. Restriction ‘*’ and ‘#’ codes for outgoing call	21

13. Conference Member VIEW&DELETE	22

14.  No touch Service with Forced Hands Free mode	25

15.  CO Base Preset Forward	26

16.  DISA Call VMIB Busy Routing	27

17.  Toll Fraud	28

18. Authorization Code	29

19. Door Open with DISA service	31

20. Missed Call Log with Ring Assigned Call.	32

23. Co Base Call Time Restriction.	33

24. Conference Group	34

24. Enhanced Wake-Up	36

# 1. DND Operation of Pre-selected MSG Station

- Description

A station which is activated a pre-selected message can select an option by [P-MSG DND] Admin - ringing or activating as DND.

If [P-MSG DND] Admin is ON for a station, then CO and ICM ring is treated just like DND feature.

So, the pre-selected message will be shown to calling station with DND tone for ICM call. And for DISA CO, external user will hear “Do not disturb” Voice Message and can retry again.

- Operation

[P-MSG DND] admin is off

- 1. Call to Station XXX and Station XXX was set Pre-Selected MSG feature.

- 2. Station XXX can get the ring.

- 3. Caller hears ring-back tone.

[P-MSG DND] admin is on

- 1. Call to Station XXX and Station XXX was set Pre-Selected MSG feature.

- 2. Station XXX doesn’t receive the ring.

- 3. Caller hears DND tone.

Programming

PGM 113 – F16

# 2. DID/DISA Call Routing of DND Station

- Description

- DID/DISA call is routed to specific destination when calling station is DND or Pre-selected message.

There are three destinations (Tone/Attendant/Hunt Group). These flows is same at DID/DISA the others Destination (PGM 167). DND/P-MSG rerouted flow is same with busy reroute flow.

- Operation

Rerouting destination is Tone

- 1. Call to Station XXX and Station XXX is DND or Pre-Selected MSG SET state.

- 2. Caller hears busy tone.

Rerouting destination is Attendant

- 1. Call to Station XXX and Station XXX is DND or Pre-Selected MSG SET state.

2. This call will be routed at Attendant.

Rerouting destination is Hunt Group

- 1. Call to Station XXX and Station XXX is DND or Pre-Selected MSG SET state.

2. This call will be routed to assigned hunt group.

Conditions

- If [P-MSG DND] Admin is off, route operation doesn’t work even tough station is Pre-Selected MSG state.

- This Admin is added at PGM167-F4, so previous admin (VMIB PROMPT USAGE admin and others Rerouted Destination admin) is move to next flex-button.

- Programming

PGM 167 – F4

# 3. Enhanced CCR Operation

- Description

CCR is the incoming CO Call type of DID/DISA, the user can route the destination by pressing only one digit. If user presses a certain digit, the corresponding VMIB announcement is played. When the user presses the desired digit again, call routing is established. A user also may access the desired destination directly by dialing the Station or Hunt Group number, or VMIB announcement.

CCR operations are somewhat changed from 3.8 version.

## 3.1 CCR Announcement Replay

- Description

Until 3.7 version, when user heard VMIB announcement, if user pressed pre-assigned digit (when this

Destination was another CCR announcement), then user heard another second CCR announcement.

And then, user did not press any digit or destination is busy or error cases, according as DISA retry count, user heard first VMIB announcement again, not current depth CCR announcement.

From 3.8 version, current CCR announcement (current depth) will be provided again until DISA retry count is over, when user does not press any  digit or destination is busy or error cases.

CCR announcement retry count is calculated by each CCR announcement depth.

## 3.2 CCR Reroute Destination

Description

When user hear VMIB announcement,

In each CCR table(01~70), user can choose destination by pressing flexible button 01~10.

Flexible button 11, 12, 13 are newly working.

The 11’th flexible button is Busy destination.

The 12’th flexible button is Time Out/Error destination.

The 13’th flexible button is No Answer destination.

That is, Each CCR table(Each CCR announcement) have each CCR reroute destination by exception cases.

Each reroute destination have three destination same as PGM 167 DID/DISA destination. Three destination are Tone, ATD(Ring assigned destination) and Hunt Group.

## 3.3 CCR Destination – Each station voice mail box.

Description

Each CCR table(01~70) had 10 destinations. 11’th Destination is added. 11’th destination is Each Station’s

Voice Mail Box.

- Operation

Incoming call to ipLDK, and CCR feature is activated.

- User hear first DISA announcement.

- User press pre-assigned CCR digit, then go to CCR destination.

- If CCR destination is another CCR VMIB announcement, user hear this another CCR announcement.

- If user does not choice any destination or destination is busy or error, user hear this CCR announcement again(up to DISA retry count).

4.1) If user choose a destination, and routed the destination, but that destination is busy, call is rerouted to BUSY destination(11’th flex button).

4.2) If user does not choose any destination until DISA retry count is over or the destination is error, call is reroute to TIME OUT /ERROR destination(12’th flex button)

4.3) If user choose a destination, and routed the destination, but that destination is no answer, call is rerouted to NO ANSWER destination(13’th flex button).

Conditions

- CCR Reroute destination is only working in CCR feature. – That is, in order to be followed by CCR reroute destination, each PGM 228 - CCR table (each VMIB announcement) must have any value. If PGM 228 – CCR table 01(system announcement 01) don’t assigned any value, call will be routed to DID/DISA destination (PGM 167).

- VMIB prompt usage option is followed by PGM 167 – Flex 5.

- Retry count followed by each CCR announcement. That is, when outside user hear DISA announcement, press a digit and hear another CCR announcement, first DISA announcement and second CCR announcement retry count is individual thing.

- If destination is No answer, after DID/DISA no answer timer expiring, call is routed to No answer destination directly, not follow retry count.

Programming

PGM 228 : Customer Call Routing.

PGM 167 : DID/DISA Destination.

PGM 160 – F4 : DISA retry count.

# 4. Ring Mode Choice – Lunch mode

- Description

When a CO call comes into the system, the destination of the CO call can be changed according to the time.

There are 5 ring modes – Day mode/Night mode/Weekend mode/On-demand mode/Automatic Ring mode. The destination of a CO call can be set differently for each ring mode, while a User sets the destination of CO call with ADMIN program. At Day mode/Night mode/Weekend mode, the User can set the appropriate destination of CO call according to the situation (day or night or weekend).

On-demand mode among the ring mode is used to supply a different destination of CO call except Day mode/Night mode/Weekend mode.

The ring mode in Automatic Ring mode is classified as Day mode or Night mode or Weekend mode according to Weekly Time Table (ADMIN program 233).

Only attendant can change the ring mode. If a user presses the [DND/FOR] button at the attendant station.

Lunch mode will be added at 3.8 version.

Lunch mode can be used among Day mode.

According as ATD ring mode setting, Lunch Mode can be applied in DISA service(PGM 140), Ring Assigned incoming call(PGM 144, PGM 145).

- Operation

- At ATD station, to change Day /  Night / Weekend / On-Demand / Automatic Ring mode/Lunch.

1.1) Press the [DND/FOR] button at the Attendant Station, choose the ring mode(1~6).

  - Press the [TRANS/PGM] button. And Dial 0 7 4.

- In Weekly Time Table, time can be set automatically.

Conditions

- The default value of Weekly Time Table is as follows (entry number: 00 ). The first table entry (00) is for main attendants and others (01-15) are for intercom tenancy group attendants.

- In PGM 233 – Weekly time table, Lunch Start time and Lunch End time are added as below.

| Date | Day Start Time | Lunch Start Time | Lunch End Time | Night Start Time | Weekend Start Time |
| --- | --- | --- | --- | --- | --- |
| Mon | 09:00 | -- : -- | -- : -- | 18:00 | -- : -- |
| Tue | 09:00 | -- : -- | -- : -- | 18:00 | -- : -- |
| Wed | 09:00 | -- : -- | -- : -- | 18:00 | -- : -- |
| Thu | 09:00 | -- : -- | -- : -- | 18:00 | -- : -- |
| Fri | 09:00 | -- : -- | -- : -- | -- : -- | 18:00 |
| Sat | -- : -- | -- : -- | -- : -- | -- : -- | 00:00 |
| Sun | -- : -- | -- : -- | -- : -- | -- : -- | 00:00 |

- On-demand mode is not available in the Automatic Ring mode.

- The attendants of intercom tenancy group can change ring mode as well as main attendants. If an attendant of intercom tenancy group changes ring mode, only the ring mode of intercom tenancy group which the attendant belongs to is changed. But if a main attendant changes the ring mode, ring mode of the system will be changed.

- When the ring mode is set to Automatic Ring mode by main attendant, the ring mode of the system will follow the first table entry (entry number: 00) of Weekly Time Table.

- If the system ring mode is changed from the lunch/night/weekend/on-demand/auto ring mode to day mode, the ring mode of all Intercom Tenancy Group will change to the previous ring mode.

- Lunch Mode default  Value is not assigned.

- In PGM 144, Weekend mode, Lunch mode, or On-Demand mode, if Ring assigned destination is Station, Ring Delay Count is enable only 0.

- In weekly time table, if input time is same(for example, Day Start Time and Lunch Start Time are same), time table can not working correctly.

- Programming

PGM 140 : DISA Attribute.

PGM 144, 145 : CO Line Ring Assignment.

PGM 233 : Weekly Time Table.

# 5. Group Name Service

- Description

Each Hunt Group can have own group name which can be set in hunt group attribute.

- Operation

Incoming CO Call

- Incoming Hunt group call is established.

- If group name exist, Hunt name will be displayed on 1st LCD field.

ICM Call

- ICM  Hunt group call is established.

- If group name exist, Hunt name will be displayed on 3rd LCD field.

Conditions

- In case of  CO incoming Hunt Call, Group Name will be displayed instead of [LINE RINGING] in LCD field 1.

- In case of ICM Hunt Call, Group Name will be displayed instead of [HUNT CALL] in LCD field 3(time displayed field).

- In case of ICM Hunt Call, Group Name can be cut off, because of LCD field length.

- If Hunt group name is not assigned, then group number will be displayed.

- The length of Hunt Group Name is maximum 12 characters.

- Programming

PGM 191 : Hunt attribute.

# 6. Secretary controls Exec’s DND

- Description

Secretary can activate and deactivate the EXEC/SEC feature from her extension.

- Operation

- From secretary’s extension, make EXEC DND flexible button by pressing

[PGM] + flex. button + [PGM] + 55 + executive’s extension number.

- Secretary starts to monitor executive’s DND state and change the state by pressing flex. button when needed.

Conditions

- Executive / Secretary Pair should be assigned in advance.

- To this feature enable, secretary station must be DKT.

- This feature only enable secretary station is idle state.

- Programming

PGM 229 Executive Secretary Table

Flex 1 : Exec / Sec pair

- Check List

- When secretary station makes Exec DND flex. button, the following station number should be correct executives extension number.

# 7. Hunt Group Mail Box

Description

Each hunt group can have own voice mail box.

User can leave a message, and each hunt group member can retrieve the message.

This Hunt group mail box is useful for Hunt Call re-routing.

There are 4 overflow destination types – Station, Hunt Group, VMIB system announcement, SPEED.

This Hunt group mail box will be added at overflow destination.

Operation

- Leaving a message.

- A call is routed to Hunt group.

- The call is not answered, and after overflow timer expired, call is routed to overflow destination – group voice mail box.

- A user hears the hunt group greeting and can leave a message.

- In case of DKT member, recording message finish, hunt group mail box button is flashing.

- In case of SLT member, recording message finish, 2’nd dial tone is served.

- Retrieving a message.

- In case of DKT, Each hunt group member can have a button for own group mail box.

- Hunt group mail box button is [PGM] + DSS + [PGM] + 88 + group number.

- Hunt member press the flashing DSS for group mail box.

- In case of SLT, WKT, Each hunt group member can have a numbering plan code for accessing to group mail box.

- Access code for hunt group mail box is [561].

- Hunt member press the access code [561] + Group number.

- In case of DKT, for repeating current message, Press [CALL BACK] button or [3soft – REPEAT] button.

- In case of SLT, for repeating a current message, Press [#3].

- In case of DKT, for retrieving a next message, Press [HOLD] button of [3soft – NEXT] button.

- In case of SLT, for retrieving a next message, Press [#2].

- For only first entering to hunt group mail box, mail box button or numbering plan code is used.

- Deleting a message.

- Deleting process is same as each station message deleting.

- In case of DKT, on retrieving a message, user can delete by using [CONF] button or [3soft - DELETE] button.

- In case of SLT, on retrieving a message, user can delete by using deleting code [#1].

- Deleting process is over, next message is played.

- Leaving a Hunt Group User Greeting.

- Press PGM  + 68. It is same as SLT.

- Enter Hunt Group Number.

- User can record hunt group greeting similar way as PGM 61.

- Deleting a Hunt Group User Greeting.

- Press PGM + 69. It is same as SLT.

- Enter Hunt Group Number.

- Hunt group greeting can be deleted similar way as PGM 66.

Conditions

- Remote VMIB feature is not possible for this hunt group mail box.

- Only one member can access hunt group mail box. That means two members can not access to group mail box, at the same time.

- Only one member can access PGM 68, or PGM 69 at the same time.

- All Hunt group member can record a hunt group greeting.

- All Hunt group member can delete a hunt group greeting.

- All Hunt group member can retrieve and delete hunt group voice mail.

- If a station is not a Hunt group member, user can not record hunt group greeting.

- If a station is not a Hunt group member, user can not delete hunt group greeting.

- Only In Idle state, Group Voice Mail button or numbering code is allowed.

- If hunt group member have no right to use VMIB (PGM 113 - F2), member can not retrieve a message.

- Adding a message to current played message is possible.

- Transfer a message to other station or other hunt group is possible.

- When user leave a message, previous used timer is same used.(PGM 181 – F3, PGM 181 – F4)

- If VMIB channel is all busy state, timer (fixed timer of 5 sec) is re-assigned repeatedly until getting available VMIB channel.

- If VMIB capacity is full, this function is not working.

- To enter the mail box, there is no need to enter the password.

- CIR/TERM group, UCD group, Ring group is allowed to this function.

- ICM call and CO call is enable to leave a message.

- When message is left, LCD display is not allowed.

- If a hunt group is deleted, then all left messages are deleted.

- If a hunt group type is changed, then all left messages are deleted.

- If a member is deleted from a hunt group, mail box button is OFF.

- If no message, hunt group mail box button is OFF.

- If no message and user press hunt group mail box button or dial hunt group mail box numbering plan code, then this is error case.

- If Hunt group greeting is not recorded, but Default user greeting is recorded, then default user greeting is played instead of hunt greeting.

Programming

PGM 190 – Hunt group.

PGM 191 – Hunt group Attribute.

# 8. Hunt Group Queued Tone

Description

When all Hunt group members are busy, if new call is routed to the hunt group, this call is queued.

At present, this queued call information is displayed at DKT LCD.

In additional, queued tone will be provided to first hunt group member.

There is option about providing queued tone or not, at each hunt group attribute.

Operation

- Incoming Hunt group call or ICM Hunt group call is established.

- If a call is queued, queued tone is provided to first Hunt group member.

Conditions

- This service is provided to Circular group, Terminal group, UCD group, and Ring group.

- This feature is possible only when first member is DKT station.

- If first member is ringing state, this feature does not working.

- This feature is possible for ICM hunt call as well as CO hunt call.

Programming

PGM 191 : Hunt attributes.

# 9. Flashing DSS of Call Forward

Description

DSS button program of forward feature is possible.

This is PGM + Flex button + Forward Code (PGM 106 – F11) + Forward type + Destination.

This programmed Flex button can work as toggle with light ON and OFF.

Operation

- If a user presses this programmed button, forward feature is enabling with light ON.

- If a user presses this programmed button again, forward feature is disabling with light OFF.

Conditions

- If user set forward feature manually and this program is matched with button forward program, button must be enable and light ON.

- If user re-set forward feature manually, this forward button must be disable and light OFF.

Programming

PGM 106 – Numbering Plan.

# 10. DND for Ringing Call

Description

When ATD call to another station, or ATD try to transfer a call to another station, the another station don’t want to get the call. In this case, that another station press DND button, DND feature does not working, because, this is ATD call.

DND feature is change to possible with system option although calling station is ATD.

Operation

Conditions

Programming

PGM 160 – F23 (DND enable for ATD call).

# 11. Enhanced Operations of Idle Line Selection Feature in Hot/Warm Line

Description

Administrator can assign additional destinations at Idle Line Selection option for Hot/Warm Line feature in PGM122. These appended destinations are Hunt-Group and Station Speed Number, System Speed Number.

If some user is assigned to additional destinations, when users lift handset or after warm-line timer is expired, users can directly be connected to assigned destinations.

[Destinations of Line Selection]

| DGT | ITEM | Operation | Remark |
| --- | --- | --- | --- |
| 1 | FLEX | To activate a feature on a flexible button as if pressed. |  |
| 2 | CO Line | To secure a CO Line |  |
| 3 | CO Line Group | To secure a CO Line Group |  |
| 4 | Station | To call an another station |  |
| 5 | Hunt Group | To call Hunt Group | New option |
| 6 | Station Speed Dial | To dial number from marked Station Speed Bin | New option |
| 7 | System Speed Dial | To dial number from marked System Speed Bin | New option |

And also the limitation is absolved when Idle Line Selection is assigned to Flex Button.

Previously the Hot/Warm line feature does not work if it is assigned to a virtual Flex Button. If Hot/Warm line feature is assigned virtual Flex Button, always Speed feature will be operated with index of virtual Flex Button instead of Flex Button’s program. But in this new version (V3.9) although virtual Flex Button is assigned for Hot/Warm line feature, preprogrammed operation in this virtual Flex Button can be executed. So, users can assign preprogrammed operation with virtual Flex Button.

Operation and Flow

To set the destination at Hunt-Group

1. PGM122 and choose Station range

2. Select Hunt-Group, Hunt Group number is 5

3. Assign specific Hunt Group number

Operation of Hunt-Group Destination

1. Users lift handset or warm line timer expired

2. System read the assigned hunt group number

3. System makes hunt-call with this group number

To set the destination at Station or System Speed Number

1. PGM122 and choose Station range

2. Select Station or System Speed, Station Speed is 6 and System Speed is 7

3. Assign specific System Speed number or their Station Speed number

Operation of Station or System Speed Number

1. Users lift handset or warm line timer expired

2. System read the assigned Speed number

3. Speed feature routine will be invoked with this Speed number

To set the destination at real or virtual Flex Button

1. PGM122 and choose Station range

2. Choose Flex Button and assign Flex-Number

4. If there are rational destination (not Empty), system invoke adjusted feature with information in Flex Button

Operation of real or virtual Flex Button on Station

1. Users lift handset or warm line timer expired

2. System read the assigned Flex number and checks the preprogrammed operation

3. If there are rational destination (not Empty), system invoke adjusted feature with information in Flex Button

Conditions

- All of users with using all kind of terminal can assign virtual Flex Button for Idle Line selection

- In case of Virtual Flex Button, previous flow is changed.

- If hunt or speed is assigned for Line selection and there are no real destinations of Hunt-Group or Speed number, user can use the call as normal operation call.

Programming

PGM 122

# 12. Restriction ‘*’ and ‘#’ codes for outgoing call

Description

When users seize the CO line and dial some digit for making outgoing call, as first digit ‘*’ and ‘#’ codes can be restricted. In some country, digit ‘*’ and ‘#’ has to be limited for security, so administrator can set ON at [Restriction * and #] admin option. This additional option is defined at PGM161-Flex24.

Operation

To set the [Restriction * and #] admin option

1. PGM161 and select F24

2. Set ON or OFF

Operation of outgoing Call

1. Users seize the CO line

2. If user press ‘* or ‘#’ for the first digit, system will check [Restriction * and #] admin option.

3. If [Restriction * and #] admin option is ON, this call will be disconnected

4. If [Restriction * and #] admin option is OFF, user can dial desired number and make outgoing call

Conditions

- [Restriction * and #] admin will be applied by base on system option.

- Default value of this new Admin is OFF, except of nation UK (Only UK default value is ON)

Programming

PGM 161 – F24

# 13. Conference Member VIEW&DELETE

Description

Conference initiator(for Add On) or creator(for CONF ROOM) can view conference members and delete him during conference.

Operation

1. Conference initiator presses the TRANS/PGM Button or VIEW button.

2. Member is displayed on LCD ( 2 Line)

3. Conference initiator finds the member to delete by pressing Vol Up/Down key (or Navi Up/Dow Key)

4. Conference initiator presses HOLD button to delete the member

5. Conference initiator presses TRANS/PGM button to return Conference LCD

### Operation from the 3 Line DKTU (24 and 32 Character LCD)

During conference

CONFERENCE

CONF VIEW MUTE

- Press TRANS/PGM button or VIEW(3soft) button, the following will be displayed on the LCD: -

1. STA 1000 (Name)

2. 0191234567

BACK DELETE

To select the Member, press {UP} or {DOWN}of navigation key(or Volume Key)

1. STA 1000 (Name)

2. 0191234567

BACK DELETE

- Press HOLD button or DELETE(3soft) button, Selected member is deleted.

1. STA 1000 (Name)

2. STA 1050 (Name)

BACK DELETE

- Press TRANS/PGM button or BACK(3soft) button, return to conference state..

CONFERENCE

CONF VIEW MUTE

### Operation from the normal 2 line DKTU

During conference

CONFERENCE

- Press TRANS/PGM button, the following will be displayed on the LCD: -

1. STA 1000 (Name)

2. 0191234567

To select the Member, press {Up} or {Down} of Volume Key

1. STA 1000 (Name)

2. 0191234567

- Press HOLD button, Selected member is deleted.

1. STA 1000 (Name)

2. STA 1050 (Name

- Press TRANS/PGM button, return to conference state..

CONFERENCE

Conditions

- During VIEW menu, All feature is operated to the same as CONFERENCE.

- In case member is CO line, dialed no or caller id should be displayed.

- This feature is only allowed to Conference initiator(for Add on) and Creator (for Conference Room)

- This feature is supported to DKTU with 2 line or 3 line LCD.

.

Programming

# 14.  No touch Service with Forced Hands Free mode

Description

A user can transfer the call to other station with Forced Hands Free feature.

When No touch Service is set to OFF, a station have to receive the ring instead of answering.

Operation

1. During conversation with CO line, User A call to User B to transfer the call.

2. User A presses Forced Hands Free Code (#) and User A can talk with User B

3. User A goes to on hook to transfer the CO call

4. In case NO TOUCH SVC is OFF, User B can receive the CO ring.

5. In case NO TOUCH SVC is ON, User B connected to CO. (Screened Transfer)

Conditions

Programming

PGM 111 –Flex7 : No Touch Answer

PGM 111 –Flex19 : Forced Hands Free

REF.

This feature is operated with Hand Free Mode.

# 15.  CO Base Preset Forward

Description

The Preset Call Forward Destination feature enables a CO line to initially ring at multiple stations and forward to a pre-determined destination.

The destination can be a station (EKT-SLT), Voice Mailbox, VMIB prompt, UCD group, or Hunt group. Each CO line has a Preset Forward Timer. Each CO line also has a VMID field to allow sending of specific VM digits when a CO line forwards to a VM group.

Operation

1. Normal Incoming Call (Ring Assigned to STN) is routed to destinations.

2. This call is not answered.

3. After CO Base Preset Forward Timer, Call is routed to CO Base Preset Forward Destinations.

Conditions

1.    Line Preset Call Forwarding will not occur if initial ringing destination is set to a group. e.g., UCD, external VM group, or hunt group.

2.    CO Line Preset Call Forwarding feature is only applied to Normal Ring Assigned to Station.

 Not allowed to transfer call, Not allowed to ring assigned to Hunt group, Not allowed to VMIB.

- CO Line Preset Call Forwarding feature overrides all call forwarding feature.

- CO Line Preset Call Forwarding overrides individual station forwarding.

- CO Line Preset Call Forwarding overrides Station Preset Call Forwarding.

- If the preset forward timer is 0, Preset Call forwarding is not available.

Programming

PGM 141 – F13, F14, F15

# 16.  DISA Call VMIB Busy Routing

Description

When VMIB channels are all busy, if DISA incoming call is routed to ipLDK, there are three way re-routing service. It can be chosen to PGM 142 – F20.

Operation

- Ring Back Tone : The incoming call goes to waiting state and hear ring back tone, until get a available VMIB channel.

- Busy Tone : The incoming call is disconnected.

- Go Error Dest : The incoming call is re-routed to Error Destination. (Go to PGM 167 Error Destination, or Go to PGM 228 Error Destination)

Conditions

1. If DISA Announcement have program in PGM 228, that means it has a valid CCR destination, then in that case, call is routed to CCR error destination.

Programming

PGM 142 – F20

# 17.  Toll Fraud

Description

Generally, ipLDK system already has ways of restriction when CO to CO (DISA incoming and CO outgoing) call – CO to CO COS and DISA Account Code (PGM 141 – F3) option with Authorization Code.

In PGM 160 – F16 : DISA OUTGOING (ENALBE/DISABLE) is newly added.

This option is another toll fraud option for DISA outgoing.

This option is only related when incoming call is routed to each station’s voice mail box.

Operation

- Incoming Call is routed to ipLDK system.

- This incoming call final destination is each station’s voice mail box.

- When outside user hears each station’s user greeting or default user greeting (system announcement 098), he tries to seize another CO for outgoing.

- In this case, if PGM 160 – F16 is set to ON, then normal DISA outgoing process can be started.

- If PGM 160 – F16 is set to OFF, then normal DISA outgoing process can not be allowed.

Conditions

- If this option is set to ON, this option have the first privilege than other option.

Programming

PGM 160 – F16 : DISA outgoing.

PGM 166 : CO to CO COS.

PGM 227 : Authorization Code.

# 18. Authorization Code

- Description

New Type authorization code is applied to ipLDK system. There were two type authorization codes in ipLDK.

One is five digit fixed authorization code, and the other is flexible digit authorization code.

Newly added type is Station number with flexible authorization code or Auth code bin number with flexible authorization code

PGM 161 – F21 is changed as followings.

User can choose one menu of 3 kinds of menus.

- 5 digit authorization code enable. – Name is 5 DIGIT AUTH CODE.

- Flexible digit authorization code enable.- Name is FLEXIBLE AUTH CODE.

- Newly added authorization code enable – Name is EXTEND AUTH CODE.

In case of 5 DIGIT AUTH CODE or FLEXIBLE AUTH CODE, it is not allowed to duplicate each auth code.

But, in case of EXTEND AUTH CODE, it is allowed to duplicate each auth code.

That means, station 100 auth code can be set to 1234567.

And station 110 auth code can be set to 1234567, and Bin 789 auth code can be set to 1234567, ALSO.

In case of ipLDK300 system, there are 1000 bin auth tables

In case of Bin 001 to 300, Station number with flexible authorization code is used.

In case of Bin 301 to 1000, Auth code bin number with flexible authorization code.is used.

Same way is applied to another system.

|  | With Station number | With auth code bin number |
| --- | --- | --- |
| ipLDK-20 | Bin 001 to 028 | Bin 029 to 200 |
| ipLDK-100 | Bin 001 to 128 | Bin 129 to 500 |
| ipLDK-300 | Bin 001 to 300 | Bin 0301 to 1000 |
| ipLDK-300E | Bin 001 to 600 | Bin 0601 to 2000 |

- Operation

- To Register and To Change Authorization code

  - This is same to current flexible authorization code.

- To Enter the Station number with Authorization code.

  - Dial the station number for the station authorization code.

  - Dial the corresponding authorization code.

  - If station number and authorization code is matched with table, user can continue to next process.

- To Enter the Auth code bin  number with Authorization code.

  - Dial ‘*’.

  - Dial Authorization table index. (ipLDK-20: 001 ~ 200, ipLDK-100: 001~500 , ipLDK-300/300E: 0001 ~ 1000/2000)

  - If  bin number and authorization code is matched with table, user can continue to next process.

Conditions

- Extend Auth Code length can be set 3 – 11 digit same as FLEXIBLE AUTH CODE.

- If PGM 161 – F21 value is changed to another value, all authorization codes that is previously programmed are initialized.

- Extend Auth Code initial value is 000.

- In case of Flexible Auth Code, when input auth code, there is auth end code (#).  This is previously existed.

- In case of Extend Auth Code, when input auth code, there is no auth end code.

- The usage for Extend Auth Code is same as previously existed auth code. (Ex : Station Account Code, Co Group Account Code….)

Programming

PGM 161 – System attribute.

PGM 227 – Authorization code.

# 19. Door Open with DISA service

- Description

When Door Open is activated with External Control Contact, outside user from DISA line can control Door Open.

- Operation

- Incoming DISA Call is routed to ipLDK system.

- ipLDK answer with system announcement xxx or system dial tone.

- Outside user press Door Open code that is activated.

- If Door Open code is normal, confirm tone is served, and outside user can hear system announcement or system dial tone again.

- If Door Open code is not correct, the call is routed to DISA error case.

- Programming

PGM 160 Flex 4 : DISA Retry Counter

PGM 167 : DID/DISA Destination

PGM 168 : External Control Contact.

# 20. Missed Call Log with Ring Assigned Call.

- Description

If Ring Assigned call is routed to a station or stations, missed call is saved in call log at pre-assigned station. Missed call is saved at a station that is programmed in PGM 141 – F16.

- Operation

- Case 1 - Normal Incoming Ring is routed to Ring Assigned Stations.

- Case 2 – DISA call is routed to a destination, but it is not answer, busy or error, call is re-routed to ring assigned station or stations.

- If outside user disconnects first before ipLDK user answers, this LOST CALL LOG is saved at pre-programmed station.

Conditions

- Programming

PGM 141 – F16 : Missed Call Log Station.

PGM 114 – F4 : CLI MESSAGE WAIT.

# 23. Co Base Call Time Restriction.

- Description

With CO base PGM 141 – F17, Call time restriction is possible.

PGM 141 – F17 is CUT OFF time, and it’s range is 00 – 99 (minute).

If this timer is set to 00, then call cut off time is not applied.

Warning tone is provided 15 seconds before the call is disconnected.

- Operation

- Outgoing call and timer start to work.

- Warning tone is provided 15 seconds before the call is disconnected.

- If timer is expired, Co line disconnected forcedly.

Conditions

- It is applied to only outgoing call.

- If Call time restriction is enabled (non-zero), the timer is still applicable when a call is transferred to another Station.

- If Call time restriction is enabled (non-zero), the timer is applicable when the call is placed on hold.

- On the add-on conference, the Call time restriction is enabled (non-zero), the timer is still applicable.

- If Co base call time restriction and station base CO call time restriction is programmed (CO Call Time Restriction simultaneously, the timer which is shorter has priority.

- Programming

- PGM 141 – F17, Call time restriction

# 24. Conference Group

Description

A station can make a conference conversation with using [Numbering Plan Code + conference group number].

Conference Group member can be assigned at admin program.

The number of Conference Group is 7.

The number of Conference Group member is 10.

Only stations can be assigned as Conference Group member. Co-line cannot be assigned.

A station that makes a conference group call is conference initiator (Conference Supervisor).

When a conference initiator tries to make Conference Group call, if conference group member is idle state, ring is served to the station.  And No Answer timer is applied.

After No Answer Timer is expired, ring stops, and the station go to idle.  And, there is no retry to call this member.

If conference group member is talk state with anybody, [Camp On] feature is established to that station automatically.

When Conference Group member answers the ring, he enters into this Conference Group call.

Operation

In PGM 207, Conference Group 1’s member is set to 100, 101, 102 and 103.

- Make a conference group call

  - A station 100 press conference group code + conference group number 1, or press flexible button that is stored [conference group code + conference group number].

  - Station 100 enter in conference state (similar as Conference Room with one member).

  - In station 100, if LCD is available, LCD shows [CONFERENCE].

  - Station 101, 102, and 103 get an ICM ring.

  - In station 101, 102, and 103, if LCD is available, LCD shows [Conference from STN 100].

  - If station 101 answer first, then two-way conference is established between station 100 and station 101 with conference entering warning tone.

  - If station 102 answers second, then three-way conference call is established among 100, 101, and 102 with conference entering warning tone.

- Disconnect a conference group call

  - After establishing conference, all features are similar as Conference Room.

  - If station 101 goes out from conference, conference out tone is provided to all conference member.

- Conference Member View and Delete

  - At conference initiator station, conference member view and forced delete is possible.

Conditions

- A station can be a member of multiple conference groups.

- If a station is not the conference group member, a station can not make a conference group call.

- Conference warning tone is controlled by PGM 160 – F11.

- If busy station receives conference group call, then LCD shows [Conference from STN 100] with Camp On tone.

- When busy station receive conference group call with Camp On, if busy station answer the conference group call, then previous call is disconnected and only conference group call is activated.

- If a member is conference state, then Camp On is not allowed. And the station can not receive another conference group call.

- If ringing station receives conference group call, then Camp On is not allowed. And the station can not receive conference group call.

- When Conference Group call initiator goes on hook before all the members answers, this Conference Group Call is cancelled and the ring at the Conference Group members stops.

- When Conference Group call initiator goes on-hook on conferencing, already connected members will not be disconnected.

- If Conference Group No Answer Timer is set to 0, then conference group ring is served to each member permanently until Conference group initiator goes On-Hook.

- Station will not receive Conference Group call if has activated DND feature, Call forward feature, Pre-selected Message.

- Conference Group call cannot be picked up by another station.

- Mobile extension feature is not working in case Conference Group call.

- This feature is not applied if station is ICM Box, ISDN Phone, DSS Console.

Programming

PGM 109 : Numbering Plan for Conference Group feature(new). Code:  #3

PGM 207 : Conference Group member assign(new).

PGM 182 – F15 : Conference Group No answer timer(new).  Range: 00 – 60 sec, Default 30 sec, 00 means not apply no answer timer.

PGM 160 – F11 : Conference Warning Tone.

# 24. Enhanced Wake-Up

Description

If PGM163 – F5 : ON, then enhanced wake-up feature will be enabled.

Each station can have and set up to 5 wake-up times. Each wake-up time is identified using an id (1-5). Each wake-up time has a wake-up type:

| 1 | YY/MM/DD | Alarm will be activated only one time in day specified by YY/MM/DD. |
| --- | --- | --- |
| 2 | Mon - Fri | Alarm will be activated every day, from Monday to Friday. |
| 3 | Mon - Sat | Alarm will be activated every day, from Monday to Saturday. |
| 4 | Mon - Sun | Alarm will be activated every day. |

Operation

To register wake-up time alarm from a Station, perform the following steps:

- Press the [TRANS/PGM] button.

- Dial [4] [1].

- Enter wake-up time id: [1] … [5].

- Dial the desired 2-digit hour (24-hour mode), then 2-digit minute for alerting.

- Enter wake-up type: [1] … [4].

- If wake-up type is 1 (YY/MM/DD), enter wake-up date: 2 digits for year, 2 digits for month and 2 digits for day.

- Press the [HOLD/SAVE] button.

To erase a wake-up time from a Station, perform the following steps:

- Press the [TRANS/PGM] button.

- Dial [4] [2].

- Enter wake-up time id: [1] … [5].

- Press the [HOLD/SAVE] button.

To register a wake-up time alarm from the Attendant Station, perform the following:

- Press the [TRANS/PGM] button.

- Dial [0] [4] [2].

- Dial the station range to be alerted. If a single station is to receive the alarm, enter ＊ instead of a second station number.

- Dial the desired 2-digit hour (24-hour mode), then 2-digit minute for alerting.

- Enter wake-up type: [1] … [4].

- If wake-up type is 1 (YY/MM/DD), enter wake-up date: 2 digits for year, 2 digits for month and 2 digits for day.

- Press the [HOLD/SAVE] button.

To erase a wake-up alarm from the Attendant Station, perform the following Steps:

- Press the [TRANS/PGM] button.

- Dial [0] [4] [3].

- Dial the station range that was to be alerted. If a single station is to receive alarm, enter ＊ instead of a second station.

- Press the [HOLD/SAVE] button.

Conditions

- When register or erase wake-up time from a DKT station, it can use [VOL UP] and [VOL DOWN] buttons to scroll between wake-up times.

- When register wake-up time from a DKT station, it can use [#] button to scroll between wake-up types.

- When register wake-up time from Attendant Station, wake-up information will be saved in a free wake-up entry, for each station from station range. If one station has not any free wake-up entry, then the wake-up time from Attendant Station will not be saved..

- When erase wake-up time from Attendant Station, all wake-up information will be erased.

- When PGM163 - F5 : changed from OFF->ON or from ON->OFF ,  all wake-up times for all stations are erased and current wake-up times are canceled.

- When wake-up time is active in current day, a * (star) will signalize near clock displayed on DKT station LCD. If there are no wake-up time enabled in current day, then there is no signalization on DKT station LCD.

- After finalizing wake-up call, wake-up time will not be automatically erased. Including wake-up time with YY/MM/DD type.  A wake-up time can be erased only manually.

- After finalizing wake-up call (with answer or not answered), a new wake-up time will be enabled, if it exist for current day.

- Any changes related with wake-up alarm, from Attendant Station, during wake-up call, will be effective only after finalizing wake-up call.

- When registering or erasing wake-up alarm using LDP-7xxx station menu, step 3 of operation (Enter wake-up time id: [1] … [5]) disappears.

- Starting from LDP-7xxx station menu, it is possible to toggle between register and erase mode, using left and right navigation buttons.

- Wake-up call functionality is similar with old wake-up feature.

- Wake-up year can be from 2000 to 2031.

Programming

PGM163 – F5 : ENABLE ENHANCED WAKEUP (ON/OFF)

Related Features

2.4.27 Wakeup

Hardware

None