LG PBX System

Software Technical Information

STI-0039

April 26, 2001

| Title | #### Answering Machine Emulation |
| --- | --- |
| System | CS1000 |
| Abstract | Function needed for DKTU subscriber to listen to the process of a voice mail to himself through the speaker and then to pick up the mail |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 04. 16. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Answering Machine Emulation	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	3

Interactions	3

Hardware	3

Title: Answering Machine Emulation

Function

This function is needed for DKTU subscriber to listen to the process of a voice mail to himself through the speaker and then, if necessary, to pick up the mail. If it is not picked up for a set time, the service is automatically cancelled.

Operation Sequence

- [Mail Pick Up]

- When a subscriber B leaves a voice mail with Voice Mail ID of DKTU subscriber A, CFWD All to VMS, CFWD Busy to VMS, or CFWD No Answer to VMS mode will be activated.

- B will hear VMS announcement and the speaker of A side will be on with the service indication tone. Then each voice will be heard from B and VMS through the speaker.

- If A put the phone in the state of Off Hook, the VMS will be released. Then A and Be will get into a call conversation.

- [Service Cancellation by DKTU]

- When a subscriber B leaves a voice mail with Voice Mail ID of DKTU subscriber A, CFWD All to VMS, CFWD Busy to VMS, or CFWD No Answer to VMS mode will be activated.

- B will hear VMS announcement and the speaker of A side will be on with the service indication tone. Then each voice will be heard from B and VMS through the speaker

- A presses the DND button.

- The service will be cancelled but B may leave voice mail following the VMS announcement.

- After A leaves the voice mail and then release the service, the mail will be registered in A as Message Wait.

Service Conditions

- DKTU only may be provided with the service.

- To use the service, you must be registered as a DKTU subscriber.

- Built-in Voice Processing Module (VPM) and separate In-band DTMF VMS only may support the service.

- Mail Access representative No must be chosen.

- Subscriber B must have a Mail Box ID registered in the service.  (Separate In-band DTMF VMS is not available to share a Mail Box ID, with the result that the Mail Box needs to be created in a special type. Separate VMS also need a Mail Box ID.)

- Without a Mail Before Pick Up, Subscriber B may not the voice from Subscriber A.

Data Generation

- Choosing Answering Machine Emulation Service DKTU Subscriber

- [3.1 Extension Information => Answering Machine EM]

- Choosing Mail Access Representative No.

- [6.4 Calling Group => Service Type item by Mail Access]

- Registering Mail Box ID for Built-in VPM

- [15.4 Mail Box Management]

- Registering Mail Box ID for Separate In-band DTMF VMS

- [15.4 Mail Box Management => MB All Creation (DTMF-VMS)]

- 5.  Setting Answering Machine Emulation Service Auto-Cancellation Time

- [2.3 System Lock Parameter => Answering Machine EM Duration Time]

Interactions

N/A

Hardware

N/A