LG PBX System

Software Technical Information

STI-0040

FEB 12, 2001

| Title | #### Automatic Privacy |
| --- | --- |
| System | CS1000 |
| Abstract | Function needed for extension subscriber in conversation to be free from interruption caused by an incoming call (Override, Camp On, Call Waiting, Auto Hold, Voice Over) |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Automatic Privacy	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

	OMS Menu Description	2

	How to Use	3

Title: Automatic Privacy

Function

This function is needed for extension subscriber in conversation to be free from interruption caused by Override, Camp On, Call Waiting, Auto Hold and Voice Over.

Operation Sequence

Service Conditions

- This service is applicable to all calls including extension calls and trunk calls.

- Even an extension in conversation with a subscriber provided with Automatic Privacy service is free from Override, Camp On, Call Waiting, Auto Hold and Voice Over.

Data Generation

- Setting Automatic Privacy

- [3.1 Extension Information => 14. Auto Privacy]

Interactions

N/A

Hardware

N/A

	OMS Menu Description

This menu is to Check or Change Auto Privacy Characteristic

[Sample Screen Description]  Menu Number : 3.1 Extension Information Characteristic => 14.Auto.Privacy

	How to Use

1	Query

  - Click Query button.

2	Change

- Click an item(14. Auto Privacy) you want to change.

- Change the data

- Click Send button.