LG PBX System

Software Technical Information

STI-0041

April 26, 2001

| Title | #### Call Recording |
| --- | --- |
| System | CS1000 |
| Abstract | Function for a subscriber in conversation to make a request for recording the speech in a separate VMS |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 04.26.2001 | H.G Jung | First issue. |

Table of Contents

Title: Call Recording	2

Function	2

Operation Sequence	2

Service Conditions	3

Data Generation	3

Interactions	3

Hardware	3

Title: Call Recording

Function

This function is needed for a subscriber in conversation to make a request for recording the speech in a separate VMS and for the VMS to record the speech in the Mail Box ID of the requesting subscriber. Replaying the recorded speech is available by way of the VMS setting mode.

Operation Sequence

- [SLT]

- A is on the line with B.

- A follows the sequence of Hook Flash + Feature Code.

- System gives a ring to VMS.

- VMS answers the call.

- System sends DTMF “Call Recording Prefix + A extension number” assigned to VMS.

- VMS starts recording the speech without the help of voice announcement and A will be given Indication of recording start through Voice Path.

- A continues the conversation with B.

- The recording ends by A and B completing the call or A pressing Hook Flash + Feature Code.

- [DKTU Flexible Button]

- A is on the line with B.

- A presses the flexible button in which the function is registered.

- System gives a ring to VMS.

- VMS answers the call.

- System sends DTMF “Call Recording Prefix + A extension number” assigned to VMS

- VMS starts recording the speech without the help of voice announcement and A will be given Indication of recording start through Voice Path

- A continues the conversation with B.

- The recording ends by A and B completing the call or A pressing the flexible button in which the function is registered.

Service Conditions

- The service is available for only SLT or DKTU subscribers.

- In-band DTMF type of separate VMS is only available for the service.

- Replaying the recorded speech in the system is subject to the specifications of the separate equipment

Data Generation

- Assigning busy recording Feature Code

- [2.1 System Numbering Plan => Call Recording item in Feature Code Plan]

- 2.  Setting Recording Delay Time for DTMF Protocol

- [2.3 System Lock Parameter => VMS Call Recording Delay Time]

Interactions

N/A

Hardware

N/A