LG PBX System

Software Technical Information

STI-0042

FEB 12, 2001

| Title | #### Copy COS Information |
| --- | --- |
| System | CS1000 |
| Abstract | Function to copy Special COS information to other Class for easy repetitive generation of COS information |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Copy COS Information	2

Function	2

Operation sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

	OMS Menu Description	3

Title: Copy COS Information

Function

This function is to copy Special COS (Class of Service) Allow/Deny Digit information to other Class for easy repetitive generation of COS information

Operation sequence

- Execute OMS to access the system.

- Click the [3.3 Class Base OG Digit Allow/Deny] of Data Management

- Click the “Query” button in the Class Base OG Digit Allow/Deny menu of Extension Data to execute the menu.

- Generate special class of Allow/Deny Digit information.

- Click the “Class Copy Tool” in the right of the class you wan to copy.

- Enter Destination Start Class and Destination End Class where to make a copy.

- Click the OK Button.

- Click the “Query” button of Class Base OG Digit Allow/Deny menu and check the result.

Service Conditions

N/A

Data Generation

- N/A

Interactions

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to execute COS Copy function.

[Sample Screen Description]  Menu Number : 3.3]