LG PBX System

Software Technical Information

STI-0043

FEB 12, 2001

| Title | #### Copy DKTU Flexible Button Information |
| --- | --- |
| System | CS1000 |
| Abstract | Function to copy information on a certain DKTU registered in a flexible button to other DKTU. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Copy DKTU Flexible Button Information	2

Function	2

Operation Sequence	2

Service Conditions	3

Data Generation	3

Interactions	3

Hardware	3

	OMS Menu Description	4

Title: Copy DKTU Flexible Button Information

Function

Function to copy information on a certain DKTU registered in a flexible button to other DKTU for easy repetitive generation of DKTU information registered in flexible button

Operation Sequence

- [Individual Copy]

- Execute OMS to access the system.

- Click the [3.1 Extension Information ]of Data Management

- Query a certain DKTU number.

- Generate DKTU Flexible Button Information.

- Click DKTU Extension No. by right mouse key and execute DKTU Flexible Button Copy Tool menu.

- Enter a DKTU source extension number in DKTU Flexible Button Copy Tool => Copy Tool 1 menu of DKTU Data to copy the number to the Source Field.

- Enter a DKTU destination extension number in DKTU Flexible Button Copy Tool => Copy Tool 1 menu of DKTU Data to copy the number to the Destination Field.

- Choose whether to copy the connected DLS information registered in Flexible Button.

- Click the One Data Copy button to execute the Copy command to the system.

- Go on executing individual copy by using skip button and next flexible button information

- Check the result.

- [Multi Copy]

  - Execute OMS to access the system.

  - Click the [3.1 Extension Information ]of Data Management.

  - Query a certain DKTU number.

  - Generate DKTU Flexible Button Information.

  - Click DKTU Extension No. by right mouse key and execute DKTU Flexible Button Copy Tool menu.

  - Enter a DKTU source extension number in DKTU Flexible Button Copy Tool => Copy Tool 2 menu of DKTU Data to copy the number to the Source Field.

  - Enter a DKTU destination extension number in DKTU Flexible Button Copy Tool => Copy Tool 2 menu of DKTU Data to copy the number to the Destination Field.

  - Choose whether to copy the connected DLS information registered in Flexible Button.

  - If you click the make data button, the copied information is displayed to OMS right menu.

  - You can delete the data by del button.

  - Click the All Data Copy button to execute the Copy command to the system.

  - Check the result.

Service Conditions

N/A

Data Generation

N/A

Interactions

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to execute Flexible Button information Copy function.

[Sample Screen Description]	Menu Number : 3.1

[Sample Screen Description]  individual copy

[Sample Screen Description]  multi copy