LG PBX System

Software Technical Information

STI-0045

Feb 26, 2001

| Title | #### Distinct Ring Group |
| --- | --- |
| System | CS1000 |
| Abstract | Function to provide Ring to subscribers according to Ring Group. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 02. 26. 2001 | H.G Jung | First issue. |

Table of Contents

Title : Distinct Ring Group	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	3

	OMS Menu Description	3

	How to use	4

	Contents	5

Title: Distinct Ring Group

Function

This function is to provide incoming call Ring to subscribers according to Ring Group. Ring grouping parameters include Ring Lock Time, Ring On/Off cycle, Ring Source*1), etc.

Note*:  Ring Source item is applicable only to DKTU connected to DSLC-LKD Issue2 and includes two types of Ring Sources as below.

- System Ring: Ring On/Off cycle is available. It is a System Tone Type of Ring limited to three kinds of tones.

- Local Ring: Ring On/Off cycle is unavailable. It is a Ring set in LKD Issue 2 in Wave File form, including various tones of eight kinds max.

Operation Sequence

- [Choosing Ring Tone Type per DKTU subscriber in use of System Ring ]

- Turn into Off Hook state.

- Press the Ring Tone Selection Feature Code (Default #1).

- Hear a Ring Tone you have selected from Type (0 ~ 2).

- Turn back into On Hook.

Service Conditions

- One system is available to assign 10 Ring Groups max..

- Assign users per Ring Group. All subscribers are initialized to Group 1 by default.

- If a subscriber of LKD is not assigned Local Ring ID per Ring Type (CO Ring, EXT Ring. Transfer Ring…), System Ring service will be provided to the subscriber. If assigned, Local Ring service is provided.

- Each DKTU subscriber using System Ring is permitted to choose Ring Tone Type.

Data Generation

- Choosing Ring Parameter and Local Ring ID per Ring Group

- [2.5 System Ring Parameter]

- Choosing extension subscriber per Ring Group

- [6.10 Ring Group]

Interactions

- If a call is made from a trunk to the representative No of a calling group and Ring Type of the calling group is set to Multi, the Ring Type is used as Calling Group Multi Ring Item instead of Trunk Ring Item.

- If a call is made from a trunk to an in-conversation subscriber while the trunk is busy state and the incoming option is set to Auto Group Hunting, the Ring Type is used as Busy Hunting Ring Item instead of Trunk Ring Item.

- If a call made from a trunk to a certain subscriber while the subscriber is in No Answer state and the Trunk option for in No Answer mode is set to Auto Group Hunting, the Ring Type is used as No Answer Hunting Ring Item instead of Trunk Ring Item.

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check or change Ring Parameter information per Ring Group.

[Sample Screen Description]   Menu Number : 2.5 System Ring Parameter

Below is an example of menu to check or change User information per Ring Group

[Sample Screen Description]   Menu Number : 6.10 Ring Group

	How to use

[System Ring Parameter]

1	Query

  - Click Query button.

2	Change

- Click the check box of an item you want to change.

- Change the data.

- Click Change button.

[Ring Group]

1	Query

  - Select Ring Group Index.

  - Click Query button.

2	Add

- Select a Ring Group Number you want to add.

- Enter an extension number you want to add in Ring Group.

- Click Add button

- Click Send button.

3	delete

- Select an extension number you want to delete.

- Click delete button.

- Click Send button

	Contents

[Item Description for System Ring Parameter]

| Item | Description |
| --- | --- |
| Item | Ring Type |
| Lock | Ring Lock (Unit 1sec) |
| Nst On | Nth Ring On Time (Unit 100msec) in Ring cycle |
| Nst Off | Nth Ring Off Time (Unit 100msec) in Ring cycle |
| Local Ring ID | Local Ring ID (DKTU connected to LKD Issue 2 is only available) |