LG PBX System

Software Technical Information

STI-0046

FEB 12, 2001

| Title | #### DKTU Enbloc Mode |
| --- | --- |
| System | CS1000 |
| Abstract | Function to make a call from DKTU to Enbloc Mode |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03.08.2001 | H.G Jung | First issue. |

Table of Contents

Title: DKTU Enbloc Mode	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

Title: DKTU Enbloc Mode

Function

This function is to make a call from DKTU to Enbloc Mode.

Operation Sequence

- [Call to Enbloc Mode]

- Get into Off Hook state.

- Dial the digits.

- Press End Digit (“#”).

- [Dialed One Digit Clear]

- Enbloc Mode is being dialed.

- Press Volume Down Button.

- Dial the following digits.

- Press End Digit(“#”).

- [Dialed All Digit Clear]

- Enbloc Mode is being dialed.

- Press Volume Up Button.

- Dial the digits from the beginning.

- Press End Digit (“#”).

Service Conditions

- DKTU subscriber must be chosen to call Enbloc Mode.

- If it is an inter-extension call or a feature code is used, the end digit (#) does not to be suffixed.

Data Generation

- Choosing DKTU subscriber to call Enbloc Mode

- [3.1 Extension Information => 41. DTEL Enbloc OG item]

Interactions

N/A

Hardware

N/A