LG PBX System

Software Technical Information

STI-0047

FEB 12, 2001

| Title | #### Door Phone |
| --- | --- |
| System | CS1000 |
| Abstract | Function to control Door at a telephone |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Door Phone	2

Function	2

Operation Sequence	2

Service Conditions	3

Data Generation	4

Interactions	4

Hardware	4

	OMS Menu Description	5

	How to use	5

	Item Description	6

Title: Door Phone

Function

This function is to control Door at a telephone, supporting Doorbell and Door Opener together at a time

Operation Sequence

- [Door Open by an incoming call to Doorbell – In case of SLT or DKTU destination]

- A guest presses Doorbell.

- A ring is given to the destination and the guest gets default RBT.

- The receiver answers the call. (If the destination-setting mode is UNA, the answer is made through UNA Answer Code.)

- The receiver starts a conversation with the guest.

- The receiver makes a Hook Flash or press the transfer button.

- The receiver dials Door Open Feature Code (Default *58) for Door Open.

- A function performance completion tone is given.

- The phone gets into On Hook state.

- [Door Open by an incoming call to Doorbell – In case of the destination using DKTU Flex Button]

- A guest presses Doorbell.

- A ring is given to the destination and the guest gets default RBT.

- The receiver answers the call. (If the destination-setting mode is UNA, the answer is made through UNA Answer Code.)

- The receiver starts a conversation with the guest.

- The receiver presses the Flex Button, in which Door Open Feature Code (Default *58) is registered by way of Program + Flexible Button + 6 + Feature Code, for Door Open

- A function performance completion tone is given.

- The phone gets into On Hook state.

- [Door Open by an incoming call to Doorbell – In case of the destination of ATD]

- A guest presses Doorbell.

- A call to extension is made to ATD and the guest gets a default RBT.

- ATD answers the call.

- The receiver starts a conversation with the guest.

- The receiver presses CONNECT Button for Door Open.

- A function performance completion tone is given.

- The receiver gets into On Hook state.

- [Door Open direct from SLT and DKTU]

- Get the phone into Off Hook state

- Presses Door Open Feature Code (Default *58).

- Dials the Index Number (0 ~ 9) of Door to open.

- A function performance completion tone is given.

- Get the phone back into On Hook state.

- [Door Open direct from ATD]

- Get the phone into Off Hook state.

- Presses CONF button.

- Dials the Index Number (0 ~ 9) of the Door to open.

- A function performance completion tone is given.

- Gets the phone back into On Hook state.

- [Door Open direct from DKTU using Flex Button]

- DKTU gets into Off Hook state (can be skipped).

- Press the flexible button in which Door Open Feature Code (Default *58) or Door Open Feature Code + Door Index is registered.

- Dial the Index Number (0 ~ 9) of the Door to open (can be skipped, if the flexible button has the Door Index registered in it.)

- A function performance completion tone is given.

- Gets the phone back into On Hook state.

- [Door Open from SLT, DKTU in the state of conversation with a third party]

- Make a Hook Flash or press a telephone button.

- Press Door Open Feature Code (Default *58).

- Dial the Index Number (0 ~ 9) of the Door to Open.

- A function performance completion tone is given.

- Make a Hook Flash or press a telephone button.

- Hold the conversation.

- [Door Open directly from DKTU by using Flex Button]

- Press the Flex Button in which Door Open Feature Code (Default *58) or Door Open Feature Code + Door Index is registered.

- Dial the Index Number (0 ~ 9) of the Door to Open (can be skipped, if the Flex button even includes Door Index registration.).

- A function performance completion tone is given.

- Press a telephone button.

- Hold the conversation.

Service Conditions

- One system is available to assign 10 Door Phones max.

- Interface for Doorbell is available only over ASLC.

- Interface for Door Opener is available over APTC.

- Seizure of APTC Line for Door Opener gets automatically released following a set time.

- RBT for Doorbell may be chosen for use or not.

- If a receiver of Doorbell has registered Call-FWD, the Call-FWD is performed normal.

Data Generation

- Door Phone Setting

- [16.14 Door Phone]

- Setting of Time for Seizing APTC for Door Opener

- [4.7 Trunk Route Base Lock Parameter => Dry Contact Release Lock item]

- Choosing RBT for Doorbell

- [2.4 System Tone Parameter => Door Bell RBT item]

Interactions

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check or change Door Phone information.

[Sample Screen Description]  Menu Number : 16.14

	How to use

1	Query

  - Click Query button.

2	Change

- Click an item you want to change.

- Change the data.

- Click Send button.

	Item Description

[Item Description]

| Items | Contents |
| --- | --- |
| Index | Door Phone Index No |
| Door Bell Telno | Extension Number to be used for Doorbell |
| Day Destination Type | Type to be terminated to System Day mode when Doorbell rings (ATD, EXT, UNA, etc are available in choosing a type) |
| Day Destination | Extension No to be terminated to System Day mode when Doorbell rings (Destination Type must be EXT) |
| Night Destination Type | Type to be terminated to System Night mode when Doorbell rings (ATD, EXT, UNA, etc are available in choosing a type) |
| Night Destination | Extension No to be terminated to System Day mode when Doorbell rings (Destination Type must be EXT) |
| Door Port No | Port No of the line to be used for Door Opener |
| RBT Option | Whether to transmit RBT to Doorbell or not |
| Comment | Door location information (12digits max.) |