LG PBX System

Software Technical Information

STI-0048

FEB 12, 2001

| Title | #### ISD/DDD Call Prefix Digit |
| --- | --- |
| System | CS1000 |
| Abstract | Function to choose information on identification of ISD/DDD needed to operate system functions. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: ISD/DDD Call Prefix Digit	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interaction	2

Hardware	2

	OMS Menu Description	3

	How to Use	3

	Item Description	3

Title: ISD/DDD Call Prefix Digit

Function

ISD/DDD Call Prefix Digit is a function to analyze a trunk outgoing call to identify ISD, DDD, Local call and is used in the modes below.

- OG Restriction

- SMDR

- Calling Party Identification Service (ISDN CLIP, R2 ANI, ACO Caller ID)

Operation Sequence

- [ISD call identification criteria]

- In case the dialed digits from the beginning is identical even to one of the assigned ISD Prefixes.

- [DDD call identification criteria]

- In case the dialed digits from the beginning is identical even to one of the assigned DDD Prefixes and non-identical to the area code of the place where a system is installed.

Service Conditions

- ISD Prefix Digit is available for two digits max and for max. 4 in quantity.

- DDD Prefix Digit is available for two digits max and for max. 4 in quantity.

Data Generation

- ISD identification Prefix Digit assigning (4 max.)

- [1.1 System Setup Information => ISD Call Type Prefix 1st ~ ISD Call Type Prefix 4th]

- DDD identification Prefix Digit assigning (4 max.)

- [1.1 System Setup Information => DDD Call Type Prefix 1st ~ DDD Call Type Prefix 4th]

- Assigning area code of place in which a system is installed

- [1.1 System Setup Information => Area Code]

Interaction

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check and change Trunk OG Pause Digit-related information.

[Sample Screen Description]   Menu Number : 1.1

	How to Use

1	Query

  - Click Query button.

2	Change

- Click an item you want to change.

- Click the data

- Click Send button.

	Item Description

[Item Description]

| Items | Contents |
| --- | --- |
| Area Code | Area code of place in which a system is currently installed |
| Own PBX Code | System identification code in using tie line |
| Default ISD Vender Code | Currently unused |
| ISD Call Type Prefix | ISD call identification Digits |
| DDD Call Type Prefix | DDD call identification Digits |
| DDI Prefix for Rout # | Route-based DDI area code |