LG PBX System

Software Technical Information

STI-0049

FEB 12, 2001

| Title | #### Non Verified Account Code |
| --- | --- |
| System | CS1000 |
| Abstract | Function to use random billing statistic code |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Non Verified Account Code	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	3

Interactions	3

Hardware	3

Title: Non Verified Account Code

Function

This function is to use random account code for billing statistic at a trunk outgoing call. Account code entry is available just before or at the call start, included in SMDR.

Operation Sequence

- [Account Code entry before an outgoing call]

- Get into Off Hook state.

- Extension dial tone is given.

- Enter Non Verified Account Code Feature Code (Default #41).

- Dial a random Account Code. (12 Digits max.)

- Press End Digit (“#”). (can be skipped, if 12 digits of Account Code is inputted)

- Extension dial tone is given.

- Dial Trunk Access Code + Outside No.

- [Enter Account Code in conversation state]

- The extension is busy with trunk line.

- Make a Hook Flash or press transfer button.

- Extension dial tone is given.

- Enter Non Verified Account Code Feature Code in Conversation. (Default #43).

- Dial a random Account Code (max. 12 Digits)

- Press End Digit (“#”). (can be skipped, if 12-digits Account Code is entered)

- A function performance completion tone is given.

- Make a Hook Flash or press transfer button.

- Extension will be established conversation state again.

- [Account Code input before Remote DOD call]

- Get into Off Hook state.

- Extension dial tone is given.

- Dial Remote DOD Feature Code (Default *78).

- Dial an extension number to be billed.

- Enter 4-digit password of an extension number to be billed

- Enter Non Verified Account Code Feature Code (Default #41).

- Dial a random Account Code. (12 Digits max.)

- Press End Digit (“#”). (can be skipped, if 12-digit Account Code is inputted)

- Extension dial tone is given.

- Dial Trunk Access Code and Outside Number.

Service Conditions

- Non Verified Account Code is available up to 12 Digits.

- In using Remote DOD, the initial password of 0000 is restricted in the function use.

- SLT or DKTU subscribers only are permitted to use the function.

- Non Verified Account Code is available for use when stored in the types below.

  - Feature Code Only

  - Feature Code + Account Code in part

  - Feature Code + Account Code in all

  - Feature Code + Account Code in all + Trunk Access Code

  - Feature Code + Account Code in all + Trunk Access Code + Outside Phone Number in part

  - Feature Code + Account Code in all + Trunk Access Code + Outside Phone Number in all

- Redialing uses the same Account code as the previous one.

- Non Verified Account Code is usable with Verified Account Code at the same time.

Data Generation

- N/A

Interactions

N/A

Hardware

N/A