LG PBX System

Software Technical Information

STI-0050

FEB 12, 2001

| Title | #### OG Restriction |
| --- | --- |
| System | CS1000 |
| Abstract | This function is for an extension subscriber to temporarily restrict outgoing calls through its own phone in his absence so that others may not make an outgoing call. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03.08.2001 | H.G Jung | First issue. |

Table of Contents

Title: OG Restriction	2

Function	2

Operation Sequence	2

Service Conditions	2

1.	Outgoing call Restriction Range	2

2.	Assign DDD identification Prefix Digit.	2

3.	Assign ISD identification Prefix Digit.	2

4.	Assign the area code to a place where the system is installed.	2

Data Generation	2

Interactions	3

Hardware	3

	OMS Menu Description	3

	How to Use	3

	Item Description	4

Title: OG Restriction

Function

This function is for an extension subscriber to temporarily restrict outgoing calls through its own phone in his absence so that others may not make an outgoing call.

Operation Sequence

- [Registration]

  - Go into Off Hook state.

  - Enter Outgoing call restriction code (Default *45).

  - Enter the 4-digit password.

  - Choose outgoing call restriction range (1 ~ 3).

  - A function registration completion tone is given.

  - Get back into On Hook state.

- [Cancellation]

- Get into Off Hook state.

- Enter Outgoing call cancellation code (Default *44).

- Enter the 4-digit password.

- A function release completion tone is given.

- Get back into On Hook state.

Service Conditions

- Outgoing call Restriction Range

    - The whole OG restriction:  1

    - Restriction of DDD and ISD:  2

    - ISD Restriction:  3

- Assign DDD identification Prefix Digit.

- Assign ISD identification Prefix Digit.

- Assign the area code to a place where the system is installed.

Data Generation

- Assigning DDD identification Prefix Digit (max. 4)

- [1.1 System Setup Information => DDD Call Type Prefix 1st ~ DDD Call Type Prefix 4th items]

- Assigning ISD identification Prefix Digit (max. 4)

- [1.1 System Setup Information => ISD Call Type Prefix 1st ~ ISD Call Type Prefix 4th items]

- Assigning the area code to a place where the system is installed

- [1.1 System Setup Information => Area Code]

Interactions

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check or change OG Restriction-related information.

[Sample Screen Description]   Menu Number : 1.1

	How to Use

1	Query

  - Click Query button.

2	Change

- Click an item you want to change.

- Click the data.

- Click Change button.

	Item Description

[Item Description]

| Items | Description |
| --- | --- |
| Area Code | Area code of a place where a system is currently installed |
| Own PBX Code | Current system identification code in using tie line |
| Default ISD Vender Code | Currently unused. |
| ISD Call Type Prefix | ISD call identification Digits |
| DDD Call Type Prefix | DDD call identification Digits |
| DDI Prefix for Rout # | Route-based DDI area code |