LG PBX System

Software Technical Information

STI-0051

Feb 18, 2001

| Title | #### Trunk OG Pause Digit |
| --- | --- |
| System | CS1000 |
| Abstract | Function to transmit to DTMF the digits dialed after Pause Digit at a trunk outgoing call, regardless of the trunk address signal. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 02. 18. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Trunk OG Pause Digit	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

	OMS Menu Description	3

	How to use	4

	Item Description	4

Title: Trunk OG Pause Digit

Function

The function is to dial additional digits through DTMF for destination-supported particular service after the answer made to a trunk call. The function supports automatically the environment of the registration in System Speed Code, Individual Speed Code, LCR DMT, and Flexible Button, according to chosen information.

Digits dialed after Pause Digit assigned to a system is unconditionally transmitted to DTMF. DTMF transmission point is dependant on whether trunk has answer signal or not as below.

[In case of No Answer Signal]

Transmission of Digits following Pause Digit after Pause Digit Time set in the system following the last digit transmission

[In case of Answer Signal]

Transmission of Digits following Pause Digit after Default 1 sec, which is a time set in a system for a receiver side to prepare DTMF Receiver, of answer signal received by destination.

Operation Sequence

- N/A.

Service Conditions

- Time parameters to be used for each function are available per route separated.

- Pause Digit use is as below.

- System Speed Dial

- Individual Speed Dial

- Outside No registered in Flexible button

- LCR DMT

- Outside Call-forwarding registered

- AND Digit of Trunk Access Code

- ARS Digit of Trunk Access Code

- This service is indicated on DKTU LCD with Digits defined in the System. (Outside Call Forwarding in the idle state is indicated with Space.

Data Generation

- Pause Digit Assigning

- [2.6 System Feature Option => What Digit Assign Trunk Pause Entry item]

- Adjusting Delay Time at No Answer signal trunk from Pause Digit until DTMF Transmission

- [4.8 Trunk Route Base All Option => Pause Time Assign item]

- Adjusting Delay Time at Answer signal Trunk until DTMF Transmission after receiving answer signal

- [4.9 Trunk Route Lock Parameter => ISDN DTMF Sending Delay Time item]

Interactions

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check or change Trunk OG Pause Digit-related information.

[Sample Screen Description]   Menu Number : 2.6

[Sample Screen Description]   Menu Number : 4.8

	How to use

1	Query

- Click Query button.

2	Change

- Click the check box of an item you want to change.

- Change the data.

- Click Change button.

- Click Send button.

	Item Description

[Item Description]

| Items | Description |
| --- | --- |
| What Digit Assign Trunk Pause Entry | Digit Entry to be used for Pause Digit |
| Pause Time Assign | Adjustment of Delay Time at No Answer signal trunk from Pause Digit until DTMF Transmission |
| ISDN DTMF Sending Delay Time | Adjustment of Delay Time at Answer signal Trunk until DTMF Transmission after receiving answer signal |