LG PBX System

Software Technical Information

STI-0052

FEB 12, 2001

| Title | #### Pause Digit Entry Insert |
| --- | --- |
| System | CS1000 |
| Abstract | Function to add Pause Digit Entry for trunk outgoing call from DKTU |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Pause Digit Entry Insert	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

	OMS Menu Description	3

	How to Use	3

Title: Pause Digit Entry Insert

Function

This function is to add Entry used as trunk Pause Digit in Registering outside subscriber’s No., individual speed dial, and call forwarding outside in flexible button.

Operation Sequence

  - Outside subscriber’s No., individual speed dial, and call forwarding outside are being registered in flexible button.

  - The entry of FLASH button in each case brings Pause Digit Entry of an assigned trunk to the LCD.

  - Press Hold/Input button.

  - A function registration completion tone is given.

Service Conditions

- System must be assigned Entry to be used as trunk Pause Entry.

- This service is indicated on DKTU LCD with Digits defined in the System. (Outside Call Forwarding in the idle state is indicated with Space.

Data Generation

- Assigning Entry to be used as trunk Pause Digit

- [2.6 System Feature Option => What Digit Assign Trunk Pause Entry item]

Interactions

N/A

Hardware

N/A

	OMS Menu Description

This menu is to Assign Pause Digit Entry

[Sample Screen Description]   Menu Number : 2.6

	How to Use

1	Query

- Click Query button.

2	Change

- Click the check box of an item you want to change

- Change the data.

- Click Change button

- Click Send button.