LG PBX System

Software Technical Information

STI-0054

FEB 12, 2001

| Title | #### System Day/Night Mode Change |
| --- | --- |
| System | CS1000 |
| Abstract | Function for System Mode to automatically turn into auto or manual. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: System Day/Night Mode Change	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	3

Interactions	3

Hardware	3

	OMS Menu Description	4

	How to Use	4

	Item Description	5

Title: System Day/Night Mode Change

Function

This function is for System Mode to automatically turn into auto or manual according to option setting

- Auto Mode: 	By information on assigned System Day/Night/Semi-Night Bar Time

- Manual Mode:	By ATD Night Button or assigned feature code

Operation Sequence

- N/A

Service Conditions

[Auto Mode]

- Tenant-based separation is available.

- Auto mode Option must be set.

- Day/Night/Semi-Night Bar Time information must be set.

- Subscriber’s Day/Night Class changes the mode automatically.

- ATD in Day Mode is available for calling regardless of system mode.

- When a trunk is in Night Mode, calling individual number of ATD helps to find each PNA subscriber of the ATD and the system to provide the service.

- When a trunk is in Night Mode, calling ATD using ACD code helps to find each PNA subscriber of the ATD and the system to provide the service.

[Manual Mode]

- Tenant-based separation is available.

- Auto mode Option must not be set.

- In the manual mode by ATD, all of the linked ATD in Night Mode change the System into Night Mode.

- In the manual mode, the option of subscriber’s Day/Night class must be set for the auto-change.

- In the manual mode by the chosen feature code, a call to ATD in Day Mode is available regardless System Mode.

- In the manual mode by the chosen feature code, the option of all the tenants must be set for the simultaneous change.

- When a trunk is in Night Mode, calling individual number of ATD helps to find each PNA subscriber of the ATD and the system to provide the service

- When a trunk is in Night Mode, calling ATD using ACD code helps to find each PNA subscriber of the whole assigned ATD and the system to provide the service.

Data Generation

- System Mode Auto Option Setting

- [16.3 Night Bar => Use Night Bar item]

- System Day/Night/Semi-Night Bar Time Setting

- [16.3 Night Bar => D Time, N Time, Semi-N Time item]

- Option of automatically changing subscriber Day/Night Class when System Mode is changed into Manual

- [2.6 System Option => Auto System Class Change item]

- Option of simultaneously changing all the tenant modes when System Mode is changed into Manual

- [2.6 System Option => TNT Group Mode Change item]

Interactions

#### N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check or change System Night Bar time per Mode and incoming Mode.

[Sample Screen Description]   Menu Number : 16.3

	How to Use

1	Query t

  - Click Query button.

2	Change

- Click an item you want to change.

- Change the data.

- Click Change button.

- Click Send button.

	Item Description

[Item Description for Announcement Line]

| Items | Description |
| --- | --- |
| Tenant No | Tenant Index |
| Day Mode | Day Mode Option at trunk incoming call (when DIL is not assigned to a destination and DDI Divert Option is set) |
| Night Mode | Night Mode Option at trunk incoming call (when DIL destination is not assigned to a destination and DDI Divert Option is set) |
| Holiday Mode | Holiday Mode Option at trunk incoming call (when DIL destination is not assigned to a destination and DDI Divert Option is set) |
| D Time | Day Mode Start Time |
| N Time | Night Mode Start Time |
| Semi-N Time | Semi-Night Mode Start Time |
| Use Night Bar | Option of auto-turning System Mode |