LG PBX System

Software Technical Information

STI-0055

FEB 12, 2001

| Title | #### Temporary Privacy |
| --- | --- |
| System | CS1000 |
| Abstract | Function for only a call connected between an extension and VIP to be free from interruption caused by another incoming call by way of Override, Camp On, Call Waiting, Auto Hold, and Voice Over. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Temporary Privacy	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

Title: Temporary Privacy

Function

This function is for only a call connected between an extension and VIP to be free from interruption caused by another incoming call by way of Override, Camp On, Call Waiting, and Voice Over.

Operation Sequence

- [Ordinary Case]

- In one-on-one conversation.

- Make a Hoof Flash or enter DKTU transfer button. In this case, the counterpart gets Hold-on tone and the subscriber is given Dial Tone.

- Dial Temporary Privacy Feature Code (Default *461).

- The call turns automatically back into the conversation state.

- [In case of using DKTU Flexible Button]

  - In one-on-one conversation.

  - Press the Flexible Button in which the function is registered (Registration: PROGRAM+ Flexible Button +6+ Feature Code).

  - Hold the conversation.

Service Conditions

- This service is applicable to all calls including extension and trunk calls.

- Even an extension subscriber connected with a subscriber provided with Automatic Privacy service is also free from interruption like Override, Camp On, Call Waiting, and Voice Over.

- Releasing the call automatically helps to cancel the Privacy mode.

Data Generation

- N/A

Interactions

N/A

Hardware

N/A