LG PBX System

Software Technical Information

STI-0057

FEB 12, 2001

| Title | #### Verified Account Code |
| --- | --- |
| System | CS1000 |
| Abstract | Function to use a fixed length of billing statistic code. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03. 08. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Verified Account Code	2

Function	2

Operation Sequence	2

Service Conditions	3

Data Generation	3

Interaction	3

Hardware	3

	How to Use	4

	Item Description	4

Title: Verified Account Code

Function

This function is to use Account Code stored in a system for billing statistic at a trunk outgoing call. Account Code entry is possible just before or at the call start, included in SMDR

Operation Sequence

- [Enter Account Code before an outgoing call]

- Get into Off Hook state.

- Extension dial tone is given.

- Enter Trunk Access Code.

- Dial the assigned Account Code. (12 Digits max.)

- Trunk dial tone is given.

- Dial an outside subscriber’s No.

- [Enter Account Code in conversation state]

- The extension is busy with trunk line.

- The extension makes a Hook Flash or press transfer button.

- Extension dial tone is given.

- Enter the Feature Code of Verified Account Code in Conversation. (Default #44).

- Dial the assigned of Verified Account Code.

- Function performance completion tone is given.

- Make a Hook Flash or press transfer button.

- Extension will be established conversation state.

Service Conditions

- One system is available for max. 1000 Verified Account Codes.

- Verified Account Code is available up to 12 Digits.

- The digit number of the initial entry of Verified Account Code means that of the system Verified Account Code. From the second time, the fixed number of digits is permitted to add.

- SLT and DKTU subscribers only are permitted to use the function.

- The use of Verified Account Code followed by an outgoing call is subject to Trunk Access Code stored in a type used for Verified Account Code.

- Non-assigned Account Code restricts an outgoing call in the service of Verified Account Service.

- A possible destination may be chosen without using Verified Account Code before sending an outgoing call.

- Redialing uses the same Account code as the previous one.

- Verified Account Code is available for Trunk Transit.

- Verified Account Code is available for system speed dial.

- Verified Account Code is available for using individual speed dial.

- Verified Account Code is available for call forwarding outside.

- Redialing uses the same Account code as the previous one.

- Verified Account Code is available for use when stored in Flexible Button in the types below.

  - Trunk Access Code only

  - Trunk Access Code + Account Code in part

  - Trunk Access Code + Account Code in all

  - Trunk Access Code + Account Code in all

  - Trunk Access Code + Account Code in all + Outside Subscriber’s No in part

  - Trunk Access Code + Account Code in all + Outside Subscriber’s No in all

- Non Verified Account Code is usable with Verified Account Code at the same time.

Data Generation

- Choosing Verified Account Code

- [11.3 Verified Account Code]

- Choosing Subscribers except Verified Account Code User

- [3.1 Extension Information => No Verified ACNT item]

Interaction

N/A

Hardware

N/A

	OMS Menu Description

Below is an example of menu to check, add, or delete Verified Account Code information.

[Sample Screen Description]   Menu Number : 11.3

	How to Use

1	Query

  - Select a First Digit to query.

  - Click Query button.

2	Add

- Enter a Code to add.

- Click Add button.

- Click Send button.

3	Delete

- Click the check box of a code you want to delete.

- Click delete button.

- Click Send button.

	Item Description

[Item Description]

| Items | Contents |
| --- | --- |
| First Digit | First Digit of Verified Account Codes |
| Verified Account Code | Verified Account Code |
| Comment | Description on Account Code (10 digits max.) |