LG PBX System

Software Technical Information

STI-0058

April 26, 2001

| Title | #### Voice Over |
| --- | --- |
| System | CS1000 |
| Abstract | Function for a busy subscriber to transfer an emergent message behind the counterpart by way of one-way break-in. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 04. 26. 2001 | H.G Jung | First issue. |

Table of Contents

Title: Voice Over	2

Function	2

Operation Sequence	2

Service Conditions	2

Data Generation	2

Interactions	2

Hardware	2

Title: Voice Over

Function

This Function is for a busy subscriber to transfer an emergent message behind the counterpart by way of one-way break-in.

Operation Sequence

- A is on the phone with B.

- Extension subscriber C calls A.

- C comes to hear Busy Tone.

- C presses busy One Digit Service Feature Code (Default: 7).

- A comes to hear a short Voice Over Indication Tone and then gets into the state below.

- A may hear the voices from B and C at a time.

- B may hear only the voice from A.

- C may not hear the voice from either A or B.

- If A makes a Hook Flash (or presses the Transfer button), the state below arises.

- A may hear the voices from both B and C at a time.

- B may not hear the voice from either A or B.

- C may hear only the voice from A.

- A makes a Hook Flash (or presses the Transfer button), the above item 5 and 6 will be repeated.

- One of B and C releases the call.

- A continue be on the phone with the rest.

Service Conditions

- This service is requested available for a pre-chosen subscriber.

- Those qualified to request this service are limited to SLT or DKTU subscribers.

- Those requested by the service are limited to SLT or DKTU subscribers.

- Any entry by A is disregarded except On Hook and Hook Flash while in the Service.

- Any entry by B and C is disregarded except On Hook while in the Service.

- Busy One Digit Service may not provided at the same time with Step Call.

- The service is unavailable for subscribers assigned to Privacy service.

Data Generation

- Choosing Voice Over Service Request Subscriber

- [3.1 Extension Information => Voice Over Reg.]

- Choosing Busy One Digit Service

- [16.5 One Digit Busy Service => Voice Over item]

Interactions

N/A

Hardware

N/A