LG PBX System

Software Technical Information

STI-02-0001

Mar 07, 2002

| Title | #### ISDN Numbering Plan ID |
| --- | --- |
| System | CS1000 |
| Abstract | Select the ISDN Numbering Plan ID at SETUP message |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 03.07.2002 | H.G. Kang | First Issue |

Table of Contents

Title : ISDN Numbering Plan ID	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

	Example TRACE message	3

Title : ISDN Numbering Plan ID

Functional Description

User can select ISDN numbering Plan ID(called) at SETUP message. Usually, CS1000 send ID 0x80 at SETUP message. This meaning is “UNKNOWN numbering Plan”.

But somewhere, They want a special numbering plan code line 0x81, 0x89. To supply it, CS1000 have optional item in OMS.

Operation Procedure

- According to option value, Numbering plan ID in setup message is changeable

| - Option Value | - Numbering Plan ID | - meaning |
| --- | --- | --- |
| - 0001 | - 0x81 | - ISDN/Telephony numbering plan |
| - 0003 | - 0x83 | - Data numbering plan |
| - 0004 | - 0x84 | - Telex numbering plan |
| - 0008 | - 0x88 | - National Standard numbering plan |
| - 0009 | - 0x89 | - private numbering plan |
| - others | - 0x80 | - Unknown |

Conditions

Default Value is 0x80(Unknown)

Data Generation

- OMS [16.12 Temporary Option] – Option 14

Interaction

none

Hardware

none

	Example TRACE message

- Default Value

- |                                      |

- |      I_SETUP       (PT:00e2 S:00)|

- |------------------------------->| 00010708 02052605

- |  SAPI=00 TEI=01 CRL=02 CR=0526  | a1040380 90a31803

- |                                      | a183826c 06018132

- |                                      | 30363470 05803230

- |                                      | 3333

- After input value 0001 in Temporary Option

- |                                      |

- |      I_SETUP       (PT:00e1 S:00)|

- |------------------------------->| 00010708 02049305

- |  SAPI=00 TEI=01 CRL=02 CR=0493  | a1040380 90a31803

- |                                      | a183816c 06018132

- |                                      | 30363470 05813230

- |                                      | 3333

- |                                      |