LG PBX System

Software Technical Information

STI-02-0009

APRIL 03, 2002

| Title | #### Automatic RAS Access by ANI |
| --- | --- |
| System | CS1000 |
| Abstract | Predefined outside subscriber can access the RAS. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| Draft 1.0 | 04.03.2001 | H.G Jung | First issue. |

Table of Contents

Title : Multiple ACNR	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

Title : Automatic RAS Access by ANI

Functional Description

Automatic RAS Access by ANI Feature is used when the manager in office wants to use the OMS through RAS device. Generally the manager should know the single line telephone number connected with RAS device. But if the service is enabled and the ANI of the manager in office is registered in the system, the manager can reach the RAS device by any number of the system.

Operation Procedure

- N/A

Conditions

- Maximum ANI for automatic RAS access is up to 5.

Data Generation

- Assign the ANI for automatic RAS access.

- [16.15 RAS Access CLI(ANI)]

Interaction

N/A

Hardware

N/A

  OMS Menu Description

The menu is to verify and change the RAS access CLI(ANI) information.

[Sample Screen Description 1]  Menu Number: 16.15

  How to use

1	Query

1) Press Query.

2	Change

- Click the desired item for change.

- Change or delete the data.

- Press Change.

- Press Send.

	Contents

[Item Description]

| Items | Contents |
| --- | --- |
| RAS Telephone Number | The number connected with RAS device |
| 1ST Calling Party Number | One of the CLI(ANI). |
| 2ND Calling Party Number | One of the CLI(ANI). |
| 3RD Calling Party Number | One of the CLI(ANI). |

| 4TH Calling Party Number | One of the CLI(ANI). |
| --- | --- |
| 5TH Calling Party Number | One of the CLI(ANI). |