LG PBX System

### Software Technical Information

STI-02-0010

Apr 15, 2002

| Title | #### Internal Call Charge |
| --- | --- |
| System | CS1000 |
| Abstract | Charge of internal call |

Revision History

| ###### Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2002.04.15 | H.G.Kang | First issue |

Table of Contents

Software Technical Information	1

Title : Internal Call Charge	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu 11.1 Charge Option	3

	OMS Functional description	3

	How to use	3

Title : Internal Call Charge

Functional Description

To supply the internal call charge, Charge information with call type 1 is sent.

Operation Procedure

- Make the conversation between extensions

- Keep the conversation minimum 5 second .

- After finish the call, The charge data is transmitted.

Conditions

- It’s call type is 1(Internal call)

- Charge data is separated to each conversation.

- The transfer subscriber is recorded in “Transfer Station Number field”

- Don’t support the conference call charge.

Data Generation

- Select the internal call charge

- [11.1 Charge Option > Internal Call Charge]

- Select the output device for charge

- [11.1 Charge Option > Charge Result Output Device]

Interaction

- Charge program must be modified for internal call type(1)

Sample data(decimal value) : calling number : 2014, called number: 2012

09152030006FFFF012014FFFF200204141245050020020414124526002012FFFFFFFFFFFFFFFFFFF

F010200020201030201040201050201070207080201120200200800000000FFFF3D

Hardware

N/A

OMS Menu 11.1 Charge Option

	OMS Functional description

Select the internal charge

	How to use

1	Query

Select the Query button to confirm the current state

2	Change

- Select the charge result output device

- Change the internal call charge to YES.

- Select the Change and Send button to apply the new option value.