LG PBX System

### Software Technical Information

STI-02-0011

Apr 15, 2002

| Title | #### Auto Hold Ring at LKD Terminal |
| --- | --- |
| System | CS1000 |
| Abstract | It can indicate by ring the incoming call when communication with another subscriber instead of LED or Tone. |

Revision History

| ###### Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2002.04.15 | H.G.Kang | First Issue |

Table of Contents

Software Technical Information	1

Title : Auto Hold Ring at LKD Terminal	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu 2.6 System Feature Option	3

	OMS Functional description	3

	How to use	3

Title : Auto Hold Ring at LKD Terminal

Functional Description

It can indicate by ring the incoming call when communication with another subscriber instead of LED or Tone.

Operation Procedure

- Make the conversation with extension or trunk from LKD terminal set auto-hold feature.

- Call to the LKD terminal form another subscriber

- Indicate the another incoming call according to option

- Indication is disappeared when answer to the new call.

Conditions

- If select the LED method, then you can see the Ring LED blinking only( Ring LED exist depend on terminal models)

- If select the TONE method, then you can hear the periodic tone through the handset with Ring LED blinking

- If select the RING method, then you can hear the ring sound from terminal’s speaker with Ring LED blinking

Data Generation

- Assign the Auto Hold Feature

- [3.1 Externsion Information >32. Auto Hold]

- Select the indicate method when happened another incoming call in LKD terminal.

- [2.6 System Feature Option > LKD Auto Hold Ring Method]

Interaction

- It is available when use with version 2.30 of DSLC-LKD F/W.

Hardware

- This option(feature) can be applied only LKD terminals.

- LKD terminal’s ring volume can’t control from OMS. It’s value is fixed itself.

OMS Menu 2.6 System Feature Option

	OMS Functional description

Assing the method to indicate the another incoming call in LKD Auto Hold Ring Method

	How to use

1	Query

Select the Query button to confirm the current state.

2	Change and Send

- Select the method to indicate the another incoming call in LKD Auto Hold Ring Method

- Select the Change and Send button to apply the new option value.