LG PBX System

### Software Technical Information

STI-02-0012

Apr 15, 2002

| Title | #### SMDA proper format display |
| --- | --- |
| System | CS1000 |
| Abstract | Display on CRT the Charge information include each field’s description |

Revision History

| ###### Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2002.04.15 | H.G.Kang | First Issue |

Table of Contents

Software Technical Information	1

Title : SMDA proper format display	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu 16.12 Temporary Option	3

	OMS Functional description	3

	How to use	3

Title : SMDA proper format display

Functional Description

Without Charge program, We can confirm the charge information.

After finished the call, it’s value include their description are displayed on CRT

Operation Procedure

- Assign the value 0001 at option 16 item in menu 16.12 Temporary option

- Try the outgoing call

- After some times, Finish the outgoing call.

- Confirm the charge information on CRT

Conditions

- When use this feature, It don’t need the select to charge output device in 11.1 Charge Result Output Device

- It must happen the charged call (Local, DDD,ISD, Incoming, Internal)

- If a field doesn’t fill, then that isn’t displayed

Data Generation

- Select the charge information display on CRT

- [16.12 Temporary Option > Option 16 set by 0001 ]

- Select the charge call type

- [11.1 Charge Option]

Interaction

- Output data is like below.

| [Charging Result:Serial_NO(00000)]------------------------------------------ |  |  |
| --- | --- | --- |
| Charge Info. | : 01 (Outgoing Call) |  |
| Call Type | : 02 (Local Call) |  |
| Called Trk. NO | : 0002 |  |
| Calling Telno | : 2014 |  |
| Trunk Acc. Code | : 9 |  |
| Dialed Digit | : 4504875 |  |
| Start Time | : 20020414 141132 |  |
| End Time | : 20020414 141141 |  |
| Calling STA Category |  | : 1 (Station with priority) |
| Calling STA Group |  | : 1 |
| Called Line Category |  | : 7 (PX Trunk Line) |
| Called Trunk Group |  | : 1 |

Hardware

1. Prepare the 9 pin SIO cable and Monitor to display it

OMS Menu 16.12 Temporary Option

	OMS Functional description

This option is used when display on CRT the charge information include each field’s description.

	How to use

1	Query

Select the Query button to confirm the current value

2	Change

- Assign the value by 0001 or 0000 at Option 16 item.

- Select the Change and Send button to apply the new option value.