Software Technical Information

STI-02-0014

Apr 12, 2002

| Title | #### Malicious Call Trace |
| --- | --- |
| System | CS1000 |
| Abstract | Malicious call trace |

- Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2002.4.12 | J.H.YEO | First Issue |

- Table of Contents

Title : Malicious Call Trace	2

Functional Description	2

Operation Procedure	2

Conditions	3

Data Generation	4

Interaction	4

Hardware	4

- Title : Malicious Call Trace

- Functional Description

To record the called communication history, System supply the formatted information through SIO cable.

- Operation Procedure

The extension received the call, caller’s number, data and time information is sent through SIO cable.

- OUTPUT FORMAT (CRT and Printer)

Calling No. : 818-4863, Called No. : 2004

| 2 | 0 | 0 | 4 |  |  |  |  |  | 8 | 1 | 8 | 4 | 8 | 6 | 3 |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 4 | - | 0 | 1 | - | 0 | 2 |  | 1 | 2 | : | 3 | 0 | : | 1 | 0 |  | d |  | p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Called(8) |  |  |  |  |  |  |  |  | Calling(18) Include ISD and DDD (if exist in CLI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date(8) mm-dd-yy |  |  |  |  |  |  |  |  | Time(8) Hh:mm:ss |  |  |  |  |  |  |  |  | A |  | B |

A : d-Direct call / t-Transfer call

B : (Output device) p-Printer / c-CRT via SIO

- The Malicious call trace data when transfer call

Blind transfer

| Call Scenario | output |
| --- | --- |
| Communication between (A) and (B) |  |
| (C) is called from (B) |  |
| (C) receive the ring | Called:C, Calling:B, ddmmyy, hhmmss (d) |
| (B) is on-hook before the answer | Called:C, Calling:A, ddmmyy, hhmmss (t) |

- Screened transfer

| Call Scenario | Output |
| --- | --- |
| Communication between (A) and (B) |  |
| (C) is called from (B) |  |
| (C) receive the ring | Called:C, Calling:B, ddmmyy, hhmmss (d) |
| Communication between (A) and (C) |  |
| (B) is on-hook | Called:C, Calling:A, ddmmyy, hhmmss (t) |

- Consultation call

| Call Scenario | Output |
| --- | --- |
| Communication between (A) and (B) |  |
| (C) is called from (B) |  |
| (C) receive the ring | Called:C, Calling:B, ddmmyy, hhmmss (d) |
| Communication between (A) and (B) |  |
| (C) is on-hook or Communication again between (A) and (B) after (C) is hold by (B) | No output |

- Conditions

- Which is can make Malicious call trace

The kind of called extension;

Digital Telephone

DECT

Analog Telephone

Attendant

The kind of calling extension;

Digital Telephone

DECT

Analog Telephone

Attendant

The kind of trunk;

C/O with CID feature

DTC-R2MCF

ISDN

- Data Generation

- Select the print out option of the malicious call trace

OMS [DATA 2.6 System feature option]

ANI Info. Print out (CRT,Printer) ? : YES

- If use the R2 signaling, Request the calling number

OMS [DATA 4.8 Trunk Route Base All Option]

Request ANI Information : YES

- If use the C/O with CID feature

OMS [DATA 4.1 Trunk Information]

25. check the Caller ID

- Select the format

OMS [DATA 16.12 Temporary Option]

Option 2 : 0004

- Who use the this feature (for extension)

OMS [DATA 3.1 Extension Information] – Extension Characteristics

25. Malice Trace : 선택

(if extension is DECT subscriber):

OMS [DATA 9.3 DECT Information] – Characteristics

25. Malice Trace : 선택

- Interaction

To assign the special output device, must make the config.sys file. Their contents is like below

- To IOT

ANITX IOT2

END

- To CN4

ANITX CN4

END

- Hardware

To print the result, you must use the serial printer