LG PBX System

Software Technical Information

LG PBX System

Software Technical Information

##### STI-CS1000-02-0015

April 15, 2002

| Title | PCATD Interface Module Protocol |
| --- | --- |
| System | CS1000 |
| Abstract | This document describes ATD Message Definition between PCATD Interface Module and ATD |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 04.15.2002 | Inhwa Cho | First issue. |

Table of Contents

TITLE: ATD MESSAGE DESCRIPTION FOR EXTERNAL DEVICE	4

Conception	4

Message Structure	4

IMPLEMENTATION CONSIDERATION	6

Implementation Consideration of SIO(Serial Input Output)	6

ATD INITIAL PROCESS	7

ATD Initial Message Flow	7

ATD Initial Message Type	8

Reset Event Message (0xFE) | Interface Module  ATD	8

Force Restart ATD Command | Interface Module  ATD	8

Force Restart MSG Receive Event Message (0x01) | Interface Module  ATD	8

Data Initial Start Command Message (0x85) | Interface Module  ATD	8

System Time Command (0xB2) | Interface Module  ATD	8

ATD Number Command (0xB7) | Interface Module  ATD	9

System Common Information Command (0x88) | Interface Module  ATD	9

10 Ext Type Message (0xA0) | Interface Module  ATD	9

Numbering Plan Message (0xA1) | Interface Module  ATD	10

Volume Control Command (0xD0) | Interface Module  ATD	10

Ring Control Command (0xD1) | Interface Module  ATD	11

Initial Lock Time (0xA3) | Interface Module  ATD	11

Start Command (0x82) | Interface Module  ATD	11

ATD Own Number Verify Request Command (0xD2) | Interface Module  ATD	12

Night Report Message (0x2B) | Interface Module  ATD	12

LED Control Command (0xA6)	12

Ready Event (0x02) | Interface Module  ATD	13

ATD MESSAGE TYPE	14

ATD  Interface Module Message	14

Force Restart MSG Receive Event Message (0x01)	14

Function Code EXP Event Message(0x27)	14

Keypad Event Message (0x28)	14

One Digit Event Message (0x29)	15

Stream Digit Event Message (0x2A)	16

Night Key Event Message (0x2B)	16

SLT State Request Event Message (0x2C)	16

Input String Digit Event Message (0x2D) – Reserved	16

Periodic Report of ATD Status(Every 15sec) Event Message (0x31)	17

Linked DLS Flexible Button Event Message (0x32)	17

Interface Module Restart Message (0xA2 0x00)	17

Interface Module Mute On Message (0xA2 0x01)	17

Interface Module Hand Free On Message (0xA2 0x02)	17

Interface Module Hand Set On Message (0xA2 0x03)	18

Interface Module SIO Test Message (0xA2 0x04)	18

Interface Module Mute Off Message (0xA2 0x05)	18

Interface Module Volume Change Message (0xA2 0x06)	18

Interface Module COMBO Set Message (0xA2 0x07)	19

Reset Event Message (0xFE)	19

Interface Module  ATD Message	19

Force Restart ATD Command	19

SLT Type Command (0xA0)	20

1st Digit Numbering Plan Command (0xA1)	20

2nd Digit Numbering Plan Command (0xA2)	20

Initail Lock Time Command (0xA3)	21

SLT Busy Information Display Command (0xA5)	21

LED Control Command (0xA6)	21

Multi Port Busy Information 1 Command (0xA8)	22

Multi Port Busy Information 2 Command (0xA9)	23

LCD Display Command (0xAA)	23

LCD Line Clear Command (0xAB)	25

ATD Key Error Command (0xAC)	25

System Time Command (0xB2)	25

Admin Mode Command (0xB4)	25

ATD Number Command (0xB7)	26

Numbering Wait Call Command (0xB8)	26

Charge Information 1 Command (0xB9)	26

Charge Information 2 Command (0xBA)	27

Supervisor Information (0xBC)	27

ATD Console Release Command (0xBF)	27

Cursor Position Command (0xC2)	28

One Digit Request Command (0xC3)	28

Stream Digit Request Command (0xC4)	28

Display Alarm Information Command (0xC7)	28

Volume Control Command (0xD0)	29

Ring Control Command (0xD1)	29

ATD Own Number Verify Request Command (0xD2)	29

Mandatory Process	30

# TITLE: ATD MESSAGE DESCRIPTION FOR EXTERNAL DEVICE

## Conception

This document is for the message between CS1000 PC-ATD Interface Box and External Device as follows.

ATD MESSAGE

CS1000 System

DSLC

CS1000 PC-ATD

Interface Box

External Device

SIO Cable

The ATD message consists of the header field, data field and the end field.

## Message Structure

Interface Module  ATD

| Classification | Data Position | Contents | Length | Data |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 |  |
|  | [02]~[N+1] | Data | 0 < N ≤ 10 |  |
|  | [N+2] | End Data Mark | 1 | 0xFF |
|  | [N+3]~[12] | Don’t Care | 10 – N | 0x00 |
| End Field | [13] | ETX | 1 | 0x03 |

◆ The Message which be sent by Interface Module to ATD has 14 byte fixed data length. This Message consists of Header field, Data Field and End Field. The Header Field and End Field Data has fixed value in fixed position but Data Field Data varies which data length(N).

◆ The STX(0x02), the End Data Field(0xFF), and the ETX(0x03) have fixed their value which length is 1 byte.

Interface Module ATD

| Classification | Data Position | Contents | Length | Data |
| --- | --- | --- | --- | --- |
| Data Field | [00] | Function Code | 1 |  |
|  | [01]~[N] | Data | N |  |
| End Field | [N+1] | End Data Mark | 1 | 0xFF |

◆ The Message which be sent by ATD to Interface Module are simple but has variable length.

◆ The Data Field consist of Function Code(fixed length 1) and Data(variable length). The End Field has only End Data Mark which value is 0xFF 1 byte.

Critical Note

Above all Message (Interface ModuleATD & Interface ModuleATD) has End Data Mark which value is 0xFF. The End Data Mark indicate the end of function data. But the Message may be has some function data which include 0xFF valued byte. Therefore, DSLC convert Message Data before send to Interface Module by reason above. DSLC convert Data Field data except End Data Mark which value is 0xFF to 0xEE. Interface Module just attaches STX and ETX So, ATD which received Message analyze 0xEE to 0xFF and when send some message to Interface Module, convert data 0xFF to 0xEE except the End Data Mark.

# IMPLEMENTATION CONSIDERATION

## Implementation Consideration of SIO(Serial Input Output)

Physical Link

| Electrical Interface Characteristic: | EIA RS-232 |
| --- | --- |
| Operation Mode: | Half Duplex |
| Interface Distance: | 15 meter (50 ft) between Switch and PMS Server |
| Baud Rate: | 19,200 bps |
| Worked Framing: | 8 bit data and 1 stop bit |
| Parity: | None |

Cable Connection

Switch has 9 pins female connector.

The follow is a method of physical link connection.

| Switch |  | PMS Server |
| --- | --- | --- |
| Pin 2 – Transmit (TX) |  | Receiver (RX) – Pin 3 |
| Pin 3 – Receiver (RX) |  | Transmit (TX) – Pin 2 |
| Pin 5 – Ground (GND) |  | Ground (GND) – Pin 5 |

# ATD INITIAL PROCESS

## ATD Initial Message Flow

| Interface Module |  |  | ATD |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ATD Restart |
|  | Reset Event (0xFE) |  |  |  |  |
|  | Force Restart ATD Command (0xAF) |  |  |  |  |
|  | Force Restart MSG Receive Event (0x01) |  |  |  |  |
|  | Data Initial Start Command (0x85) |  |  |  |  |
|  | System Time (0xB2) |  |  |  |  |
|  | ATD Number (0xB7) |  |  |  |  |
|  | System Common Data Initial (0x88) |  |  |  |  |
|  | 10 Ext Type (0xA0) |  |  |  |  |
|  | Numbering Plan Data (0xA1) |  |  |  |  |
|  | Volume Control (0xD0) |  |  |  |  |
|  | Ring Control (0xD1) |  |  |  |  |
|  | Initial Lock Time (0xA3) |  |  |  |  |
|  | Start Command (0x82) |  |  |  |  |
|  | ATD Own No. Req. (0xD2) |  |  |  |  |
|  | Night Report (0x2B) |  |  |  |  |
|  | (Display String) |  |  |  |  |
|  | Night LED (0xA6) |  |  |  |  |
|  | (Display String) |  |  |  |  |
|  | Ready Event (0x02) |  |  |  |  |
|  |  |  |  |  | End of ATD Initial Process |

## ATD Initial Message Type

### Reset Event Message (0xFE) | Interface Module  ATD

| Description | Inform Restart of ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xFE |
| Data Field | [01] | Sub Function Code | 1 | 0xEF |
| End Field | [01] | End Data Mark | 1 | 0xFF |

### Force Restart ATD Command | Interface Module  ATD

| Description | Force Restart ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | [0xAF] | 1 | 0xAF |
|  | [02] | [0xAF] | 1 | 0xAF |
|  | [03] | End Data Mark [0xFF] | 1 | 0xFF |
|  | [04]~[12] | Don’t Care | 9 | 0x00 |
| End Field | [13] | ETX | 1 | 0x03 |

### Force Restart MSG Receive Event Message (0x01) | Interface Module  ATD

| Description | Inform that Force Restart MSG from Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x01 |
|  | [01] | Sub Function Code | 1 | 0x01 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

### Data Initial Start Command Message (0x85) | Interface Module  ATD

| Description | Inform of data initial start. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0x85 |
|  | [02]~[11] | Reserved | 10 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### System Time Command (0xB2) | Interface Module  ATD

| Description | Inform ATD of system time. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB2 |
|  | [02] | Hour | 1 |  |
|  | [03] | Minute | 1 |  |
|  | [04] | Second | 1 |  |
|  | [05]~[11] | Don’t Care | 7 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Number Command (0xB7) | Interface Module  ATD

| Description | Inform serial number of ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB7 |
|  | [02] | Serial Number 0 ~ 7 | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### System Common Information Command (0x88) | Interface Module  ATD

| Description | Inform that system common information. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0x88 |
|  | [02] | Reserved 1 | 1 | 0xFF |
|  | [03]~[04] | Display Language 0x34FF: SPAIN 0x39FF: ITALY 0x45FF: DENMARK 0x82FF: KOREAN | 2 |  |
|  | [05]~[06] | Reserved 2 | 2 | 0x00 |
|  | [07] | Voice Coding Type (a-law fixed) | 1 | 0x01 |
|  | [08] | Reserved 3 | 1 | 0x01 |
|  | [09]~[11] | Reserved 4 | 3 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### 10 Ext Type Message (0xA0) | Interface Module  ATD

| Description | Request SLT type from ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA0 |
|  | [02] | Data 0 : 10 station LED general use | 1 | 0x00 |
|  | [03] ~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Numbering Plan Message (0xA1) | Interface Module  ATD

| Description | Inform ATD of numbering plan at system. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data | [01] | Function Code | 1 | 0xA1 |
|  | [02] | Type 0: Extension 1: Speed 3: Trunk 4: Emergency 7: Trunk Serial No. 8: ATD Serial No. | 1 |  |
|  | [03]~[06] | In Case of Extension Type [NP1] [NP2] [0xFF] [RC] In Case of Speed Type [NP1] [NP2] [0xFF] [RC] In Case of Trunk Type [NP1] [NP2] [0xFF] [RC] In Case of Emergency Type [ET1] [ET2] [ET3] [0x00] In Case of Trunk Serial No. [0xFF] [0xFF] [0xFF] [0x04] In Case of ATD Serial No. [0xFF] [0xFF] [0xFF] [0x02] | 4 |  |
|  | [07]~[11] | Don’t Care | 5 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

◆ NP: Numbering Plan Data	◆ RC: Rest Count		◆ ET: Emergency Telephone No. Data

### Volume Control Command (0xD0) | Interface Module  ATD

| Description | Control volume at ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD0 |
|  | [02] | Mode 0: Day Mode 1: Night Mode | 1 |  |
|  | [03] | Head Set Gain | 1 |  |
|  | [04] | Hand Set Gain | 1 |  |
|  | [05]~[11] | Reserved | 7 | 0x20 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Ring Control Command (0xD1) | Interface Module  ATD

| Description | Control ring volume at ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD1 |
|  | [02] | Ring Gain | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Initial Lock Time (0xA3) | Interface Module  ATD

| Description | Request night information from ATD |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA3 |
|  | [02] | Data 0: Day 1: Night | 1 | 0x00 |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Start Command (0x82) | Interface Module  ATD

| Description | Inform of start process. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0x82 |
|  | [02] | Reserved 1 | 1 | 0x03 |
|  | [03]~[11] | Reserved 2 | 9 | 0x00 |
|  | [12] | End Data Field | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Own Number Verify Request Command (0xD2) | Interface Module  ATD

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD2 |
|  | [02] | ATD Extension (reserved) | 1 | 0x00 |
|  | [03]~[05] | Extension Number (digit) | 3 |  |
|  | [06] | Reserved | 1 | 0x00 |
|  | [07]~[11] | Don’t Care | 5 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Night Report Message (0x2B) | Interface Module  ATD

| Description | Inform that Night Key button was pushed on ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x2B |
|  | [01] | Mode 0x00: Day Mode 0x01: Night Mode | 1 |  |
| End | [02] | End Data Field | 1 | 0xFF |

### LED Control Command (0xA6)

| Description | Control LED at ATD |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA6 |
|  | [02] | LED No. 0x00: Paging Key 0x01: R/D Ring Key 0x02: Charge Key 0x03: Overflow Key 0x04: ADM Key 0x05: Enter Key 0x06: Check Out Key 0x07: Check In Key 0x10: Room Cut Key 0x11: Message Wait Key 0x12: DND Key 0x13: Wake Up Key 0x14: Room Status Key 0x15: Forced Release Key 0x16: Supervisor Key 0x17: Trunk Selection Key 0x20: Conference Key 0x21: Busy Group Key 0x22: ATD Transfer Key 0x23: Mute Key 0x24: Answer Key 0x25: Station Answer Key 0x26: Trunk Answer Key 0x27: Recall Answer Key 0x30: Group Call Key 0x31: Break In Key 0x32: Camp On Key 0x33: Hold Verify Key 0x34: Hold Key 0x35: Release Key 0x36: Verify Key 0x37: Clear Key 0x40: Connect Key 0x41: Call Wait Key 0x42: Bell Off Key 0x43: Night Key 0x44: Minor Alarm Key 0x45: Major Alarm Key 0x50: Hold 1 Key 0x51: Hold 2 Key 0x52: Hold 3 Key 0x53: Hold 4 Key 0x54: Hold 5 Key | 1 |  |
|  | [03] | Rate 0: LED Off 1: LED On 2: LED Flash | 1 |  |
|  | [04]~[11] | Don’t Care | 8 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Ready Event (0x02) | Interface Module  ATD

| Description | Inform that ready status. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [01] | Function Code | 1 | 0x02 |
|  | [02] | Initial Status 0x00: Initial Gook 0x01: Initial Error | 1 |  |
| End Field | [03] | End Data Field | 1 | 0xFF |

# ATD MESSAGE TYPE

## ATD  Interface Module Message

### Force Restart MSG Receive Event Message (0x01)

| Description | Inform that Force Restart MSG from Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x01 |
|  | [01] | Sub Function Code | 1 | 0x01 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

### Function Code EXP Event Message(0x27)

| Description | Inform of function code EXP event. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x27 |
| End Field | [01] | End Data Mark | 1 | 0xFF |

### Keypad Event Message (0x28)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x28 |
|  | [01] | Key Value 0x8A: Paging (OPT 1) 0x8B: R/D Ring (OPT 2) 0x8C: Charge (CHRG) 0x8D: Overflow (OVF) 0x8E: Admin (ADM) 0x9A: Enter (ENT) 0x9B: Check In/Out (CHK IN/OUT) 0x9C: Pick Up (PICKUP) 0x9D: Room Cut (ROOM CUT) 0x9E: Message Wait (MSG WAIT) 0xAA: DND (DND) 0xAB: Wake Up (WAKE UP) 0xAC: Room Status (ROOM STS) 0xAD: Force Release (FORC RLS) 0xAE: Supervisor (SUPV) 0xBA: Trunk Selection (TRK SEL) 0xBB: Conference (CONF) 0xBC: Busy Group (BUSY GRP) 0xBD: ATD Transfer (ATD XFR) 0xC0: Bell Volume Up (BELL UP) 0xC1: Bell Volume Down (BELL DN) 0xC2: Volume Up (VOLM UP) 0xC3: Volume Down (VOLM DN) 0xC8: Answer (ANSWER) 0xC9: Station Answer (STA ANS) 0xCA: Trunk Answer (TRK ANS) 0xCB: Recall Answer (RCL ANS) 0xD8: Group Call (GRP CALL) 0xD9: Break-In (BRK IN) 0xE8: Camp On (CAMP ON) 0xE9: Hold Verify (HOLD VERF) 0xEA: Hold (HOLD) 0xF8: Release (RLS) 0xF9: Verify (CALL HOLD) 0xFA: Clear (CLR) 0xFB: Connection (CONNECT) | 1 |  |
| End Field | [02] | End Data Mark | 1 | 0xFF |

### One Digit Event Message (0x29)

| Description | Inform that Digit Key was pushed on ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x29 |
|  | [01] | Digit 0x00~0x0C 0x9C (Check In Key) | 1 |  |
| End Field | [02] | End Data Field | 1 | 0xFF |

### Stream Digit Event Message (0x2A)

| Description | Inform of stream digit status on ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x2A |
|  | [01] | Digit Type 0x00: Extension 0x01: Speed Dial 0x02: TIE 0x03: C/O 0x04: Emergency Dial 0x05: Ind. Paging 0x06: Trk. Paging 0x07: Trk. Serial No. 0x08: ATD Serial No. 0x09: Time 0x0A: Date | 1 |  |
|  | [02]~[04] | Digit | 3 |  |
| End | [05] | End Data Mark | 1 | 0xFF |

### Night Key Event Message (0x2B)

| Description | Inform that Night Key button was pushed on ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x2B |
|  | [01] | Mode 0x00: Day Mode 0x01: Night Mode | 1 |  |
| End | [02] | End Data Field | 1 | 0xFF |

### SLT State Request Event Message (0x2C)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x2C |
|  | [01] | Tel No. (High) | 1 |  |
|  | [02] | Tel No. (Middle) | 1 |  |
|  | [03] | Tel No. | 1 |  |
| End | [04] | End Data Mark | 1 | 0xFF |

### Input String Digit Event Message (0x2D) – Reserved

| Description | Inform that pushed string digit at admin status. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x2D |
| End Field | [01] | End Data Mark | 1 | 0xFF |

### Periodic Report of ATD Status(Every 15sec) Event Message (0x31)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x31 |
| End Field | [01] | End Data Mark | 1 | 0xFF |

### Linked DLS Flexible Button Event Message (0x32)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0x32 |
| End Field | [01] | End Data Mark | 1 | 0xFF |

### Interface Module Restart Message (0xA2 0x00)

| Description | Restart Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x00 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

### Interface Module Mute On Message (0xA2 0x01)

| Description | Active Mute On Mode in Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x01 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ Interface Module keeps Mute On mode until receives Interface Module Mute Off Message.

### Interface Module Hand Free On Message (0xA2 0x02)

| Description | Active Hand Free On Mode in Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x02 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ Interface Module keeps Hand Free On mode until receives Interface Module Hand Set On Message.

### Interface Module Hand Set On Message (0xA2 0x03)

| Description | Active Hand Set On Mode in Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x03 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ Interface Module keeps Hand Set On mode until receives Interface Module Hand Free On Message.

### Interface Module SIO Test Message (0xA2 0x04)

| Description | Test SIO Port. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x04 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ When Interface Module receives this MSG, Interface Module response as [0xB3] [0xA5] [0xFF].

### Interface Module Mute Off Message (0xA2 0x05)

| Description | Active Mute Off Mode in Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x05 |
| End Field | [02] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ Interface Module keeps Mute Off mode until receives Interface Module Mute On Message.

### Interface Module Volume Change Message (0xA2 0x06)

| Description | Control Hand Free/Hand Set Receive Volume. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x06 |
|  | [02] | Volume Type 0x01: Hand Free Volume 0x02: Hand Set Volume | 1 |  |
|  | [03] | Gain Value (0x01 ~ 0x0F) | 1 |  |
| End Field | [04] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

◆ The Gain Value is larger, the sound is louder.

◆ When Hand Free/Hand Set Mode is changed, Interface Module applies each gain value because of Interface Module keeps each gain value.

### Interface Module COMBO Set Message (0xA2 0x07)

| Description | Set COMBO Mode of Interface Module. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xA2 |
|  | [01] | Sub Function Code | 1 | 0x07 |
|  | [02] | COMBO Mode 0x00: A-law 0x01: U-law |  |  |
| End Field | [03] | End Data Mark | 1 | 0xFF |

◆ Interface Module recognizes this MSG, but doesn’t forward one to DSLC.

### Reset Event Message (0xFE)

| Description | Inform Restart of ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Data Field | [00] | Function Code | 1 | 0xFE |
| Data Field | [01] | Sub Function Code | 1 | 0xEF |
| End Field | [01] | End Data Mark | 1 | 0xFF |

## Interface Module  ATD Message

### Force Restart ATD Command

| Description | Force Restart ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | [0xAF] | 1 | 0xAF |
|  | [02] | [0xAF] | 1 | 0xAF |
|  | [03] | End Data Mark [0xFF] | 1 | 0xFF |
|  | [04]~[12] | Don’t Care | 9 | 0x00 |
| End Field | [13] | ETX | 1 | 0x03 |

### SLT Type Command (0xA0)

| Description | Request SLT type from ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA0 |
|  | [02] | Data 0 : 10 station LED general use | 1 | 0x00 |
|  | [03] ~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### 1st Digit Numbering Plan Command (0xA1)

| Description | Inform ATD of numbering plan at system. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA1 |
|  | [02] | Type 0: Extension 1: Speed 3: Trunk 4: Emergency 7: Trunk Serial No. 8: ATD Serial No. | 1 |  |
|  | [03]~[06] | In Case of Extension Type [NP1] [NP2] [0xFF] [RC] In Case of Speed Type [NP1] [NP2] [0xFF] [RC] In Case of Trunk Type [NP1] [NP2] [0xFF] [RC] In Case of Emergency Type [ET1] [ET2] [ET3] [0x00] In Case of Trunk Serial No. [0xFF] [0xFF] [0xFF] [0x04] In Case of ATD Serial No. [0xFF] [0xFF] [0xFF] [0x02] | 4 |  |
|  | [07]~[11] | Don’t Care | 5 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

◆ NP: Numbering Plan Data	◆ RC: Rest Count		◆ ET: Emergency Telephone No. Data

### 2nd Digit Numbering Plan Command (0xA2)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA2 |
|  | [02]~[11] | Don’t Care | 10 |  |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Initail Lock Time Command (0xA3)

| Description | Request night information from ATD |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA3 |
|  | [02] | Data 0: Day 1: Night | 1 | 0x00 |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### SLT Busy Information Display Command (0xA5)

| Description | Response to SLT State Request Event Message that display SLT busy information. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA5 |
|  | [02] | Information 1 | 1 |  |
|  | [03] | Information 2 | 1 |  |
|  | [04]~[11] | Don’t Care | 8 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### LED Control Command (0xA6)

| Description | Control LED at ATD |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA6 |
|  | [02] | LED No. 0x00: Paging Key 0x01: R/D Ring Key 0x02: Charge Key 0x03: Overflow Key 0x04: ADM Key 0x05: Enter Key 0x06: Check Out Key 0x07: Check In Key 0x10: Room Cut Key 0x11: Message Wait Key 0x12: DND Key 0x13: Wake Up Key 0x14: Room Status Key 0x15: Forced Release Key 0x16: Supervisor Key 0x17: Trunk Selection Key 0x20: Conference Key 0x21: Busy Group Key 0x22: ATD Transfer Key 0x23: Mute Key 0x24: Answer Key 0x25: Station Answer Key 0x26: Trunk Answer Key 0x27: Recall Answer Key 0x30: Group Call Key 0x31: Break In Key 0x32: Camp On Key 0x33: Hold Verify Key 0x34: Hold Key 0x35: Release Key 0x36: Verify Key 0x37: Clear Key 0x40: Connect Key 0x41: Call Wait Key 0x42: Bell Off Key 0x43: Night Key 0x44: Minor Alarm Key 0x45: Major Alarm Key 0x50: Hold 1 Key 0x51: Hold 2 Key 0x52: Hold 3 Key 0x53: Hold 4 Key 0x54: Hold 5 Key | 1 |  |
|  | [03] | Rate 0: LED Off 1: LED On 2: LED Flash | 1 |  |
|  | [04]~[11] | Don’t Care | 8 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Multi Port Busy Information 1 Command (0xA8)

| Description | Inform ATD of busy group information. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA8 |
|  | [02] | Busy Group Information 1 | 1 |  |
|  | [03] | Busy Group Information 2 | 1 |  |
|  | [04] | Busy Group Information 3 | 1 |  |
|  | [05] | Busy Group Information 4 | 1 |  |
|  | [06] | Busy Group Information 5 | 1 |  |
|  | [07] | Busy Group Information 6 | 1 |  |
|  | [08] | Busy Group Information 7 | 1 |  |
|  | [09]~[11] | Don’t Care | 3 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Multi Port Busy Information 2 Command (0xA9)

| Description | Inform ATD of busy group information. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xA9 |
|  | [02] | Busy Group Information 1 | 1 |  |
|  | [03] | Busy Group Information 2 | 1 |  |
|  | [04] | Busy Group Information 3 | 1 |  |
|  | [05] | Busy Group Information 4 | 1 |  |
|  | [06] | Busy Group Information 5 | 1 |  |
|  | [07] | Busy Group Information 6 | 1 |  |
|  | [08]~[11] | Don’t Care | 4 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### LCD Display Command (0xAA)

| Description | Display specific message in ATD LCD |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xAA |
|  | [02] | ATD LCD Field 0x01: LCD Field 1 0x02: LCD Field 2 0x03: LCD Field 3 0x04: LCD Field 4 0x05: LCD Field 5 0x06: LCD Field 6 0x07: LCD Field 7 0x08: LCD Field 8 | 1 |  |
|  | [03] | Display Type 0x00: LCD Field Clear In case of LCD field 5 0x01: Calling Extension (Tenant, Class) 0x02: Outgoing Trunk (Tenant, Trunk Serial, Class) In case of LCD field 6 0x01: IDLE 0x02: BUSY 0x03: CONNECT 0x04: BRK-IN 0x05: GRP-BUSY 0x06: Wake Up Time (Time, Day) 0x07: MSG 0x08: DND 0x09: CHK-IN 0x0A: CHK-OUT 0x0B: ROOM-CUT 0x0C:CMP-BUSY 0x0D: RING 0x0E: RBT 0x0F: DIAL 0x10: TCONV 0x11: ECONV 0x12: HOLD 0x13: CONF 0x14: HOWLING 0x15: GUARD 0x16: ALL-BUSY In case of LCD field 7 0x01: Held Extension (Tenant, Extension, Class) 0x02: Held Trunk (Tenant, Trunk Serial, Class) 0x03: NIGHT 0x04: ERROR 0x05: Incoming Extension (Tenant, Extension, Class) 0x06: Incoming Trunk (Trunk, Trunk Serial, Class) 0x07: CFWD Outside Tel No. 0x08: OUT OF LUNCH 0x09: ON VACATION 0x0A: BACK AM 0x0B: BACK PM 0x0C: BACK TOMORROW 0x0D: OUT OF TOWN 0x0E: IN A MEETING 0x0F: AT HOME 0x10: AT BRANCH OFFICE 0x11: TO BE CLEAR 0x12: CLEARED 0x13: RESERVED 0x14: SERVICE END 0x15: UNDER REPAIR 0x16: REPAIRED In case of LCD field 8 0x01: Incoming Station 0x02: Incoming Trunk 0x03: Recall Paging 0x04: Recall No Answer 0x05: Recall Camp On 0x06: Recall Transfer 0x07: Recall Wake Up 0x08: Recall Message 0x09: DND 0x0A: AFWD 0x0B: BFWD 0x0C: NFWD 0x0D: OFWD | 1 |  |
|  | [04]~[11] | Display Buffer or 0x00 | 8 |  |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### LCD Line Clear Command (0xAB)

| Description | Request clear specific LCD line. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xAB |
|  | [02] | LCD Line Number 0x01: Line 1 0x02: Line 2 0x03: Line 3 0x04: Line 4 | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Key Error Command (0xAC)

| Description | Inform ATD of key operation error. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xAC |
|  | [02]~[11] | Don’t Care | 10 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### System Time Command (0xB2)

| Description | Inform ATD of system time. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB2 |
|  | [02] | Hour | 1 |  |
|  | [03] | Minute | 1 |  |
|  | [04] | Second | 1 |  |
|  | [05]~[11] | Don’t Care | 7 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Admin Mode Command (0xB4)

| Description | Inform that ATD works in admin mode. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB4 |
|  | [02] | Admine On | 1 | 0x00 |
|  | [03]~[11] | Don’t Care | 9 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Number Command (0xB7)

| Description | Inform serial number of ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB7 |
|  | [02] | Serial Number 0 ~ 7 | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Numbering Wait Call Command (0xB8)

| Description | Inform of call number which wait for response of ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB8 |
|  | [02] | Trunk Call Number | 1 |  |
|  | [03] | Recall Call Number | 1 |  |
|  | [04] | Extension Call Number | 1 |  |
|  | [05]~[11] | Reserved | 7 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Charge Information 1 Command (0xB9)

| Description | Inform of charge information. (tenant, tel no., duration time) |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xB9 |
|  | [02] | Tenant Number | 1 |  |
|  | [03] | Tel No. (High) | 1 |  |
|  | [04] | Tel No. (Middle) | 1 |  |
|  | [05] | Tel No. (Low) | 1 |  |
|  | [06] | Duration Time (Hour) | 1 |  |
|  | [07] | Duration Time (Minute) | 1 |  |
|  | [08] | Duration Time (Second) | 1 |  |
|  | [09]~[11] | Reserved | 3 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Charge Information 2 Command (0xBA)

| Description | Inform of charge information. (outgoing digit) |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xBA |
|  | [02] | Number of Outgoing Digit | 1 |  |
|  | [03]~[11] | Dialed Digit | 9 |  |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Supervisor Information (0xBC)

| Description | Display ATD Monitoring Information. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xBC |
|  | [02] | Trunk Serial Number | 1 |  |
|  | [03] | Rel 1 Tenant Number | 1 |  |
|  | [04] | Rel 1 Trunk ID (Decimal) | 1 |  |
|  | [05] | Rel 1 Trunk ID (Decimal) | 1 |  |
|  | [06] | Reserved | 1 | 0xFF |
|  | [07] | Rel 2 Tenant Number | 1 |  |
|  | [08] | Rel 2 Trunk ID (Decimal) | 1 |  |
|  | [09] | Rel 2 Trunk ID (Decimal) | 1 |  |
|  | [10] | Reserved | 2 | 0xFF |
|  | [11] | Don’t Care | 1 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Console Release Command (0xBF)

| Description | Release ATD Console. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xBF |
|  | [02]~[11] | Don’t Care | 10 | 0xFF |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Cursor Position Command (0xC2)

| Description | Appoint position of cursor on ATD LCD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xC2 |
|  | [02] | Position of Cursor | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0x20 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### One Digit Request Command (0xC3)

| Description | Request one digit event. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xC3 |
|  | [02]~[11] | Don’t Care | 10 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Stream Digit Request Command (0xC4)

| Description | Request stream digit event. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xC4 |
|  | [02]~[11] | Don’t Care | 10 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Display Alarm Information Command (0xC7)

| Description | Display alarm information |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xC7 |
|  | [02] | Critical Alarm 0: Off 1: On | 1 |  |
|  | [03] | Major Alarm 0: Off 1: On | 1 |  |
|  | [04] | Minor Alarm 0: Off 1: On | 1 |  |
|  | [05] | Buzzer 0: Off 1: On | 1 |  |
|  | [06]~[11] | Don’t Care | 6 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Volume Control Command (0xD0)

| Description | Control volume at ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD0 |
|  | [02] | Mode 0: Day Mode 1: Night Mode | 1 |  |
|  | [03] | Head Set Gain | 1 |  |
|  | [04] | Hand Set Gain | 1 |  |
|  | [05]~[11] | Reserved | 7 | 0x20 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### Ring Control Command (0xD1)

| Description | Control ring volume at ATD. |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD1 |
|  | [02] | Ring Gain | 1 |  |
|  | [03]~[11] | Don’t Care | 9 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

### ATD Own Number Verify Request Command (0xD2)

| Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Classification | Data Position | Contents | Length | Data |
| Header Field | [00] | STX | 1 | 0x02 |
| Data Field | [01] | Function Code | 1 | 0xD2 |
|  | [02] | ATD Extension (reserved) | 1 | 0x00 |
|  | [03]~[05] | Extension Number (digit) | 3 |  |
|  | [06] | Reserved | 1 | 0x00 |
|  | [07]~[11] | Don’t Care | 5 | 0x00 |
|  | [12] | End Data Mark | 1 | 0xFF |
| End Field | [13] | ETX | 1 | 0x03 |

## Mandatory Process

In ATD, when operator push any digit button which unmatched system numbering plan received from system(Interface Module) at initial time, ATD must display Error Message instead of sending One Digit Event Message(0x29) or Stream Digit Event Message(0x2A).

In ATD state is IDLE or CONVERSATION, operator push digit button which matched system numbering plan(Extension Number, Trunk Access Code or Speed Dial) received from system(Interface Module) at initial time, send Stream Digit Event Message(0x2A) instead of One Digit Event Message(0x29). If ATD doesn’t received special digit input mode message(0x29, 0x2A), ATD must send One Digit Event Message(0x29) for the following digit which operator pushed.