LG PBX System

### Software Technical Information

STI-02-0016

Apr 30, 2002

| Title | #### Inter Tenant Connection |
| --- | --- |
| System | CS1000 |
| Abstract | To allow or deny the connection between different tenants |

Revision History

| ###### Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2002.04.30 | H.G. Kang | First Issue |

Table of Contents

Software Technical Information	1

Title : Inter Tenant Connection	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

OMS Menu 3.4 Inter Tenant Connection Allow/Deny	3

	OMS Functional description	3

	How to use	3

Title : Inter Tenant Connection

Functional Description

The connection is programmable when try the connection trunk port from extension that have different tenant value

Operation Procedure

If try the connection between tenants that is denied, then hear the congestion tone.

Conditions

- This feature can use when want to use common trunk port from each extension that have different tenants

Data Generation

- Assign of extension’s tenant

- [3.2 Extension Tenant Assign]

- Assign of trunk’s tenant

- [4.5 Trunk Tenant Assign]

- Inter tenant connection allow between extension and trunk

- [3.4 Inter Tenant Connection Allow/Deny – Tenant Connection Allow/Deny(Trunk)]

Interaction

N/A

Hardware

N/A

OMS Menu 3.4 Inter Tenant Connection Allow/Deny

	OMS Functional description

Tenant Connection Allow/Deny(Extension)

-> Select it when want the confirm or change the connection each extension tenant

Tenant Connection Allow/Deny(Trunk)

-> Select it when want the confirm or change the connection between extension tenant and trunk tenant

	How to use

1	Query

Select the Query button to confirm the current state

2	Change and Send

- Check the item to connect tenants

- Select the Change and Send button to apply the new option value.