LG PBX System

### Software Technical Information

STI-xxxx

Jan 23, 2002

| Title | #### ATD Screen Transfer |
| --- | --- |
| System | CS1000 |
| Abstract | 중계대 협의 전환 기능. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.23.2002 | 정서영 | First developed feature. |

Table of Contents

Software Technical Information	1

Title : ATD Screen Transfer	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

Title : ATD Screen Transfer

Functional Description

중계대가 A가입자와 통화 중 B가입자 번호를 눌러 B가입자와 통화 후, 중계대가 A가입자를 B가입자로 전환하는 기능.

Operation Procedure

- ATD Screen Transfer

  - 중계대가 A가입자와 통화 중 B가입자 번호를 누른다.

  - B가입자가 IDLE인 경우 A가입자는 SPLIT되어 보류음을 듣고, 중계대는 Ring Back Tone을 듣고, B가입자는 Ring을 받는다.

  - B가입자가 응답하면 중계대와 통화 상태가 된다.

  - 중계대에서 연결(CONNECT) Key를 누르면 A가입자와 B가입자가 통화 상태가 된다.

- ATD Unscreened Transfer

  - 중계대가 A가입자와 통화 중 B가입자 번호를 누른다.

  - 중계대 LCD에 B가입자의 정보가 표시되고 B가입자가 IDLE인 경우 중계대가 연결(CONNECT) Key를 누르면 A가입자는 Ring Back Tone을 듣고, B가입자는 Ring을 받는다.

  - B가입자가 응답하면 A가입자와 통화 상태가 된다.

Conditions

중계대 Option에서 ATD Screen Transfer를 YES로 하면 위 Operation Procedure의 1과 같이 동작하고, NO로 하면 2와 같이 동작한다.

Data Generation

본 기능을 위해 설정하여야 할 Option은 다음과 같다.

5.6 ATD Service Option중 [ATD Screen Transfer]를 YES 또는 No로 지정한다.

Interaction

Hardware

본 기능을 위해 새로운 H/W는 필요 없음.