LG PBX System

### Software Technical Information

STI-xxxx

Feb 04, 2002

| Title | #### DTC(E1) nailed connection |
| --- | --- |
| System | CS1000 |
| Abstract | 임의의 DTC 포트의 통화로를 점유하여 다른 포트와 강제 switching하여 주는 기능 |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | Feb.04.2002 | H.G. Kang | First developed feature. |

Table of Contents

Software Technical Information	1

Title : ANI into AUTO ATD MSI Message	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

Title : DTC(E1) nailed connection

Functional Description

임의의 DTC 포트의 통화로를 점유하여 다른 포트와 강제 switching하여 주는 기능

Operation Procedure

시스템을 모니터링할 수 있도록 PC와 RS-232C를 연결한다.

모니터링 프로그램에서 아래와 같이 명령어를 수행하면 된다.

1) 강제 switching을 실행할 때

$call

CALL_MON> MBWSW P1 P2 cnt

2) 연결된 switching를 강제로 해제할 때

$call

CALL_MON> CBWSW P1 P2 cnt

* P1 : P2와 bothway로 switch를 연결시킬 임의의 포트

* P2 : P1과 bothway로 switch를 연결시킬 임의의 포트

* cnt : P1포트로부터 시작하여 강제 switching하기 위한 포트 개수

Conditions

Data Generation

Interaction

Hardware

N/A