LG PBX System

Software Technical Information

STI-0004

Dec 12, 2000

| Title | #### Calling Group – Log off (DND) |
| --- | --- |
| System | CS1000 |
| Abstract | In case an extension belonging to Calling Group has registered Group Call-Log Off function in its Group using the function code, the Calling Group called by its representative number is processed as DND. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 12.14.2000 | C.K Ahn | First issue |

Table of Contents

Title : Calling Group – Log off	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

	OMS Menu description	3

	How to use	3

	Contents	3

Title : Calling Group – Log off

Functional Description

In case an extension belonging to Calling Group has registered Group Call-Log Off function in its Group using the function code, the Calling Group called by its representative number is processed as DND.

Operation Procedure

Group Call Log-Off Registration

- An extension of Calling Group goes into Off-Hook.

- And registers Group Call Log-Off by dialing the code (Default: *991).

- Then can get confirmation tone if it is successfully registered.

Group Call Log-Off Cancellation

- An extension of Calling Group goes into Off-Hook..

- And cancels Group Call Log-Off by dialing the code (Default: *990).

- Then can get OK tone if it is successfully canceled

Group Call Log-Off Use

- Calls an extension.

- Dials the representative number of Calling Group in which Group Call Log-Off is registered.

- Group Call Log-Off means DND is already registered. Therefore Congestion Tone is listened in case of using a single line telephone. And in case of using DTEL, LCD indicates that the called extension registered DND plus Congestion Tone is listened.

Conditions

- In case an extension of Calling Group registers Group Call Log-Off in its Group, Ring Type of the Group must be one of "FST", "UCD", "MUL", and "LONGEST IDLE".

- Extension is permitted to register Group Call Log-Off only in its Group.

- If an extension belongs to two Calling Groups or more, the extension is permitted to register Group Call Log-Off only in the Calling Group.

- to which the terminal of the extension first belongs.

Data Generation

- Calling Group Log-Off Registration/Cancellation Code

- [2.1 Numbering Plan]

- Calling Group Assignment

- [6.4 Calling Group]

Interaction

- N/A

Hardware

N/A

	OMS Menu description

Refer to CS1000 System Manual – OMS Port. [2.1 System Numbering Plan  Feature Code]

	How to use

1	Query

N/A

2	Add

N/A

3	Change

N/A

4	Delete

	Contents

Function code plan

| Item | Description | Default |
| --- | --- | --- |
| Calling Group DND Cancel | Calling Group Log-Off Cancellation code | *990 |
| Calling Group DND Registration | Calling Group Log-Off Registration code | *991 |