LG PBX System

Software Technical Information

STI-0010

Jan. 31, 2001

| Title | #### System Installation OMS Guide |
| --- | --- |
| System | CS1000 |
| Abstract | System & OMS Installation Guide |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.31.2001 | B.D KIM | First Issue |

Table of Contents

System Installation Quick Guide	4

1. System Preparation	4

Check PWR	4

Check CPM/CPM2	4

Check a Firmware of all cards.	4

Cards Installation	4

SIO Check	4

Turn on System Power	4

Set IP	4

PC APP Program	5

Check DISKON Chip total size	5

CPM version Loading	5

Transferred file Size Check.	5

* FDISK Procedure	5

CPM Reset	6

Check board status	6

Select National Version (Country Code) and Input the information of installation place	6

Data Initialize	6

Customer Data Generation	6

Set Clock Detection Link	6

Make seed file for data back up	6

* Data Back-Up Download	6

2. Service Setting	7

BRI S0	7

BRI T0	7

PRI	7

QSIG	8

Card	8

BS	8

Phone	8

SLT	8

LKD	9

DSS	9

D-ATD	9

ITGC	9

VPM	9

ACD	9

Title : SLT Quick Guide

# System Installation Quick Guide

## 1. System Preparation

| No | Check Point | Detail Description | Remark |
| --- | --- | --- | --- |
| 1 | Check PWR | +5V |  |
| 2 | Check CPM/CPM2 | Version (Boot EPROM U37) Tone Spec (Tone EPROM U38) Check DTRU Position |  |
| 3 | Check a Firmware of all cards. | Verify a checksum of EPROM on each cards |  |
| 4 | Cards Installation | Verify card status with LED, Jumper setting. Insert in system. |  |
|  | SIO Check | System Monitor SIO Billing Monitor SIO |  |
| 5 | Turn on System Power | Verify auto generated files in DISKON Chip.($dir) func.dt dconf.dt. fauit.dq pmstx.dq |  |
| 6 | Set IP | Setting IP Address with SIO |  |
|  |  | H/W Jumper Setting Select Mode (User/ Network) | Using Cable Type (User=Direct / Network=Cross) |
|  |  | LAN Connection Test | Ping Test |
|  |  | * Trouble Shooting When Ping Test / OMS Access / FTP is not executed. 1.Check Network Information (PC and System) 2.Check User Mode/Network Mode in IOTU board  Setting IOTU Jumper after turning off Power  Power On 3.Check the connection status between Hubs 4.Subnet Mask Check 5.Connect LAN cable to System directly IOTU Network Mode : Use straight LAN cable IOTU User Mode : Use twisted LAN cable Note1) System Network information is saved in Clock Chip on the CPM board, so if you change network information, you have to reset CPM board. Note2) The setting information of system network is Clock Chip (U25) in CPM card. If CPM were replaced, User has to input network information for new card after connecting SIO(IOT1) |  |
| 7 | PC APP Program | Install OMS |  |
| 8 | Check DISKON Chip total size | Maximum 8M CPM.img = about 3M ( Be careful when you make backup fine in DISKON Chip.) |  |
| 9 | CPM version Loading | File Loading / CPM.img backup .DT/DQ file backup (optional) | By FTP Manager in OMS or DOS FTP |
| 10 | Transferred file Size Check. | $ dir |  |
| 11 | * FDISK Procedure | In case system restarts continuously because of a problem caused from CPM.img file in DISKON Chip, User should execute format for DISKON Chip and then must load the CPM.img file again with DOS ftp in off line mode. Please don’t forget CPM reset manually after finishing file Loading always. |  |
| 12 | CPM Reset | Press push button in faceplate of CPM or reset by OMS. | Manual/OMS |
| 13 | Check board status | [Configuration]  [1.1 System H/W Configuration] | By OMS |
| 14 | Select National Version (Country Code) and Input the information of installation place | Set Country Code + Installation area code + ISD Call Type Prefix + DDD Call Type Prefix + DDI Prefix for # Route + Own PBX Code [Data Management]  [1. System Initial Data Setup] [1.1 System Setup Information] | By OMS |
| 15 | Data Initialize | [Data Management] [2. System Data]  [2.2 System Local Board Information ; Press Sys_Init ] This means that customer data generated automatically at initial time will clear without board configuration data. | By OMS |
| 16 | Customer Data Generation | Numbering Plan for Extension / Trunk / Feature code | By OMS |
| 17 | Set Clock Detection Link | [Diagnosis][1.1 System Network Synchronize] | By OMS |
| 18 | Make seed file for data back up | [Data Management] ⓡmake data file (all data) = Now cs1000.dt is made. | By OMS |
| 19 | * Data Back-Up Download | From system to PC HDD. | By OMS FTP Manager. |

## 2. Service Setting

| Types | Items | Logical Setting | Physical Setting |
| --- | --- | --- | --- |
| ISDN | BRI S0 | [Data Management] [2. System Data]  [2.2 Local Board Information=Select BRIC + ⓡ Board Data Generation = Dialog : Select Use Type BRIC Board=Input the port of BRI extension] [Extension Data Handling Plan Dialog Window=Extension Type BRIS + Input Extension No base on numbering plan + Press Verify][3.1 Extension Information+ Send ] | Set Jumper for S0 point on BRIC |
|  | BRI T0 | [Data Management] 1. [2. System Data][2.1 System Numbering PlanTrunk Code Plan] 2. [2. System Data][2.2 Local Board Information=Select BRIC + ⓡ Board Data Generation][Dialog: Select Use Type BRIC Board= Trunk=Input the port of BRI trunk] [Trunk Data Handling Plan Dialog Window=Trunk Type BRIT + Press Verify][4.1 Trunk Information+ Send] 3. [4. Trunk Data][4.2 Trunk Access Code ⓡ on the Trk Acc Code, Trunk O/G Group ViewAssign trunk outgoing port for trunk access code] | CLKU Set Jumper for T0 point |
|  | PRI | [Data Management] - [2. System Data][2.1 System Numbering PlanTrunk Code Plan] - [2. System Data]  [2.2 Local Board Information=Select DNIC(PRI) + ⓡ Board Data Generation] [Trunk Data Handling Plan Dialog Window=Trunk Type DTC-PRI + Press Verify][4.1 Trunk Information+ Send] - [4. Trunk Data][4.2 Trunk Access Code  ⓡ on the Trk Acc Code, Trunk O/G Group ViewAssign trunk outgoing port for trunk access code] - [2. System Data]  [2.2 Local Board Information=Select DINC(PRI) + ⓡ CRC Enable/Disable] 5. [2. System Data]  [2.2 Local Board Information=Select DINC(PRI) + ⓡ ISDN PRI Mode] | Select Cable Type (Twist or Coaxial) |
|  | QSIG | [Data Management] [2. System Data]  [2.2 Local Board Information ⓡ Board Exchange=change to EMPTY from DNIC(PRI)] [Local Board Information= Select DNIC-PRI + ⓡ Board Exchange =DNIC(PRI) + ⓡ CRC Enable / Disable + ⓡ ISDN PRI Mode ViewSelect User/Network Mode] [2. System Data]  [2.2 Local Board Information=Select DNIC-PRI + ⓡ Board Data Generation][Trunk Data Handling Plan Dialog Window=Trunk Type QSIG+ Press Verify][4.1 Trunk Information+ Send] [8. QSIG Data] 1. [OGIG Option / Supplementary Service] 2. ⓡ on the card [QSIG Group Outside Numbering Plan (Note; please avoid [2.1 System Numbering Plan]) 3. ⓡ on the card [QSIG Group O/G Port No] | Select Cable Type (Twist or Coaxial) |
| DECT | Card | Set DECT PARK Code from WTIC to CPM. Set DECT Authentication from WTIC to CPM. [Data Management] [9. DECT Data][9.1 PARK CODE][9.2 Authentication Code] | Refer to the WTIC board part in Installation Manual to register DECT Phone. |
|  | BS | --------------------------------------------------------------------------------------------------------------------------------- | Connect between WTIC and BS. |
|  | Phone | [9. DECT Data][9.3 DECT Information] 1. Data Clear, 2. Register Phone 3. Cancel Phone |  |
| A-EXT | SLT | [Data Management] [2. System Data]  [2.2 Local Board Information=Select ASLC + ⓡ Board Data Generation][Extension Data Handling Plan Dialog Window=Extension Type SLT+ Input Extension No base on numbering plan + Press Verify][3.1 Extension Information+ Send ] |  |
| D-TEL | LKD | [Data Management] 1.[2. System Data][2.2 Local Board Information=Select DSLC(LKD) + ⓡ Board Data Generation] [Extension Data Handling Plan Dialog WindowExtension Type DTEL LKD+ Input Extension No base on numbering plan + Press Verify][3.1 Extension Information+ Send ] 2.[3. Extension Data Management][3.1 Extension Information]  ⓡ [ DTEL Feature Inf.View]  ⓡ [D-Tel Flexible Button View= To assign flexible buttons] |  |
|  | DSS | [Data Management] 1.[3. Extension Data][3.1 Extension Information=Ext Type;DLS DTEL / Delete the D-TEL Telephone number + ⓡ on the Father DTEL Number =DTEL Feature Inf. View ][DTEL Feature + put mouse curse on D-TEL Father Number Port + Assign DSS# Port ] 2.[3. Extension Data][3.1 Extension Information]  ⓡ on the [DSS- Flexible Button View] |  |
|  | D-ATD | *1.[Data Management] [2. System Data]  [2.2 Local Board Information=Select DTEL-LGP + ⓡ Board Data Generation][Extension Data Handling Plan Dialog Window=Extension Type LGP] ][Extension Information=Ext Type ATD] 2.[.Data Management] [Extension Data Management][Extension Information=Ext Type ATD] 3.[Attendant Data] | DSLC-LGP |
| VoIP | ITGC | .[Trunk Data][Trunk Access Code]Trunk Outgoing Group Information] .[System Data]  [Local Board Information=Select ITGC + ⓡ Board Data Generation] [Trunk Data Handling Plan Dialog Window=Trunk Type = VoIP][4.1 Trunk Information] [Data Management] [VOIP Data] 1[VOIP Own Identification= Group Number(=Card No.) & Own IP & Netmask &Default IP] 2[VOIP O/G Call Routing Table] *3[VOIP Gatekeeper] |  |
| VMS | VPM | Refer to VPM Installation Manual |  |
| ACD | ACD | Refer to VPM Installation Manual |  |