LG PBX System

Software Technical Information

STI-0011

Jan. 31, 2001

| Title | #### Camp-On |
| --- | --- |
| System | CS1000 |
| Abstract | The sending method of Alert tone for Camp-On is changed |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.01.2000 | C.K Ahn | First issue. (CS1000 System Manual) |
| 1.1 | 01.31.2001 | C.K Ahn | Second issue from CS100 system Manual) |

Table of Contents

Title : Camp-On	2

Functional description	2

Operating procedure	2

Service conditions	3

Data generation	3

Interactions	3

Title : Camp-On

Functional description

The function allows a subscriber to inform a waiting call to an extension on line with other extension, and to wait for the answer from the extension.

Operating procedure

- Camp-On on single line telephone (SLT)

1	Extension (A) calls extension (B) whose line is busy.

2	Extension (A) hears busy tone.

3	Extension (A) presses Camp-On Registration Feature Code (Default: 6).

4	The hold tone is transmitted to extension (A), and the warning tone to extension (B).

5	Extension (B) hooks on the handset.

6	The ring back tone is transmitted to extension (A), and ring current is transmitted to extension (B).

7	Extension (B) hooks off the handset.

8	The call is performed between extension (A) and (B).

- Camp-On on D-Tel telephone

1	D-Tel (A) calls another D-Tel (B) whose line is busy.

2	The busy tone is heard as D-Tel (B) is on line.

3	The D-Tel (A) presses Camp-On Registration Feature Code (Default: 6).

4	The hold tone is transmitted to D-Tel (A), and the warning tone to D-Tel (B).

5	D-Tel (B) hooks on the handset.

6	Ring back tone is transmitted to D-Tel (A), and ring current is transmitted to D-Tel (B).

7	D-Tel (B) hooks off the handset.

8	The call is performed between D-Tel (A) and (B).

- Camp-On on attendant

1	Extension (A) is on line with extension (B).

2	The attendant is on line with extension (C) or outside subscriber (C).

3	The attendant calls extension (A).

4	The attendant hears the busy tone as extension (A) is on line.

5	The attendant presses “CAMP-ON” Key.

6	The attendant is released. The hold tone is transmitted to outside subscriber (C) and the warning tone is transmitted to extension   (A).

7	Extension (A) hooks on the handset.

8	the ring back tone is transmitted extension (C), and ring current is transmitted to extension (A).

9	If extension (A) hooks off the handset, a call is set up between extension (A) and (C).

Service conditions

- This function is not available when Step Call is used in the system.

- This function is not provided if the OMS does not assign Camp-On function to the called party, although Camp-On is available in the system.

- Camp-On cannot be registered to the extension assigned as Hot Line.

- Camp-On cannot be registered to the call group.

- 20 Camp-On registrations are available for an extension.

- If the extension that requested Camp-On hangs on, Camp-On function is cancelled.

- The OMS determines whether to transmit the warning tone to the extension that the Camp-On is requested to.

Option: Periodic    :  The warning tone will be sent periodically to extension requested camp-on do not response during caller hangs on the line.

One Time   :  The warning tone will be sent one-time only to extension requested camp-on do not response during caller hangs on the line.

- Warning tone is transmitted at regular intervals to the extension that the Camp-On is requested to. The OMS can adjust the time interval.

- When the country code is Italy, Camp-On service is not automatically performed when the opposite side is busy, but is performed only with pressing Camp-On request code (Default: 6).

Data generation

- Assign whether to use Step Call in the system.

[2.6 System option ⇒ Step Call Use]

- Assign whether to use automatic Camp-On.

[2.6 System option ⇒ Auto Camp-On Use]

- Assign whether to allow Camp-On function when required.

[3.1 Extension information – Extension Characteristic ⇒ 33 Camp-On]

- Determine whether to transmit which kinds of warning tone to caller when an extension requests Camp-On.

[2.6 System option ⇒ Camp-On Tone Type  Periodic / One Time]

- Assign the digit that performs the same work that H/F does.

[2.6 System option ⇒ DP Hook Flash Digit ]

Interactions

- When an extension enters Camp⇒On Digit to request the Camp-On, if the digit matches the value in [2.6 System option-DP Hook Flash Digit], the digit performs the same work that H/F does.

- In case the extension presses Hook Flash to request Camp-O, Off-Hook or On-Hook process is performed according to [2.6 System option-Hook Flash].

- If the extension is in the following service, the function is performed in the order of Auto Group Hunting/ Auto Hold/ Auto Call Waiting/ Auto Camp-On.