LG PBX System

- Software Technical Information

STI-0012

Jan. 31, 2001

| Title | #### Message Wait |
| --- | --- |
| System | CS1000 |
| Abstract | Message Wait Register method added in Idle |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.01.2000 | C.K Ahn | First issue. (CS1000 System Manual) |
| 1.1 | 01.31.2001 | C.K Ahn | Second issue from CS100 system Manual) |

Table of Contents

Title : Camp-On	2

Functional description	2

Operating procedure	2

Service conditions	5

Data generation	5

Interactions	5

Title : Message Wait

Functional description

When the called extension is absent, this function enables the calling extension to leave a message and notifies the called extension of the message when the called extension comes back.

Operating procedure

- Message waiting registration on SLT

Registration: Called No Answer

1	An extension dials an extension number.

2		The called extension does not answer the call since the called extension is absent.

3 The extension makes “hook flesh”

4	The calling extension presses the Hook Flash button and enters the message registration code (Default: *71).

5	The calling extension hears the service set tone.

6	If the terminal to which the message waiting is registered is D-Tel, the following message is displayed.

MSG EXT(1)TRK(0) VM(0) 2000-03-01 09:00 am

Registration : Idle Status ( This services to SLT Only)

1	An extension lifts handset.

2 	Press the message registration code (Default: *71) and destination extension number.

The calling extension hears the service set tone.

- How to verify Message waiting.

1. User press consecutively “CALL BK” button in waiting messages. And then the waiting messages will be displayed in order of arrived message.

- Message waiting answer in SLT

1	The extension for whom a message is left hooks off the handset.

2	The extension hears the function registration tone notifying that there is a message registered.

3	The extension enters the message answer code (Default: *72).

4	The extension receiving the message hears the ring back tone while the extension that registered a message hears the ringing.

5	The extension that registered the massage answers the ringing.

6	The call between the two extensions is established.

- Message waiting cancellation on SLT

1	The extension that leaves a message hooks off the handset

2	The extension enters the message cancellation code (Default: *70*) and the phone the number of the extension that the message is left for.

3	The service set tone is heard.

- Message waiting registration on D-Tel

Registration : Called No Answer

1	An extension dials another extension number.

2	The extension does not answer the call since the extension is absent.

3 	Press “Trans” Button.

3	The calling presses the “CALLBK”(Call Back) button.

4	The calling extension hears the service set tone.

5	If the terminal to which the message waiting is registered is D-Tel, the following message is displayed.

“MSG EXT(1)TRK(0) VM(0)    “

“2000-03-01       09:00 am  “

Registration : Idle Status

1	An extension lifts handset.

2 	Press the message “CALLBK”(Call Back) button and destination extension number.

3 The calling extension hears the service set tone.

- Message waiting answer on D-Tel

1	The message waiting is displayed on the LCD of the extension.

2	The extension presses the “MON” button.

3	The extension presses the “CALLBK” button.

4	The extension receiving the message hears ring back tone while the extension that registered the message hears the ringing.

5	The extension that registered the message answers the ring.

6	A call between the two extensions is established.

- Message waiting cancellation on D-Tel

1	An extension hooks off the handset.

2	The extension enters the message cancellation code (Default: *700). At this time, the following message is displayed on the LCD of D-Tel.

“ ENTER A EXTENSION NUMBER  “

3	The extension presses the number of another extension that received the message.

4	The extension hears the service set tone.

- Message waiting answer rejection on D-Tel

1	The extension presses “CALLBK” button.

2	The extension presses the “DND/FWD” button.

3	The extension hears the service set tone.

4	At this time, the deleted message is not displayed on the LCD screen any more.

- Message waiting registration by the outside subscriber

1	An outside subscriber calls an extension.

2	The called extension does not answer the call.

3	The outside subscriber is released.

4	The message waiting is automatically registered to the extension according to the message waiting registration option.

MSG EXT(1)TRK(0) VM(0) 2000-03-01 09:00 am

- Deletion of all message waiting information registered to D-Tel

1	An extension dials the deletion code (Default: *70*) of all messages waiting information.

2	All information is deleted and the result is displayed on the LCD of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the extension

1	An extension dials the deletion code (Default: *701) of all messages waiting information registered by the request from the extension.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the outside subscriber

1	An extension dials the deletion code (Default: *702) of all messages waiting information registered by the request from the outside subscriber.

2	All information is deleted and the result is displayed on the LCD screen of D-Tel.

- Deletion of all message waiting information registered to D-Tel by the request from the VPM

- An extension dials the deletion code (Default: *703) of all messages waiting information registered by the request from the VPM.

- All information is deleted and the result is displayed on the LCD screen of

D-Tel.

- One by one deletion of the message waiting information registered to D-Tel

1	The LED of the reserve button flashes if there is any registered message.

2	The extension presses the Waiting button while the handset is in position.

3	If there are more than two registered messages, they are displayed on the LCD of the D-Tel in the order that they were received.

MESSAGE:XXXX 18:50 2000-03-01 09:00 am“

4	To delete any registered message, the extension should press the “DND/FOR” button while the message to delete is displayed on the LCD.

Service conditions

- If the message is not registered correctly, congestion tone is heard.

- Only one message waiting can be registered to a general extension.

- The message waiting cannot be registered to the Hot Line extension.

- The message waiting cannot be registered to the calling group or representative attendant.

- Only one message waiting can be registered to a SLT.

- If the message waiting is requested in the status that another message waiting has already been registered to SLT, the request is processed as errors.

- Up to 10 message waiting can be registered to one D-Tel.

- (Note: A calls from same subscriber will be counted just once)

- If the extension who has already registered a message waiting requests another message waiting to be registered to the same D-Tel, the second request is disregarded.

- If the message waiting is requested from the outside subscriber to D-Tel, the number of waited messages, which are registered in the system, from the outside subscriber should not exceed 2000 messages.

- If the number of waited messages in D-Tel is 10 and the message waiting is requested from the extension, the request is registered only in case that there should be message waiting from the extension among the existing message waiting contents registered. Note that in this case, the first registered information among the messages that were registered by the request from the extensions is deleted in order to register a new message waiting.

- In case that the message waiting is requested from the outside subscriber to D-Tel in the status that the number of messages waited from the outside subscriber in the system is 2000, if there are messages waited from the outside subscriber to related D-Tel, the oldest information is deleted and the newly requested message is waited.

- If the message waiting is registered to D-Tel, the number of registered message waiting for each trunk/extension/VMS is displayed.

- The telephone for message waiting connected to ASC-MW can assign that the telephone informs the extension of the message waiting registration by turning on the message waiting lamp,

- If SLT or the telephone for message waiting does not assigns that SLT/the telephone informs the extension of the message waiting registration by turning on the message waiting lamp, the ring is sent to the extension periodically to notify the extension of message waiting status.

- Once a call is set up by the message answer code, the registered message is automatically deleted.

Data generation

- Assign whether to display the message waiting registration to the extension with ring, or lamp when the message waiting is registered to SLT.

[3.1 Extension information – Extension Characteristic ⇒ 8 MW Lamp Use SLT]

- Ring information used for informing the extension of the message waiting registration to SLT

[2.5 System ring parameter ⇒ Message Wait Request Ring]

- Automatically allow the message waiting when the extension does not answer the call terminated from the ISDN trunk.(For operating for whole extension in system)

[4.9 Trunk route base all options ⇒ Auto Message Wait reg. by CLI]

- Automatically allow the message waiting when the extension does not answer the call terminated from the ISDN trunk.(For operating for individual extension in system)

[3.1 Extension Information ⇒ Extension Characteristics ⇒28.Auto CLI MW Use]

Interactions