LG PBX System

- Software Technical Information

STI-0013

Jan. 31, 2001

| Title | #### Call Charging Data |
| --- | --- |
| System | CS1000 |
| Abstract | Charging Result Output Device is changed |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.01.2000 | C.K Ahn | First issue. (CS1000 System Manual) |
| 1.1 | 01.31.2001 | C.K Ahn | Second issue from CS100 system Manual) |

Table of Contents

Title : Camp-On	2

11.	Charging Management	2

11.1	  Charging/printer option	2

Title : Call Charging Data

# 11.	Charging Management

11.1	Charging/printer option

- 	Functional description

This screen is used to verify and assign options for charging and display device.

[Figure  Charge option information]

- How to use

1	Query

1)	Click Query.

2)	Verify options for charge and output device.

2	Change

1)	Click the item to be changed.

2)	In each item, change values of list.

3)	Click Change.

4)	Click Send

5)	Click Query

- Contents

| Item | Description |
| --- | --- |
| How much for a metering Tone? | When selecting method of charge account by metering tone sent from trunk side, determines charge per metering tone |
| Charge Result Output Device ① ATD ② PMS ① ATD&PMS | Assigns unit to output charge information. ① Attendant ② Account unit or PMS Host ③ Attendant and account unit or PMS Host |
| Printer Breakdown Alarm | Currently not used |
| Printer Type | Currently not used |
| Space between line | Currently not used |
| How many line in a page | Currently not used |
| Output format | Currently not used |
| VIP shadowed digit | Currently not used |
| Charge start time | When there is no signal of elapsed time (external subscriber answer signal), the time that charge account is started after user sends last dialed number. |
| Charge Tax rates | Currently not used |
| Local Call Charge | When displaying charge information, assigns whether to display list of local calls |
| Tie Call Charge | When displaying charge information, assigns whether to display list of dedicated line calls |
| Use Host PMS | Currently not used |

Reference) When Charge Result Output Device is attendant, charge information displayed to attendant is designated for each subscriber. [3.1 Extension information] – [Extension feature information] – [Charge Send to ATD]