LG PBX System

Software Technical Information

STI-0014

April 9, 2001

| Title | #### Billing Protocol and Message |
| --- | --- |
| System | CS1000 |
| Abstract | Addition of verified account code |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.07.2000 | C.K Ahn | First issue. |
| 1.1 | 01.31.2001 | C.K Ahn | Second issue |
| 1.2 | 04. 09. 2001 | C.K Ahn | Third issue |

Table of Contents

1	Message Description	3

1.1	Conception	3

1.2	Message Structure in case of LAN	3

1.3	Message Structure in case of SIO	3

2	Implementation Considerations	4

2.1	Implementation Considerations of LAN	4

2.1.1	Physical Link	4

2.1.2	Connection Port	4

2.2	Implementation Considerations of SIO(Serial Input Output)	4

2.2.1	Physical Link	4

2.2.2	Cable Connection	4

3	Billing Protocol	5

3.1	The normal link procedure	5

3.1.1	In case of existing a message to sending.	5

3.1.2	In case of nothing a message to sending.	5

3.2	The normal link procedure	6

3.2.1	In case of nothing ACK signal,	6

3.2.2	In case of sending continuously NAK signal from billing server,	6

3.2.3	In case of no answer from Switch	7

3.2.4	In case of nothing EOT from Switch,	7

4	Message Type	8

4.1	Control Message	8

4.2	Data Message	9

4.2.1	Switch System ID	11

4.2.2	Charge Type	11

4.2.3	Call Type : Charging call information	11

4.2.4	Calling Station Category	11

4.2.5	Calling Station/Trunk Group	11

4.2.6	Calling Trunk Number	11

4.2.7	Called Trunk Category	11

4.2.8	Called Station/Trunk Group	11

4.2.9	Called Trunk Number	11

4.2.10	Feature Service Category	11

4.2.11	Charge Information	11

4.2.12	Miscellanies Type	12

4.2.13	Call Integrity	12

4.2.14	Transfer Station Number : a subscriber number who tries to transfer for incoming call	12

4.2.15	Calling Station Number : a subscriber number who used to outgoing call	12

4.2.16	Trunk Access Code	12

4.2.17	Conversation Start Time	12

4.2.18	Conversation End Time	12

4.2.19	Dialed Digits	12

4.2.20	Number of metering Pulse : In case of plural billing method, this is “pulse counter”.	12

4.2.21	Miscellanies Data	12

4.2.22	Real Dialed Digits in LCR Service	12

4.2.23	Verified Account Code	12

# Message Description

## Conception

This document is the protocol procedure and messages to bill between switch and billing server.

The billing message consists of the header and the mandatory, the optional and the end field. The optional field can be added in the mandatory field.

## Message Structure in case of LAN

| Memory Location | Message Field |
| --- | --- |
| [00] ~ [01] | LAN Data Length |
| [02] ~ [05] | Header field |
| [06] ~ [83] | Mandatory field |
| [84] ~ [N-7] | Optional field |
| [N-6] ~ [N] | End field |

- The LAN data length consists of 2 bytes hexadecimal value.

- The LAN data length is different to each message type.

- In case of data packet, the LAN data length is length from start part of header field to end part of end field.

- In case of control packet (EOT/ ENQ/ACK/ NAK), the value of length is 0x0001.

## Message Structure in case of SIO

| Memory Location | Message Field |
| --- | --- |
| [00] ~ [03] | Header field |
| [04] ~ [81] | Mandatory field |
| [82] ~ [N-7] | Optional field |
| [N-6] ~ [N] | End field |

※ The header, mandatory, optional and end field’s contents of SIO Message is the same those of LAN.

# Implementation Considerations

## Implementation Considerations of LAN

### Physical Link

The physical connection is Ethernet 10BaseT.

TCP/IP internet domain stream sockets are used over the Ethernet.

### Connection Port

The Billing Server has to initial a connection by calling TCP/IP port 6016 on the Switch.

## Implementation Considerations of SIO(Serial Input Output)

### Physical Link

- Electrical Interface Characteristic : EIA RS-232

- Operation Mode	: Half Duplex

- Interface Distance	: 15 meters(50 ft) between Switch and Billing Server

- Baud Rate		: 19,200 BPS

- Word Framing	: 8 bit data and 1 stop bit

- Parity		: None

### Cable Connection

Switch has 9 pin female connector.

The follow is a method of physical link connection.

Switch 					Billing Server

Pin 2 – Transmit (TX)				    RX (3)

Pin 3 – Receiver (RX)				    TX (2)

Pin 5 – Ground (GND)				    GND (5)

# Billing Protocol

The special code for protocol consists of 1 byte.

| CODE | VALUE(Hex) |
| --- | --- |
| STX | 0x02 |
| ETX | 0x03 |
| EOT | 0x04 |
| ENQ | 0x05 |
| ACK | 0x06 |
| NAK | 0x15 |

## The normal link procedure

### In case of existing a message to sending.

- The DATA packet should be transmitted after receiving ENQ

### In case of nothing a message to sending.

- In case of nothing a message to sending, NAK will be transmitted.

- In case that the billing server receives NAK, it will send ENQ continuously after waiting for 5 seconds.

- Switch regards as the failure of billing server when it doesn’t receive ENQ within 20 seconds.

## The normal link procedure

### In case of nothing ACK signal,

- Switch will transmit EOT if it doesn’t received ACK within 5 seconds after sending DATA packet.

### In case of sending continuously NAK signal from billing server,

- Switch sends EOT to the billing server after receiving over 4 times NAK, and

then releases the link. (LAN case)

- Switch sends EOT to the billing server after receiving over 4 times NAK, and

then goes initial state. (SIO case)

### In case of no answer from Switch

- In case of no answer for ENQ from Switch, the billing server retries to connect after released the link. (LAN case)

### In case of nothing EOT from Switch,

- In case of nothing EOT from Switch, the billing server is processing continuously protocol after waiting for 5 seconds.

# Message Type

## Control Message

| Classification | Data Position | Contents | Length (Bytes) |  |
| --- | --- | --- | --- | --- |
| Header | [00] | Special code for protocol | 1 | Note) |

Note) Special Code Table

| CODE | VALUE(Hex) |
| --- | --- |
| STX | 0x02 |
| ETX | 0x03 |
| EOT | 0x04 |
| ENQ | 0x05 |
| ACK | 0x06 |
| NAK | 0x15 |

## Data Message

- Message Contents

| Classification | Data Position | Contents | Length (Bytes) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 | Hex |
|  | [01]~[03] | Message Length of Data | 3 | Note2) |
| Mandatory Field | [04]~[05] | Feature Code | 2 | Note4) |
|  | [06]~[07] | Direction | 2 | Note5) |
|  | [08]~[11] | Billing Message Sequence Number | 4 | Note6) |
|  | [12]~[15] | Called Trunk Number | 4 |  |
|  | [16]~[17] | Charge Information | 2 |  |
|  | [18]~[25] | Calling Station Telephone Number | 8 |  |
|  | [26]~[41] | Conversation Start Time | 16 |  |
|  | [42]~[57] | Conversation End Time | 16 |  |
|  | [58]~[81] | Dialed Digits | 24 |  |
| Optional Field | [82]~[83] | Parameter ID 1 | 2 |  |
|  | [84]~[85] | Length | 2 |  |
|  | [96]~[M] | Data of Parameter ID 1 | Note1) |  |
|  | [M+1]~[M+2] | Parameter ID 2 | 2 |  |
|  | [M+3]~[M+4] | Length | 2 |  |
|  | [M+5]~[K] | Data of Parameter ID 2 | Note1) |  |
|  | [K+1]~[N-7] | ~ |  |  |
| End Field | [N-6]~[N-5] | End of Optional Field | 2 |  |
|  | [N-4]~[N-3] | End of Field | 2 |  |
|  | [N-2]~[N-1] | BCC (Block Check Code) | 2 | Note3) |
|  | [N] | ETX(Special Code) | 1 |  |

※ The format of all data excluding special code is ASCII characteristic.

Note 1) Refer to “Parameter Table of Option Field”

Note 2) The field’s value of “Message Length of Data” is [N-3] (Hex Value)

Note 3) This block calculates substances with EXCLUSIVE OR successively from[01] to [N-3].

The result will be attached to BCC after converting into ASCII 2 bytes.

(Hexadecimal format)

Note 4) The value of feature code is ASCII ‘52’.

Note 5) The value of direction is ASCII ‘03’. (Switch  Billing Server)

Note 6) The value of billing message sequence number is hexadecimal’s ASCII from ‘0000’

to ‘9999’. The value must add 1 by packet order and is ‘0000’ after ‘9999’.

- Parameter Table of Optional Field

| Parameter ID (ASCII value) | Contents | Length (Bytes) | ASCII Value of Length | Remark |
| --- | --- | --- | --- | --- |
| ’00’ | Not Used | 2 | ‘02’ |  |
| ’01’ | Switch System ID | 2 | ‘02’ |  |
| ’02’ | Charge Type | 2 | ‘02’ |  |
| ’03’ | Call Type | 2 | ‘02’ |  |
| ’04’ | Calling Station Category | 2 | ‘02’ |  |
| ’05’ | Calling Station/Trunk Group | 2 | ‘02’ |  |
| ’06’ | Calling Trunk Number | 4 | ’04’ |  |
| ’07’ | Called Trunk Category | 2 | ’02’ |  |
| ’08’ | Called Station/Trunk Group | 2 | ’02’ |  |
| ’09’ | Called Trunk Number | 4 | ’04’ |  |
| ’10’ | Feature Service Category | 2 | ’02’ |  |
| ’11’ | Charge Information | 2 | ’02’ |  |
| ’12’ | Miscellanies Type | 2 | ’02’ |  |
| ’13’ | Call Integrity | 2 | ’02’ |  |
| ’14’ | Transfer Station Number | 8 | ’08’ |  |
| ’15’ | Calling Station Number | 8 | ’08’ |  |
| ’16’ | Trunk Access Code | 4 | ’04’ |  |
| ’17’ | Conversation Start Time | 16 | ’16’ |  |
| ’18’ | Conversation End Time | 16 | ’16’ |  |
| ’19’ | Dialed Digits | 24 | ’24’ |  |
| ’20’ | Number of Metering Pulse | 8 | ’08’ |  |
| ’21’ | Miscellanies Data | 16 | ’16’ |  |
| ’22’ | Real Dialed Digits in LCR Service | 32 | ’32’ |  |
| ’23’ | Verified Account Code | 12 | ’12’ |  |
| ‘FF’ | End of Optional Field | 2 | ’02’ |  |

### Switch System ID

‘00” ~ ‘99’

### Charge Type

‘00’ : Unknown

‘01’ : Outgoing Call

### Call Type : Charging call information

‘00’ : Unknown

‘01’ : Internal Call

‘02’ : Local Call

‘03’ : DDD Call

‘04’ : ISD Call

‘05’ : Station to Station Collect Call

‘06’ : Person to Person Collect Call

‘07’ : Collect Call

‘08’ : TIE Access

‘09’ : Data Service

‘0A’ : Public Call

‘0B’ : Incoming Call

‘0C’ : Transit Call

‘0D’ : Feature Service

### Calling Station Category

‘00’ : Unknown

‘01’ : Station without priority

‘03’ : Operator without priority

‘15’ : Test station

‘64’ : Tie trunk line

‘80’ : PX trunk line

### Calling Station/Trunk Group

‘00’ ~ ‘99’

### Calling Trunk Number

‘0000’ ~ ‘9999’

### Called Trunk Category

‘00’ : Unknown

‘06’ : Tie Line

‘07’ : PX trunk line

### Called Station/Trunk Group

‘00’ ~ ‘99’

### Called Trunk Number

‘0000’ ~ ‘9999’

### Feature Service Category

Reserved

### Charge Information

Bit 0=1 : Singular billing method

Bit 1=1 : Plural billing method

Bit 2=1 : No charge subscribe

Bit 2=0 : Charge subscribe

Bit 3=1 : Received charging start signal from counterpart switch

Bit 4=1 : Immediately billing

### Miscellanies Type

‘01’ : Account code

‘02’ : Forced account code

‘03’ : Authorization code

‘16’ : an ANI information of the calling party

others : Not defined.

### Call Integrity

‘00’ : Normal call

‘01’ : Forced release call

### Transfer Station Number : a subscriber number who tries to transfer for incoming call

Ex) station 2001 : ‘2001FFFF’

### Calling Station Number : a subscriber number who used to outgoing call

Ex) station 2010 : ‘2010FFFF’

### Trunk Access Code

Ex) code 0  	: ‘0FFF’

Ex) code 91	: ‘91FF’

### Conversation Start Time

Ex) 1999-04-24 10:59:59	: ‘1999042410595900’

Ex) 2000-01-01 16:59:59	: ‘2000010116595900’

### Conversation End Time

Ex) 1999-04-24 10:59:59	: ‘1999042410595900’

Ex) 2000-01-01 16:59:59	: ‘2000010116595900’

### Dialed Digits

Ex) dialed digits “0123456789*#” : ‘0123456789BCFFFFFFFFFFFF’

### Number of metering Pulse : In case of plural billing method, this is “pulse counter”.

AOC unit counter

‘00000000’ ~ ‘99999999’

### Miscellanies Data

Ex) digits “0123456789*#” : ‘0123456789BCFFFF’

### Real Dialed Digits in LCR Service

Ex) dialed digits “0123456789*#” : ‘0123456789BCFFFFFFFFFFFF’

### Verified Account Code

Ex) account code “0123456789*#” : ‘0123456789BCFFFFFFFFFFFF’