LG PBX System

- Software Technical Information

STI-0015

May 11, 2001

| Title | #### PMS Protocol and Message |
| --- | --- |
| System | CS1000 |
| Abstract | PMS Protocol and message definition |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 0.9 | 11.01.1999 | S.G Ko | First issue. |
| 1.0 | 14.04.2001 | S.G Ko | First revision of specification |
| 1.1 | 11.05.2001 | S.J Ryu | Second revision of specification |

Table of Contents

Title: PMS Protocol and Message	3

Conception	3

Message Structure in case of LAN	3

Message Structure in case of SIO	3

Implementation Considerations	4

Implementation Considerations of LAN	4

Physical Link	4

Connection Port	4

Implementation Considerations of SIO (Serial Input Output)	4

Physical Link	4

Cable Connection	4

PMS Protocol	5

Special Code Table	5

The normal link procedure	5

Normal Link Procedure	5

Error Link Procedure	5

Message Type	7

Data Message	7

General Message Format	7

Check-In Message  (PBX  PMS server)	7

Check-In (for Bypass) Message  (PBX  PMS server)	8

Check-Out Message  (PBX  PMS server)	8

Wake Up Time Registration Message (PBX  PMS server)	8

Wake Up Service Message  (PBX  PMS server)	9

Maid Status Message  (PBX  PMS server)	9

Maid Time Message  (PBX  PMS server)	9

Room Cut-Off Message  (PBX   PMS server)	10

Station Class Modification Message  (PBX  PMS server)	10

Mini-Bar Information Message  (PBX  PMS server)	10

Do Not Disturb Message  (PBX  PMS server)	11

Message Wait Message  (PBX  PMS server)	11

Bath Alarm Message  (PBX  PMS server)	11

Warm Line Message  (PBX  PMS server)	12

Detailed Information	12

Feature Code	12

Direction	13

Guest ID	13

Guest Name	13

Language ID	13

Wake Up Type	13

Wake Up Service ID	13

Room Status Code	13

Cut-Off Status	13

Station Class	14

DND Status	14

Message Status	14

Station Number/Room Telephone Number	14

Maid Time	14

Bath Alarm Type	14

Warm Line Type	14

Title: PMS Protocol and Message

## Conception

This document is for the protocol procedure and messages between PBX and PMS server. The PMS message consists of the header field, data field and the end field.

## Message Structure in case of LAN

| Memory Location | Message Field |
| --- | --- |
| [00] ~ [01] | LAN Data Length |
| [02] ~ [05] | Header field |
| [06] ~ [N-3] | Data field |
| [N-2] ~ [N] | End field |

The LAN data length consists of 2 bytes hexadecimal value.

The LAN data length is different to each message type.

In case of data packet, the LAN data length is from start part of header field to end part of end field.

In case of control packet (EOT/ ENQ/ACK/ NAK), the value of length is 0x0001.

## Message Structure in case of SIO

| Memory Location | Message Field |
| --- | --- |
| [00] ~ [03] | Header field |
| [04] ~ [N-3] | Data field |
| [N-2] ~ [N] | End field |

※ The header, data and end field’s contents of SIO Message is the same those of LAN.

# Implementation Considerations

## Implementation Considerations of LAN

### Physical Link

The physical connection is Ethernet 10BaseT.

TCP/IP internet domain stream sockets are used over the Ethernet.

### Connection Port

The PMS Server has to initial a connection by calling TCP/IP port 6016 on the Switch.

Port 6016 is used with Billing together.

Note: This port number can be changed on a certain occasion.

## Implementation Considerations of SIO (Serial Input Output)

### Physical Link

Electrical Interface Characteristic: 	EIA RS-232

Operation Mode: 			Half Duplex

Interface Distance: 			15 meters (50 ft) between Switch and PMS Server

Baud Rate: 				19,200 bps

Word Framing: 			8 bit data and 1 stop bit

Parity: 				None

### Cable Connection

Switch has 9 pins female connector.

The follow is a method of physical link connection.

Switch 						PMS Server

Pin 2 - Transmit (TX)				    	Receiver (RX) - Pin 3

Pin 3 - Receiver (RX)				   	Transmit (TX) - Pin 2

Pin 5 - Ground (GND)				    	Ground (GND) - Pin 5

# PMS Protocol

## Special Code Table

The special code for protocol consists of 1 byte.

| CODE | VALUE (Hex) |
| --- | --- |
| STX | 0x02 |
| ETX | 0x03 |
| EOT | 0x04 |
| ENQ | 0x05 |
| ACK | 0x06 |
| NAK | 0x15 |
| ENQ (PMS) | 0x16 |

## The normal link procedure

### Normal Link Procedure

PBX

PMS Server

ENQ (PMS)

ACK

DATA

ACK

EOT

### Error Link Procedure

#### ACK timeout for ENQ (PMS)

PBX

PMS Server

ENQ (PMS)

ACK

×

Wait 5sec

ENQ (PMS)

ACK

×

Wait 5sec

If PMS server does not receive ACK, wait 5 sec and try again until receiving ACK.

#### ACK timeout for DATA

PBX

PMS Server

ENQ (PMS)

ACK

DATA

ACK

×

Wait 5sec

ACK

×

Wait 5 sec

ACK

×

Wait 5sec

ACK

4 times repetition

×

EOT

PMS server wait until receiving ACK (4 times repetition)

If PMS server fails to receive ACK, PMS server should send EOT to PBX for finishing this protocol.

# Message Type

## Data Message

### General Message Format

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 | Hex |
|  | [01]~[03] | Message Length of Data | 3 | Note1) |
| Data Field | [04]~[05] | Feature Code | 2 |  |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
| End Field | [N-2]~[N-1] | BCC (Block Check Code) | 2 | Note 2) |
|  | [N] | ETX (Special Code) | 1 | Hex |

※ The format of all data excluding special code is ASCII characteristic.

Note 1) The field’s value of “Message Length of Data” is [N-3]

Note 2) This block calculates substances with EXCLUSIVE OR successively from [01] to [N-3].

The result will be attached to BCC after converting into ASCII 2 bytes.

(Hexadecimal format)

### Check-In Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “11” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Station Number | 8 |  |
|  | [20]~[21] | Guest ID | 2 |  |
|  | [22]~[23] | Language ID | 2 |  |
|  | [24]~[39] | Guest Name | 16 |  |
| End Field | [40]~[41] | BCC (Block Check Code) | 2 |  |
|  | [42] | ETX (Special Code) | 1 |  |

### Check-In (for Bypass) Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “11” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Station Number 1(START) | 8 |  |
|  | [20]~[21] | Guest ID | 2 |  |
|  | [22]~[23] | Language ID | 2 |  |
|  | [24]~[31] | Station Number 2(END) | 8 |  |
| End Field | [32]~[33] | BCC (Block Check Code) | 2 |  |
|  | [34] | ETX (Special Code) | 1 |  |

### Check-Out Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “12” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Station Number 1 (START) | 8 |  |
|  | [20]~[27] | Station Number 2 (END) | 8 |  |
| End Field | [28]~[29] | BCC (Block Check Code) | 2 |  |
|  | [30] | ETX (Special Code) | 1 |  |

### Wake Up Time Registration Message (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “13” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Station Number 1 (START) | 8 |  |
|  | [20]~[27] | Station Number 2 (END) | 8 |  |
|  | [28]~[29] | Wake Up Type (00:Cancel, 01:Today, 02:Tomorrow, 03:Every Day) | 2 |  |
|  | [30]~[33] | Wake Up Time (HH:MM) | 4 |  |
| End Field | [34]~[35] | BCC (Block Check Code) | 2 |  |
|  | [36] | ETX (Special Code) | 1 |  |

### Wake Up Service Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “14” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Station Number | 8 |  |
|  | [20]~[21] | Wake Up Service ID (1:Answer/2:No Answer/3:Busy) | 2 |  |
|  | [22]~[29] | Wake Up Time (HH:MM) [26~29]: FF | 8 |  |
| End Field | [30]~[31] | BCC (Block Check Code) | 2 |  |
|  | [32] | ETX (Special Code) | 1 |  |

### Maid Status Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “17” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Room Telephone Number | 8 |  |
|  | [20]~[21] | Room Status Code | 2 |  |
|  | [22]~[25] | Maid Time (HH:MM) | 4 |  |
|  | [26]~[29] | Maid ID | 4 |  |
| End Field | [30]~[31] | BCC (Block Check Code) | 2 |  |
|  | [32] | ETX (Special Code) | 1 |  |

### Maid Time Message (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “18” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Room Telephone Number | 8 |  |
|  | [20]~[23] | Maid Time (HH:MM) | 4 |  |
|  | [24]~[27] | Maid ID | 4 |  |
| End Field | [28]~[29] | BCC (Block Check Code) | 2 |  |
|  | [30] | ETX (Special Code) | 1 |  |

### Room Cut-Off Message  (PBX   PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “19” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Room Telephone Number (Start) | 8 |  |
|  | [20]~[27] | Room Telephone Number (End) | 8 |  |
|  | [28]~[29] | Cut-Off Status | 2 |  |
| End Field | [30]~[31] | BCC (Block Check Code) | 2 |  |
|  | [32] | ETX (Special Code) | 1 |  |

### Station Class Modification Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “20” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Room Telephone Number | 8 |  |
|  | [20]~[21] | Station Class | 2 |  |
| End Field | [22]~[23] | BCC (Block Check Code) | 2 |  |
|  | [24] | ETX (Special Code) | 1 |  |

### Mini-Bar Information Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “22” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Room Telephone Number | 8 |  |
|  | [20] | The Number of Item 1 | 1 |  |
|  | [21] | The Number of Item 2 | 1 |  |
|  | Continue | Continue |  |  |
|  | [82] | The Number of Item 63 | 1 |  |
|  | [83] | The Number of Item 64 | 1 |  |
| End Field | [84]~[85] | BCC (Block Check Code) | 2 |  |
|  | [86] | ETX (Special Code) | 1 |  |

### Do Not Disturb Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “23” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[13] | DND Status | 2 |  |
|  | [14]~[21] | Room Telephone Number | 8 |  |
| End Field | [22]~[23] | BCC (Block Check Code) | 2 |  |
|  | [24] | ETX (Special Code) | 1 |  |

### Message Wait Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “24” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[13] | Message Status | 2 |  |
|  | [14]~[21] | Room Telephone Number (Called) | 8 |  |
|  | [22]~[29] | Room Telephone Number (Calling) | 8 |  |
| End Field | [30]~[31] | BCC (Block Check Code) | 2 |  |
|  | [32] | ETX (Special Code) | 1 |  |

### Bath Alarm Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “26” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Telephone Number | 8 |  |
|  | [20]~[21] | Type (1: Occur, 2: Answer) | 2 |  |
|  | [22]~[23] | Hour (HH) | 2 |  |
|  | [24]~[25] | Minute (MM) | 2 |  |
|  | [26]~[33] | Station Number | 8 |  |
| End Field | [34]~[35] | BCC (Block Check Code) | 2 |  |
|  | [36] | ETX (Special Code) | 1 |  |

### Warm Line Message  (PBX  PMS server)

| Classification | Data Position | Contents | Length (Digits) |  |
| --- | --- | --- | --- | --- |
| Header Field | [00] | STX (Special Code) | 1 |  |
|  | [01]~[03] | Message Length of Data | 3 |  |
| Data Field | [04]~[05] | Feature Code | 2 | “27” |
|  | [06]~[07] | Direction | 2 |  |
|  | [08]~[11] | Message Sequence Number | 4 |  |
|  | [12]~[19] | Telephone Number | 8 |  |
|  | [20]~[21] | Type 1: Calling Warm Line 2: Answer 3. Release in Calling Warm Line | 2 |  |
|  | [22]~[23] | Month (MM) | 2 |  |
|  | [24]~[25] | Day (DD) | 2 |  |
|  | [26]~[27] | Hour (HH) | 2 |  |
|  | [28]~[29] | Minute (MM) | 2 |  |
|  | [30]~[37] | Trunk Code | 8 |  |
|  | [38]~[59] | Dialed Digit (Max 22 Digits) | 22 |  |
| End Field | [60]~[61] | BCC (Block Check Code) | 2 |  |
|  | [62] | ETX (Special Code) | 1 |  |

## Detailed Information

### Feature Code

| Feature Code | Function |
| --- | --- |
| ”11” | Check In/Check In (By pass) |
| “12” | Check Out |
| “13” | Wake Up Time Registration |
| “14” | Wake Up Service |
| “17” | Maid Status |
| 18 | Maid Time |
| “19” | Room Cut-Off |
| “20” | Station Class Modification |
| “22” | Mini-Bar Information |
| “23” | Do Not Disturb |
| “24” | Message Wait |
| 26 | Bath Alarm Message |
| 27 | Warm Line Message |
| “52” | Call Charge |

The feature code is ASCII values.

### Direction

“01”: PBX  PMS

“02”: ATD  PMS

“03”: EXT  PMS

### Guest ID

“01”: VIP

“02”: Non-VIP

### Guest Name

Ex)	James Lee  “James Lee”  (Max. 16 chars)

### Language ID

“01”: Korean

“02”: English

“03”: Japanese

“04”: German

“05”: French

“06”: Spanish

“07”: Chinese

### Wake Up Type

“00”: Cancel

“01”: Today

“02”: Tomorrow

“03”: Every Day

### Wake Up Service ID

“01”: Answer

“02”: No Answer

“03”: Busy

### Room Status Code

“00”: Service End

“01”: Cleaning Needed

“02”: Under Cleaning

“03”: Cleaned Up

“04”: Repair Needed

“05”: Under Repair

“06”: Repair Completed

“07”: Reserved

### Cut-Off Status

“00”: Cancel

“01”: Cut-Off

### Station Class

“00”:  Blocking

“02”:  F Class

“03”:  E Class

“04”:  D Class

“05”:  C Class

“06”:  B Class

“07”:  A Class

“08”:  S Class

“09”:  I Class

### DND Status

“00”: Cancel

“01”: Registration

### Message Status

“00”: Cancel

“01”: Registration

### Station Number/Room Telephone Number

Ex) 	1001  “1001FFFF”  (8 chars)

### Maid Time

Ex)	9:10am	 “0910”

10:23pm	 “2223”

### Bath Alarm Type

“1”: Occur

“2”: Answer

### Warm Line Type

“1”: Calling Warm Line

“2”: Answer

“3”: Release in Calling Warm Line