LG PBX System

Software Technical Information

STI-0016

Jan. 31, 2001

| Title | #### Attendant Collect Call |
| --- | --- |
| System | CS1000 |
| Abstract | The operating procedure is changed |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 01.01.2000 | S.H Jin | First issue. (CS1000 System Manual) |
| 1.1 | 01.31.2001 | S.H Jin | Second issue from CS100 system Manual) |

Table of Contents

Title : Attendant – Collect Call	2

Functional description	2

Operating procedure	2

Service conditions	2

Data Generation	2

Interactions	2

Title : Attendant – Collect Call

Functional description

When the call incoming to attendant requests collect call, this function enables attendant to register the related information.

Operating procedure

- Collect call information registration in attendant.

1	Attendant answers an outside subscriber’s call.

- The outside subscriber requests Collect Call to Extension A from ATD.

- ATD call Extension A. In this case, the outside subscriber gets to hear Hold Music.

4	Extension A answers the call of ATD and allows the Collect Call.

5	ATD holds “CHRG” Key down and enters “3”.

6	LCD of ATD indicates “Error”.

7	ATD holds “CLR” Key down and enters “3”.

8	ATD dials the outside subscriber’s No. and presses “ENT” Key.

9	The subscriber is successfully connected to Extension A.

- When the extension refuses to receive the collect call.

1	The extension refuses to receive the collect call.

2	The attendant presses “CLR” key to talk with the outside subscriber.

3	In case of mistype, the operator presses “CLR” key and starts entering again.

Service conditions

- Collect Call Information:

1	Extension to Extension Collect Call

2	Person to Person Collect Call

3	Collect Call

- 11.1 Charge/printer option: Assign the device for charging and charging result display/print.

- 3.1 Extension information – Extension Characteristic – 38 Charge Send to ATD : Determines whether to transfer the charging result of each extension to attendant.

Data Generation

Interactions