LG PBX System

### Software Technical Information

STI-0023

Apr. 9, 2001

| Title | #### CLI(ANI) Print to CRT |
| --- | --- |
| System | CS1000 |
| Abstract | Function to display outside caller information in receiving an incoming trunk call. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| Draft 1.0 | 02.20.2001 | J.H.YEO | Initial Development (Spec. release to Promelit) |
| 1.0 | 04.09.2001 | H.G Kang | Revised specification |

Table of Contents

Software Technical Information	1

Title : CLI(ANI) print to CRT	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	3

Interaction	3

Hardware	3

Title: CLI (ANI) print to CRT

Functional Description

Function to display outside caller information in a desired format using SIO in receiving an incoming call through DTC(R2) and ISDN Trunk.

Operation Procedure

Insert comments described in Interaction item into Config.sys file and reboot the system. Then set the option described in Data Generation item below.

Connecting a cable to SIO channel assigned for display brings the information in a format chosen to the screen when there is an incoming trunk call.

Caller number of 818-4863 and called number of 2004 is displayed as below.

- Setting the value of Option 2 to 1 : Type 1

2001:04:0703:00:35200408184863

- Setting the value of Option 2 to 2 : Type 2

2001:04:07 03:01:02 2004 8184863

- Setting the value of Option 2 to 3 : Type 3

2001:04:07 03:02:18 2004 0000 08184863

: Message generated date

: Message generated time

: Call terminated extension number

: DDD code (meaningful value is displayed only domestically)

: Outside caller number

TYPE 1: “0” is automatically prefixed to the caller number in case the first digit does not start with “0”.

TYPE 2: The whole digits are displayed as input..

TYPE3 : “0” is prefixed to the caller’s number unconditionally for a display.

- All format have 0x02 for Header and 0x0D, 0x0A, and 0x00 for Tail.

Caller number is only one time displayed for calling an IDLE subscriber.

Conditions

The function is available for only a call accessing a trunk enabled to receive outside caller number. The trunks currently developed for this function are ISDN Trunk and DTC(R2).

The function, as developed, is usable above the version 1.53 home and abroad.

Data Generation

- The function is subject to setting options below.

- In 2.6 System Feature Option, choose YES for [ANI Info. Print out (CRT,Printer) ?].

- In 4.9 Trunk Route base All Option, choose YES for [Request ANI Information].

  - For the use of DTC(R2) trunk

- In 7.2 ISDN Option, choose a type you want from [Line Identification Sending Type] item.

  - For the use of ISDN Trunk

- In 16.12 Temporary Option, set the value of [Option 2] value between 1 and 3 and choose output format you want.

Interaction

You must add the contents as below to system config.sys file to check the function by displaying information on the screen.

Sample :

- To display contents in IOT2

ANITX IOT2

END

- To display contents in CN4

ANITX CN4

END

Hardware

It is not that H/W be necessary for the function.