LG PBX System

Software Technical Information

- STI-0024

April 9, 2001

| Title | #### Account Code with Password (Verified Account Code) |
| --- | --- |
| System | CS1000 |
| Abstract | Function to use a set length of billing statistics code. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| Draft 1.0 | 02.21.2001 | H.G Jung | Initial Development (Spec. release to Promelit) |
| 1.0 | 04.09.2001 | H.G Jung | First revised specification |

Table of Contents

Title: Verified Account Code	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	3

Interaction	3

Hardware	3

	OMS Menu Description	3

	How to use	4

	Contents	4

Title: Verified Account Code

Functional Description

The function is needed to use Account Code stored in the system for billing statistics in sending an outgoing trunk call. Account code input is available before sending a call or on the line. The account code is included in billing information (SMDR).

Operation Procedure

- [Account Code Input Before Sending a Call]

- Get into Off Hook state.

- Get an extension dial tone.

- Press Trunk Access Code.

- Press the chosen Account Code. (Max 12 Digits)

- Get a trunk dial tone.

- Dial an outside phone number.

- [Account Code Input on the Line for SLT/DTEL/DECT]

- During conversation.

- Make a Hook Flash or press the transfer button.

- Get an extension dial tone.

- Press Verified Account Feature Code in Conversation. (Default #44).

- Press the chosen Account Code. (Max. 12 Digits)

- Function completion tone is given.

- Make a Hook Flash or press the transfer button.

- Continue the call conversation.

- [Account Code Input on the Line for ATD]

- During conversation.

- Press Charge Key.

- Press the chosen Account Code. (Max. 12 Digits)

- Continue the call conversation.

Conditions

  - One system is available to use a maximum of 1,000 verified Account Codes.

  - Verified Account Code is available for max 12 Digits.

  - The number of the first input Verified Account Code digits becomes that of system Verified Account Code digits.  From the second input forwards, the same number of digits is permitted to add.

  - SLT, DKTU, DECT, and ATD subscribers may use the function.

  - Trunk Access Code must be assigned to a type using Verified Account Code, in case of using Verified Account Code before sending an outgoing call.

  - Inputting non-assigned Verified Account Code restricts an outgoing call, in case of using Verified Account Code before sending an outgoing call

  - It is possible to choose an available destination instead of using Verified Account Code before sending an outgoing call

  - Redialing uses the same Account Code as the previous one.

  - The function is available for trunk transit.

  - The function is available for system speed dial.

  - The function is available for individual speed dial.

  - The function is available for call forwarding to outside.

  - Redialing uses the same Account Code as the previous one.

  - The verified account code is possible to be defined in the flexible button as follows:

    - Trunk access code + Feature code only

    - Trunk access code + Feature code + Part of account code

    - Trunk access code + Feature code + All of account cod

    - Trunk access code + Feature code + All of account code + Part of Outside subscriber number

    - Trunk access code + Feature code + All of account code + All of Outside subscriber number

  - It is possible to be used with non-verified account code.

Data Generation

- Choosing Verified Account Code

- [11.3 Verified Account Code]

- Choosing subscribers not permitted to use Verified Account Code

- [3.1 Extension Information => No Verified ACNT item]

- Choosing trunk access code to use Verified Account Code

- [4.2 Trunk Access Code => Info 3]

Interaction

- PMS Protocol

Added to Parameter 23 of “Parameter Table of Optional Field”.(See PMS Protocol 1.1)

Hardware

N/A

	OMS Menu Description

This menu is to check, add, or delete Verified Account Code information.

[Sample Screen Description]  Menu Number: 11.3

	How to use

1	Query

  - Click Query.

2	Add

- Add a table in the last item using downward arrow key( ↓).

- Enter an extension number.

- Click Add.

- Click Send.

3	Delete

- Select an extension number you want to delete.

- Click Delete.

- Click Send.

	Contents

[Item Description]

| Items | Contents |
| --- | --- |
| First Digit | First Digit of Verified Account Code |
| Verified Account Code | Verified Account Code |
| Comment | Comments on Account Code (Max 12 Digits) |