LG PBX System

Software Technical Information

- STI-0025

Feb 23, 2001

| Title | #### Change of Bell Sound for DKTU |
| --- | --- |
| System | CS1000 |
| Abstract | This function is used to distinguish the bell sound for DKTU. |

- Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| Draft 1.0 | 02.21.2001 | H.G Jung | Change bell sound of DKTU (Spec. release to Promelit) |
| 1.0 | 04.14.2001 | H.G Jung | First specification |

- Table of Contents

Title : Change of Bell Sound	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	2

Interaction	3

Hardware	3

- Title: Change of Bell Sound

- Functional Description

This is used to distinguish the bell sound for DKTU connected to DSLC-LKD Issue 2.

- Operation Procedure

- N/A

- Conditions

DSLC-LKD Ring Control Specification

- SLT and DKTU connected to DSLC-LKD Issue1

The ring type can be used differently depending on a kind of incoming calls (trunk ring, extension ring, recall ring, etc.) controlling the cadence and time assigning the ring group (maximum 10 ring groups in a system).

- DKTU connected to DSLC-LKD Issue2

  - The ring type can be selectable between the system ring tone depending on ring group (maximum 10 ring group in a system) and the ring of wave file form built in DSLC-LKD Issue2.

  - The ring type can be used differently depending on a kind of incoming calls (trunk ring, extension ring, recall ring, etc.) controlling the cadence and time assigning the ring group (maximum 10 ring groups in a system) when use the system ring tone.

  - The ring type can be used differently depending on a kind of incoming calls (trunk ring, extension ring, recall ring, etc.) the ring ID (maximum 8 types) of wave file form built in DSLC-LKD Issue2.

- Data Generation

[2.5 System Ring Parameter]

Group Number: 1

| Ring ID | Ring Lock | 1st On | 1ST Off | 2nd On | 2nd Off | 3rd On | 3rd Off | 4th On | 4th Off | ## Local Ring ID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Trunk Ring | 25 sec | 1 sec | 4 sec | 1 sec | 4 sec | 1 sec | 4 sec | 1 sec | 4 sec | ## ID 1 |

[Additional part in the existing menu]

Note:	1.  Maximum 10 ring groups are assigned in a system.

2. 	Maximum 8 ring type IDs built in local board for DKTU connected to DSLC-LKD Issue2 are selectable.

- Interaction

- N/A

- Hardware

- N/A