LG PBX System

Software Technical Information

STI-0027

April 14, 2001

| Title | #### Автоматический переход на летнее/зимнее время |
| --- | --- |
| System | CS1000 |
| Abstract | Автоматический перевод станционного времени на один час вперед/назад |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| Draft 1.0 | 02.18.2001 | H.G Jung | First issue. (Spec. release to Promelit) |
| 1.0 | 04.14.2001 | H.G Jung | First revision of specification |

Содержание

Автоматический перевод  времени	2

Описание	2

2

Conditions	2

Data Generation	2

Interaction	2

Hardware	2

	OMS Menu Description	3

	How to use 	3

	Contents 	3

Title: Automatic Daylight Time Saving Service

Functional Description

Automatic Daylight Time Saving Service is used in Day/Night Time policy countries to put Day time forward one hour in summer season and put it back in winter season. If you set the date and time, system automatically adjusts the system time. Also if a subscriber is in on-hook state on the time, the charging information is adjusted.

Operation Procedure

- N/A.

Conditions

- Set the date and time to put system time forward one hour and back.

- Charging information on call time adjustment is made only for subscribers in on-hook state on the point of system time adjustment.

Data Generation

- Set the date and time to put system time forward one hour and back.

- [16.13 Automatic Daylight Saving Time]

Interaction

N/A

Hardware

N/A

	OMS Menu Description

This menu is for checking or changing the date and time to auto-perform Daylight Time Saving Service.

[Sample Screen Description]  Menu Number: 16.19

	How to use

1	Query

  - Click Query.

2	Change

- Click the item you want to change.

- Change Data.

- Click Send.

	Contents

[Item Description]

| Items | Contents |
| --- | --- |
| Current Time +1 Hour | Data of date and time to put the current system time forward one hour |
| Current Time -1 Hour | Data of date and time to put the current system time backward one hour |
| Month | Range 1 ~ 12 |
| Day | Range 1 ~ 31 |
| Hour | Range 0 ~ 23 |