LG PBX System

### Software Technical Information

STI-0030

April 6, 2001

| Title | #### RX Numbering Plan |
| --- | --- |
| System | CS1000 |
| Abstract | Subscriber is permitted to make its own extension number up to 12 digits including PBX OWN Code. |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 06.04.2001 | H.G Kang | First developed feature. |

Table of Contents

Software Technical Information	1

Title : RX Numbering Plan	2

Functional Description	2

Operation Procedure	2

Conditions	2

Data Generation	3

Interaction	3

Hardware	3

Title : RX Numbering Plan

Functional Description

Subscriber is permitted to make its own extension number up to 12 digits including PBX OWN Code so that inter-subscriber calls may be available.

Operation Procedure

- SLT

- An extension subscriber picks up the handset.

- He presses PBX Own Code.

- Then he dials an extension No he wants to call.

- RBT is given in case the called extension is available.

- The call is successfully made in the way that the called extension makes an answer to it.

- DKTU

- An extension subscriber picks up the handset or presses its speaker button. (Skippable over this item)

- He presses PBX Own Code.

- Then he dials an extension No he wants to call.

- RBT is given in case the called extension is available.

- The call is successfully made in the way that the called extension makes an answer to it.

- ISDN Phone (Overlap Outgoing)

- An extension subscriber picks up the handset or presses its speaker button.

- He presses PBX Own Code.

- Then he dials an extension No he wants to call.

- RBT is given in case the called extension is available.

- The call is successfully made in the way that the called extension makes an answer to it.

- ISDN Phone (Enbloc Outgoing)

- An extension subscriber presses PBX Own Code.

- He dials an extension No he wants to call.

- Then he presses the speaker button.

- RBT is given in case the called extension is available.

- The call is successfully made in the way that the called extension makes an answer to it.

- DECT Terminal

- An extension subscriber dials PBX Own Code and extension number he wants to call, and then presses ON/OFF button.

- Or he presses ON/OFF button and then dials PBX Own Code and extension number he wants to call.

- RBT is given in case the called extension is available.

- The call is successfully made in the way that the called extension makes an answer to it.

Conditions

Display of billing information in SMDA indicates only extension number, excluding Own Code.

ATD is not permitted to use Own Code.

DKTU or DECT terminal displays only extension number for information on an incoming call.

PBX Own Code is even available for Transfer or Conference mode.

Whether to use only extension number excluding PBX Own Code is definable with its option.

Numbering plan is applied to registration of Conference, Transfer, Pickup, Call Forwarding, Message Wait, DKTU Flexible Button.

Data Generation

Interaction

When there is a call to an extension from a different PBX using a tie line and the caller creates a Trunk Access Code in the same way as PBX Code, the same numbering plan is available for inter-trunk calls.

Hardware

N/A